TRAINING SUPPORT PACKAGE (TSP)

Note: This is the TSP title page

| TSP Number | 88K12A05 |
| --- | --- |
| Task Number(s) Title(s) | Temporary Eyes |
| Effective Date | 1 October 2000 |
| Supersedes TSP(s) | PFN 421-107-04 22 FEBUARY 1995 |
| TSP User | MOS 88K WATERCRAFT OPERATOR |
| Proponent | DEPARTMENT OF THE ARMY TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street Suite 227 Ft. Lee, VA 23801-1511 |
| Comments/ Recommen- dations | Send comments and recommendations directly to: DEPARTMENT OF THE ARMY TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street Suite 227 Ft. Lee, VA 23801-1511 |
| Foreign Disclosure Restrictions | This product has been reviewed by the product developers in coordination with the Commander, U.S. Army Combined Arms Support Command foreign disclosure authority. This product is releasable to military students from foreign countries on a case-by-case basis. |

PREFACE

Purpose	This training support package provides the instructor with a

standardized lesson plan for presenting instruction for:

TASK NUMBER:

TASK TITLE: Temporary Eyes

CONDITIONS: Given class notes, the evaluation plan and a five question quiz.

STANDARD: To receive a “GO” for this Lesson Unit the student will answer five questions correctly.

This TSP

contains

| TABLE OF CONTENTS |  |  |
| --- | --- | --- |
|  |  | Page |
| Preface |  | 2 |
| Lesson | Section I - Administrative Data | 3 |
| Plan | Section II - Introduction | 6 |
|  | Terminal Learning Objective | 6 |
|  | Section III - Presentation |  |
|  | A. Enabling Learning Objective 1 | 7 |
|  | B. Enabling Learning Objective 2 |  |
|  | C. (etc.) |  |
|  | Section IV - Summary | 12 |
|  | Section V - Student Evaluation | 13 |
| Appendixes | A. Viewgraph Masters |  |
|  | B. Test and Test Solutions |  |
|  | C. Practical Exercises and Solutions | 14 |
|  | D. Student Handouts |  |

Watercraft Operator

[Effective Date]

SECTION I - ADMINISTRATIVE DATA

| All Courses Including This Lesson | COURSE NUMBER(S) 88K10 |  | COURSE TITLE(S Water Craft Operator |  |  |
| --- | --- | --- | --- | --- | --- |
|  | TASK NUMBER 88K11B14 551-730-1609 |  | TASK TITLE Construct a Temporary eye in Wire Rope |  |  |
| Reinforced Task(s) | TASK NUMBER |  | TASK TITLE |  |  |
| Academic Hours: | The Academic hours required to teach this course are as follows: |  |  |  |  |
| *TEST *TEST REVIEW TOTAL HOURS | PEACETIME HOURS/METHOD 1.5/C .5/D 2.0/PE1 /0000000000 4.0 |  | MOBILIZATION HOURS/METHOD / / / /000000000000000 |  |  |
| Test Lesson [List the lesson number in which the terminal learning objective of Number this lesson are tested and the test results are reviewed with the students.] |  |  |  |  |  |
|  |  |  |  | Hours | Lesson No. |
|  |  | Testing: |  | _______________ _______________ | _______________ _______________ |
|  |  | Review of test results: |  | _______________ | _______________ |

| Prerequisite Lesson(s) | (State if there are none.) LESSON NUMBER None | LESSON TITLE |
| --- | --- | --- |
| Clearance and Access | UNCLASSIFIED (U) |  |

| References |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Number FM 55-501 | Title Marine Crewman’s Handbook | Date 15 Mar 1983 | Para No. | Additional Information |

| Student Study Assignments | None |
| --- | --- |
| Instructor Requirements | [These are special qualifications or additional requirements.] |
| Additional Support Personnel Requirements | None |
| Equipment Required | Overhead Projector |
| Materials Required | Pencil, paper, chalk and eraser |

| Classroom, Training Area, and Range Requirements | Classroom with chalkboard |
| --- | --- |
| Ammunition Requirements | None |
| Instructional Guidance | Note: Ensure facilities, equipment, and training materials are ready prior to class start time. Instructors will have a thorough knowledge by studying this lesson and identified reference material. |

| Proponent Lesson Plan Approvals | NAME ___________________ ___________________ | Rank _____ _____ | Position __________________ __________________ | Date _______ _______ |
| --- | --- | --- | --- | --- |

SECTION II - INTRODUCTION

|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: 0.3hr |
| --- | --- |
| Motivator | Introduce yourself and the lesson in wire rope with clips. If you were to compute the safe working load (SWL)of a fiber line to secure a sixty ton tank or crane, the line would be too bulky and heavy to handle. So. to handle this amount of weight we must turn to fiber line's big brother, wire rope. |
| Show VG 2-10 Terminal Learning Objective | Making temporary eyes |

| ACTION: | After this lesson unit, the students will demonstrate their knowledge of temporary eyes. |
| --- | --- |
| CONDITIONS: | Given FM 55-501, class notes, and a quiz. |
| STANDARD: | To receive a “GO” for this lesson unit, the students must answer all of the check-on-learning questions correctly. |

| Safety Requirements | [State if there are none.] None |
| --- | --- |
| Risk Assessment Level | Low |
| Environmental Considerations | [State if there are none.] None |
| Evaluation | None |
| Instructional Lead-in |  |

SECTION III - PRESENTATION

| Note: | Inform the students of the Enabling Learning Objectives requirements. |
| --- | --- |
| A. | ENABLING LEARNING OBJECTIVE A |

| ACTION | After this lesson unit, the students will demonstrate their knowledge of temporary eyes. |
| --- | --- |
| CONDITIONS | Given FM 55-501, class notes, and a quiz. |
| STANDARD | To receive a “GO” for this lesson unit, the students must answer all of the check-on-learning questions correctly. |

|  | Learning Step/Activity 1 [A statement describing what is to be done.] |
| --- | --- |
|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: 1:0 hr Media Chalkboard |
| NOTE | Let's begin by discussing the need for seizing wire pope. |
| NOTE | Supplies needed for this lesson unit are an 8 foot Length of wire rope, 5/8' diameter (per 2 students), 1 spool of seizing wire (per class),nippers or pliers (12 pair per class),wire rope clips (3 per 2 students), crescent wrenches (12 inch, 1 per 2students). |
|  | A. SEIZING WIRE ROPE 1. When to seize. We must seize all wire rope before cutting. If the ends of the wire are not properly secured, maximum service can not be obtained. Some strands would carry a greater load than other strands. 2. Type of seizing wire. Use annealed wire for seizing. 3. Applying the turns. The turns of the annealed wire must be placed close together, and tight, so that it will not be necessary to tighten them when the ends are twisted together. The turns must end at one end of the seize to make it easy to tap them into the groove. The ends must be twisted counterclockwise. 4. Both sides of the intended cut must be seized. |
| NOTE | See FM 55-501 illustration 1-6, pg. 12-20 |
|  | B. FORMULAS FOR SEIZING Write each formula on the chalkboard prior to instructing the following. 1. Number of seizing. a. The formula for determining the number of seizings required is 3 X D.' D is the diameter of 5/8 the wire rope. b. If the answer results in a traction, round up to the next higher whole number. c. The standard wire rope used aboard U. S. Army WATERCRAFT is new improved plow steel with a diameter of 5/8th inch. 5/8 inch wire rope is considered 'general purpose' and is used for securing cargo as well as many other purposes. d. Sample - 3 X 5/8 = 15/8 or 1 7/8 (round up to 2.) In this case, the correct number of seizings is 2. 2. Space between seizings. a. The formula for determining the distance between seizings is 2 X D. D is the diameter of the wire rope. b. If the answer results in a fraction round up to the next higher whole number. The answer is in inches. c. Sample - 2 X 5/8 = 10/8, or 1 2/8 or 1 1/4 (round up to 2.) in this case the correct distance between the seizings is 2 inches. |
| # NOTE | - Width of the seizing. The width of the seizings is equal to the diameter of the wire rope. Instructor will ask if there are any questions to this point. |
| NOTE | Transition students into temporary eyes |
| # NOTE | Because not all cargo has the game configuration, often you will have to install temporary eyes with wire rope clips in the wire rope you are using to secure your cargo. Carelessness at securing cargo can cause a disaster at sea. If you fail to use the correct number of wire rope clips when making your temporary eyes or properly tightening down the clips, your cargo may break loose. One of the things you hope you will never have to do is to try to secure a bull dozer that has broken loose during a storm at sea. Place the appropriate formulas on the chalkboard as you instruct. |
|  | C. FORMULAS FOR TEMPORARY EYES 1. Determine the number of wire rope clips required. a. The formula for determining the number of wire rope clips required is: 3 X D + 1 = number of clips. (D = diameter of the wire rope.) b. If the answer results in a fraction, round up to the next higher whole number. c. Sample - How many wire rope clips are required to make a temporary eye in 5/8 inch wire rope? ANSWER = 3. 3 X 5/8 = 1 7/8 + I = 2 7/8 (round up to 3). 2. Determine the space between wire rope clips. a. The formula for determining the space between wire rope clips is: 6 X D = Space ( in inches). (D = diameter of the wire rope.) b. Sample - What distance must be maintained between wire rope clips when making temporary eyes in 5/8 inch wire rope? ANSWER = 4 inches. 6 X 5/8 = 30/8 = 3 6/8 = 3 3/4 (round up to 4) = 4 inches. |
|  | 3. Determine the length of the working end. a. The space between the clips, the number of clips, equals to the length of the working end, in inches. b. Sample - What is the length of the working end of a 5/8th wire rope? ANSWER. 12 inches. (number of clips = 3, space between the clips = 4 inches.) 4 X 3 = 12 inches. |
|  | Instructor will ask are there any questions to this point. Instructor will transition students into a demonstration, following the sequence listed below. D. WIRE ROPE 1. Measurement. 2. Marking. 3. Cutting. E. WIRE ROPE CLIPS 1. U bolt. 2. Roddle 3. Size. 4. Nut. 5. Improved type clip. F. FORMING THE EYE 1. Placing clips on wire. 2. Placing roddle on clip. 3. Placing nut on U bolt. G. WRENCH 1. Size of wrench. 2. Use of wrench. 3. Tightening clips. 4 . Tighten clips once more H. USE OF PLIERS AND NIPPERS 1. Wire. 2. Application Instructor will ask are there any question about the lesson covered |
| # CHECK-ON-LEARNING | Students will write the answers to the following questions pertaining to temporary eyes in wire Pope. Q. How many seizings does 1 3/4 inch wire -rope require? A. 6. (3 X 1 3/4 = 3/1 X 7/4 = 21/4 = 5 1/4, round up to 6.) Q. What is the space between seizings for 1 1/2”wire Pope? A. 3 inch (2 X 1 1/2 = 2/1 X 3/2 = 6/2 or 3. Q. What is the formula for determining how many wire rope clips are required to make a temporary eye in any given size wire rope? A. 3 X D + 1 of the wire rope = number of clips. Q What is the formula for determining the length of the working end, for an eye when using clips? A. Number of clips X space between clips = length of working end in inches. Q. How many wire rope clips are required for an eye using 5/8 inch wire rope. A. 3 (3 X 5/8 = 15/8 or 1 7/8 + I = 2 7/8 round u to 3.) Q. What is the space between clips for 5/8 inch wire rope? A. 4o (O X 5/8 = 30/8 or 3 3/4' round up to 4".) Q. How much working end are required for an eye using 5/8 inch wire rope? A. 12' (No. of clips = 3 Space = 4' 3 X 4 = 12a.)I |

SECTION IV - SUMMARY

|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: 0.3hr |
| --- | --- |
| NOTE Review/ Summarize Lesson | Guide the students through an after-action review with focus on all learning steps. |
| Check on Learning | Students will write the answers to the following questions pertaining to temporary eyes in wire Pope. Q. How many seizings does 1 3/4 inch wire -rope require? A. Q. What is the space between seizings for 1 1/2”wire Pope? A. Q. What is the formula for determining how many wire rope clips are required to make a temporary eye in any given size wire rope? A. Q What is the formula for determining the length of the working end, for an eye when using clips? A. Q. How many wire rope clips are required for an eye using 5/8 inch wire rope. A. Q. What is the space between clips for 5/8 inch wire rope? A. Q. How much working end are required for an eye using 5/8 inch wire rope? |
| Transition To Next Lesson | Transition student into practical exercise. |

SECTION V - STUDENT EVALUATION

| Testing Requirements | B. This block of instruction is testable |
| --- | --- |
| Note: | Refer student to the Student Evaluation Plan |
| Feedback Requirement | a. Schedule and provide feedback on the evaluation and any information to help answer students’ questions about the test. |
|  | b. Provide remedial training as needed. |
| Note: | Rapid, immediate feedback is essential to effective learning. |

PRACTICAL EXERCISE SHEET NO.

| Title | None |  |
| --- | --- | --- |
| Introduction |  |  |
| Motivator |  |  |
| Terminal Learning Objective | Note: Inform the students of the following terminal learning objective requirements. At the completion of this lesson you [the student] will: |  |
|  | ACTION | During this lesson unit, the students will demonstrate their ability to create temporary eyes. |
|  | CONDITIONS | (Given tools, materials, work gloves, and class notes. |
|  | STANDARD | To receive a “GO” foe this lesson Unit, the students will perform all task in accordance with |

| Safety Requirements | [State if there are none.] None |
| --- | --- |
| Risk Assessment Level | Low |
| Environmental Considerations | [State if there are none.] None |

| Evaluation |  |
| --- | --- |
| Instructional Lead-in | None |
| Resource Requirements | None |
| NOTE | These procedures will be followed by all students during PE. Students begin practical exercise. Students complete practical exercise. |
| Special Instructions NOTE: | Students will be moved to rigging loft, and divided into 2 person teams for this PE. Normal breaks will not be given. Students breaks will be given at the discretion of the instructors based on class progress. Students will not be allowed to remove any tools or materials from the class room. The instructor is to read the following list of performance measures to the class prior to students starting work. |
| Procedures | 1. Measure an 8 foot length of wire rope from the reel. 2. Mark the wire rope. 3. Seize the wire rope on both sides of the intended cut. 4. Cut wire rope. 5. Place wire rope on work bench, and mark for working end, and the eye. 6. Secure wire rope to over head block and chain system provided. - Place the working end into riggers vice, and adjust, making sure working end is to right while facing the front of the vice. 8. Place the wire rope clips on the wire rope at the proper intervals. 9. Tighten wire rope clips to proper torque. 10. Remove wire rope from the vice. Place on the work bench. 11. Install seizing to bitter end it necessary. 12. Check all spacing for correctness. 13. Tighten wire rope clips to proper torque. |
| Feedback Requirements | Critique practical exercise Guide students through an after-action review, with focus on all learning steps. In conclusion, we will summarize the last four hours of instruction on temporary eyes. |
| # SUMMARY | A. Review last four hours of instruction. B. This block of instruction is examinable under test #9(36-301-04, and the MCE test. |