TRAINING SUPPORT PACKAGE (TSP)

Note: This is the TSP title page

| TSP Number | 88K13B06 |
| --- | --- |
| Task Number(s) Title(s) | SHIPBOARD FIRE FIGHTING |
| Effective Date | I OCT 1997 |
| Supersedes TSP(s) | 421-110-14 NOV 1991 |
| TSP User | 88K WATERCRAFT OPERATORS |
| Proponent | DEPARTMENT OF THE ARMY TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street, Suite 227 Fort Lee, VA 23801-1511 |
| Comments/ Recommen- dations | Send comments and recommendations directly to: DEPARTMENT OF THE ARMY TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street, Suite 227 Fort Lee, VA 23801-1511 |
| Foreign Disclosure Restrictions | This product has been reviewed by the product developers in coordination with the Commander, U.S. Army Combined Arms Support Command foreign disclosure authority. This product is releasable to military students from foreign countries on a case-by-case basis. |

PREFACE

Purpose	This training support package provides the instructor with a

standardized lesson plan for presenting instruction for:

TASK NUMBER: 421-110-14

TASK TITLE: Shipboard firefighting

CONDITIONS: given a five question oral quiz

STANDARD: To receive a “GO” for this lesson unit, the student must answer all the question correctly.

This TSP

contains

| TABLE OF CONTENTS |  |  |
| --- | --- | --- |
|  |  | Page |
| Preface |  | 2 |
| Lesson | Section I - Administrative Data | 3 |
| Plan | Section II - Introduction | 6 |
|  | Terminal Learning Objective | 6 |
|  | Section III - Presentation |  |
|  | A. Enabling Learning Objective 1 | 11 |
|  | B. Enabling Learning Objective 2 |  |
|  | C. (etc.) |  |
|  | Section IV - Summary | 28 |
|  | Section V - Student Evaluation | 29 |
| Appendixes | A. Viewgraph Masters |  |
|  | B. Test and Test Solutions |  |
|  | C. Practical Exercises and Solutions | 30 |
|  | D. Student Handouts |  |

Shipboard Fire Fighting

[Effective Date]

SECTION - I. ADMINISTRATIVE DATA

| All Courses Including This Lesson | COURSE NUMBER(S) __________________ ___________________ |  | COURSE TITLE(S) ________________________________ ________________________________ |  |  |
| --- | --- | --- | --- | --- | --- |
| Task(s) Taught or Supported | TASK NUMBER _______________ _______________ _______________ |  | TASK TITLE _____________________________ _____________________________ _____________________________ |  |  |
| Reinforced Task(s) | TASK NUMBER |  | TASK TITLE ________________________________ |  |  |
| Academic Hours: | The Academic hours required to teach this course are as follows: |  |  |  |  |
| **TOTAL HOURS | PEACETIME HOURS/METHOD 4.2/C 1.0/D 7.0/PE1 1.8/TV /0000000000 14.0HRS |  | MOBILIZATION HOURS/METHOD / / / / /000000000000000 |  |  |
| Test Lesson [List the lesson number in which the terminal learning objective of Number this lesson are tested and the test results are reviewed with the students. |  |  |  |  |  |
|  |  |  |  | Hours | Lesson No. |
|  |  | Testing: |  | _______________ _______________ | _______________ _______________ |
|  |  | Review of test results: |  | _______________ | _______________ |

| Prerequisite Lesson(s) | (State if there are none.) LESSON NUMBER NONE _________________ | LESSON TITLE ________________________ ________________________ |
| --- | --- | --- |
| Clearance and Access | UNCLASSIFIED (U) |  |

| References |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Number | Title | Date | Para No. | Additional Information |
|  | FM 55-501 | Marine Crewman Handbook | CH 1 |  |  |

| Student Study Assignments |  |
| --- | --- |
| Instructor Requirements | [These are special qualifications or additional requirements.] |
| Additional Support Personnel Requirements | Fire Fighting Lab |
| Equipment Required | TV VCR TAPES |
| Materials Required | INSTRUCTOR AND STUDENT PACKET STUDENT PACKET |
| Classroom, Training Area | CLASSROOM FIRE FIGHTING LAB |
| Ammunition Requirements | NONE |
| Instructional Guidance | Note: Ensure facilities, equipment, and training materials are ready prior to class start time. Instructors will have a thorough knowledge of reference prepare by studying this lesson and identified reference material. |

| Proponent Lesson Plan Approvals | NAME ___________________ ___________________ | Rank _____ _____ | Position __________________ __________________ | Date _______ _______ |
| --- | --- | --- | --- | --- |

SECTION II - INTRODUCTION

DAY 1

|  | Method of instruction: C/TV Instructor to student ratio is 1:C Time of instruction: .2 hr |
| --- | --- |
| Motivator | Gain students attention and introduce self and lesson. Explain to the students as Watercraft Operators you must know how to identify what class of fire you could have on board and the proper way extinguish it. Fire aboard ship is a terrifying experience. Fire creates situations in which the crew must work together as a team in order to survive. To do this you must know the type of fire you are fighting, select the correct extinguishing agent, and be able to use the fire fighting equipment aboard your ship. |
| Show VG 1-12 Terminal Learning Objective | Note: Inform the students of the following terminal learning objective requirements. At the completion of this lesson you [the student] will: |

| ACTION: | After this lesson unit, the student will answer oral questions about the nature of fire. |
| --- | --- |
| CONDITIONS: | Given a five question oral quiz. |
| STANDARD: | To receive a go for this lesson unit, the student must answer all the questions correctly. |
| FORCE PROTECTION | There are no safety considerations for this lesson unit, however whenever dealing with fire or one of its extinguishing agents, extreme caution is always advised. |

| Safety Requirements | NONE |
| --- | --- |
| Risk Assessment Level | LOW |
| Environmental Considerations | NONE |
| Evaluation |  |
| Instructional Lead-in |  |

SECTION III - PRESENTATION

| Note: | We will begin this lesson unit by showing the VCT “Trial by Fire” (running time 20 minutes). |
| --- | --- |

| ACTION: |  |
| --- | --- |
| CONDITIONS: |  |
| STANDARD: |  |

| 1. | Learning Step/Activity 1 [A statement describing what is to be done.] |
| --- | --- |
|  | Method of instruction: C/TV Instructor to student ratio is 1:C. Time of instruction: .4 HR. MediaTV |
| NOTE: | Ask check and developmental questions throughout body. |
|  | INTRODUCTION TO SHIPBOARD FIRE FIGHTING. |
|  | A. Trial by Fire VCT. (20 minutes). 1. Introduce tape. 2. Show tape. 3. Critique tape. |
| NOTE: | Refer students to FM 55-501, pg. 11-1 thru 11-4. |
|  | B. Chemistry of Fire. 1. Oxidation - Is a chemical process in which a substance combines with oxygen. During this process energy is given off, usually in the form of heat. 2. Slow Oxidation - Rusting iron and rotting wood. 3. Rapid Oxidation - Fire or combustion is rapid oxidation; the burning substance combines with oxygen at a very high rate. Energy is given off in the form of heat and light. 4. All matter exists in one of three states: solid, liquid, or gas (vapor). a. Solid - Molecules are packed closely together. b. Liquid - Molecules are packed loosely. c. Gas - Molecules are not packed together at all; they are free to move about. 5. In order for a substance to oxidize, its molecules must be pretty well surrounded by oxygen molecules. The molecules of solids and liquids are to tightly pack to be surrounded. Thus, only vapors can burn. 6. Radiant Heat - Pure energy, the same sort of energy the sun radiates that we feel as heat. Its radiates, or travels in all directions, therefore part of it moves back to the seat of fire, to the “burning” solid or liquid (fuel). 7. Radiation feedback - Heat that radiates back to the fuel. |
| NOTE: | Ask check questions before proceeding to next learning step. |
| NOTE: | Develop on chalkboard and discuss. |
|  | C. The Fire Triangle. 1. Fuel - To vaporize and burn. 2. Oxygen - To combine with fuel vapor. 3. Heat - To raise the temperature of the fuel vapor to its ignition temperature. 4. Facts in preventing and extinguishing fires: a. If any side of the fire triangle is missing, a fire cannot start. b. If any side of the fire triangle is removed, the fire will go out. |
| NOTE: | Ask check question before proceeding to next learning step. |
|  | D. Fuels and Fuel Characteristics. 1. Solid Fuels - Most obvious are, wood, paper, and cloth. 2. Ignition Temperature - Is the lowest temperature at which sustained combustion will occur without the application of spark or flame. 3. Liquid Fuels - Most commonly found aboard ship are bunker fuel, lubricating oil, diesel oil, kerosene, and oil-based paints and their solvents. 4. Vapors - Flammable liquids release vapor in much the same way as solid fuels. The rate of vapor release is greater for liquids than solids, since liquids have less closely packed molecules. In addition, liquids can release vapor over a wide temperature range. 5. Flash Point - The flash point of a liquid fuel is the temperature at which it gives off sufficient vapor to form an ignitable mixture near its surface. 6. Gaseous Fuels - These are both natural and manufactured flammable gases. Examples found on board a vessel include acetylene, propane, and butanes. 7. Burning - Gaseous fuels are already in the required vapor state. Gases, like flammable liquids, always produce a visible flame; they do not smolder. |
| Note: | Conduct a check on learning and summarize the learning activity. Administer the oral quiz. |
| Review: | Using APC, critique the quiz. |
| Transition: | The next hour we will discuss classes of fire and their extinguishing agents. |

|  | ENABLING LEARNING OBJECTIVE |
| --- | --- |
| Attention | Gain class attention. Introduce lesson unit II. In the first hour we discussed the chemistry of fire. During this hour, we will discuss the classes of fire along with their extinguishing agent. |
| Motivator | Before you can properly fight a fire you need to know what class of fire it is so that you can choose the correct extinguishing agent. |

| ACTION: | After this lesson unit the student will be able to identify the classes of fire, and the proper extinguishing agent. |
| --- | --- |
| CONDITIONS: | Given class notes and a four question oral quiz. |
| STANDARD: | To receive a passing score, each student must answer all questions correctly. |
| FORCE PROTECTION: | There is not safety consideration for this lesson unit. |

|  | Learning Step/Activity 1 |
| --- | --- |
| Transition: | Let’s begin by discussing class “A” fires. |
|  | Method of instruction: C Instructor to student ratio is 1:C. Time of instruction: .9 hours. Media ______(State if there is none.)_____. |
| NOTE: | Ask check and developmental questions throughout body. |
|  | CLASSES OF FIRE/EXTINGUISHING AGENTS. |
|  | A. Classes of Fire. |
| NOTE: | Refer students to FM 55-501, pg. 11-4 thru 11-6. |
|  | 1. Class “A” Fires - Common flammable solid fuel (ash producing) 2. Class “B” Fires - Flammable liquid or gaseous fuel. 3. Class “A” and “B” Fires - Solid fuel combined with liquid or gaseous fuel. 4. Class “A” and “C” Fires - Solid fuel combined with electrical equipment. 5. Class “B” and “C” Fires - Liquid or gaseous fuel combined with electrical equipment. 6. Class “D” Fires - Combustible-metal fuel. |
| NOTE: | Ask check question before proceeding to next learning step. |
|  | B. Extinguishing Agents. 1. Definition - An extinguishing agent is a substance that will put out a fire. 2. Every extinguishing agent operates by attacking one or more sides of the fire triangle. a. Cooling - Reduces the temperature of the fuel below its ignition temp (heat side of the triangle). b. Smothering - Separates the fuel from the oxygen. c. Oxygen dilution - Reduces the amount of available oxygen below that needed to sustain combustion. |
| NOTE: | Ask check questions before proceeding to next learning step. |
|  | C. Types of Extinguishing Agents. 1. Liquids. a. Water spray. b. Foam. 2. Gases. a. Carbon Dioxide (CO2). b. Halon 1301. 3. Solids (Dry Chemical) a. Monoammonium Phosphate. b. Bicarbonate. c. Potassium Bicarbonate. d. Potassium Chloride. |
| NOTE: | Ask check question before proceeding to the next learning step. |
|  | C. Extinguishing Agents for the Different Classes of Fire. 1. Class A fire - Most effective extinguishing agent is water, a cooling agent. Foam and dry chemical may also be used; they act mainly as smothering agents. 2. Class B fire - A smothering agent is most effective. Water fog, dry chemical, foam, and carbon dioxide (CO2) may be used. Note if the fire is being supplied fuel by an open valve or a broken pipe, a valve o the supply side should be shut down. 3. Class A and B fires - Water spray and foam may be used to smother these fires. 4. Class A and C fires - Non-conducting extinguishing agent must be used. Carbon dioxide, Halon, and dry chemical are the most efficient agents. 5. Class B and C - Non-conducting extinguishing agent must be used. Halon or dry chemical acting as a chain breaker are most effective. 6. Class D fire - Generally smothered and controlled with specialized agents known as dry powders. Dry powders are not the same as dry chemicals. |
| Note: | Conduct a check on learning and summarize the learning activity. Administer oral quiz and give a go if all questions are answered correctly/ |
| Review: | Using APC, critique the quiz. |

| Transition: | The next hour we will discuss extinguishing methods and extinguishing agents. |
| --- | --- |
|  | ENABLING LEARNING OBJECTIVE C |
| Attention: | Gain class attention. Introduce lesson unit III. In the second hour we discussed the classes of fires and their extinguishing agents. During this hour we will discuss methods of extinguishing using the proper extinguishing agent. |
| Motivator: | There is a major fire on board your Army tug, you are the first fire fighter on the scene, would you know the correct method to extinguish this fire? |

| ACTION: | After this lesson unit the student will be able to utilize the proper extinguishing agent as well as the correct extinguishing methods. |
| --- | --- |
| CONDITIONS: | Given class notes and a four question oral quiz. |
| STANDARD: | To receive a passing score, each student must answer all questions correctly. |
| FORCE PROTECTION: | There is not safety consideration for this lesson unit. |

| 1. | Learning Step/Activity 1 |
| --- | --- |
| Transition: | Let’s begin by viewing the VCT Fire Prevention (running 14 minutes). |
|  | Method of instruction: C/TV Instructor to student ratio is 1:C. Time of instruction: .9 hours. Media TV |
| NOTE: | Ask check and developmental questions throughout body. |
| 3H+05 | EXTINGUISHING METHOD AND EXTINGUISHING AGENTS. |
|  | A. Fire Prevention (14 minutes). 1. Introduce tape. 2. Show tape. 3. Critique tape. |
| NOTE: | Refer students to FM 55-501, pg. 11-6 thru 11-10. |
|  | B. Water (cooling agent). 1. Straight stream - Sometimes called the solid stream, is the oldest and most commonly used form of water for firefighting. a. The distance that a straight stream travels before breaking up or dropping is called its reach. b. Less than 10 percent of the water from a straight stream actually absorbs heat from the fire. c. A straight stream should be directed into the seat of the fire. d. The main use of solid streams is to break up the burning material and penetrate to the seat of a class A fire. 2. Low-Velocity Fog Streams. a. Obtained by using an applicator along with a combination nozzle. b. Applicators are tubes or pipes that are angles at 60 degrees, or 90 degrees at the water outlet end. |
| NOTE | Refer students to FM 55-501, pg. 11-8, figure on low-velocity fog applicators. |
|  | c. Nozzles 1 1/2 inch. [1] Use 4 foot, 60 degree angle. [2] Use 10 foot, 90 degree angle. d. Nozzles 2 1/2 inch. [1] Use 12 foot, 90 degree angle. e. Uses of low-velocity fog. [1] Combating class B fires in spaces where entry is difficult. [2] Applicators can be poked into areas that can not be reached with other types of nozzles. [3] Can provide a heat shield for fire fighters advancing with foam or high-velocity fog. [4] Can be used to extinguish small tank fires, especially when the mist of the applicator can cover the entire tank. |
| NOTE: | Refer student to FM 55-501, pg. 11-9. |
|  | 3. Straight stream. a. Can be aimed accurately. b. Has good reach. c. Must hit seat of fire to cool efficiently. d. Runoff of water may be excessive. e. Generates very little stream. 4. Fog stream. a. Difficult to aim. b. Limited reach. c. Excellent cooling abilities. d. Generates steam. e. Has small amount of runoff. f. Pushes fire and smoke. g. Does not have to hit seat of fire to be effective. |
| NOTE: | Ask check question. |
|  | 5. Combination nozzle operation (Navy All Purpose Nozzle). a. Can produce a straight stream, obtained by pulling the nozzle handle all the way back toward the operator. b. Can produce a high-velocity fog, obtained by pulling the handle back halfway (perpendicular). c. The nozzle shut down is accomplished by pushing the handle as far forward as possible. |
| NOTE: | Demonstrate all three position on this nozzle using an NAP in the classroom. Ask for a volunteer from the students. 6. Foam. a. Is a blanket of bubbles that extinguishes fire mainly by smothering. b. Bubbles are formed by mixing water and a foam making agent (foam concentrate ). The result is called foam solution. c. Foam should be light enough to float on the surface of flaming liquids, but heavy enough to resist winds. d. It is used to form a blanket on the surface of flaming liquids, including oils. e. Two basic types. [1] Chemical foam - Formed by mixing an alkali (usually sodium bicarbonate) with an acid (usually aluminum sulfate) in water. [2] Aquenos Film Forming Foam (AFFF). 7. Advantages of foam. a. Effective smothering agent. b. Set up a vapor barrier that prevents flammable vapors from rising. c. Effective in blanketing oil spills. d. Foam is the most effective extinguishing agent for fires involving large tanks of flammable liquids. e. Made from fresh water or sea water, hard or soft water. f. Foam stays in place, covers, and absorbs heat from materials that could cause reignition. g. Uses water economically and does not tax the ship’s fire pumps. h. Foam concentrates are not heavy, take up little space. 8. Limitations on the use of foam. a. Electrically conductive and should not be used on live electrical equipment. b. Do not use on combustible-metal fires. c. Many foams must not be used with dry chemical extinguishing agents. d. Sufficient foam must be on hand to make sure that the entire surface of the burning material can be covered. 9. Mechanical (Air) foam - Produced by mixing a foam concentrate with water to produce a foam solution. |
| NOTE: | Inform the student this type of foam will be demonstrated on the PE portion of this lesson. |
| NOTE: DAY 2 | Ask check question. |
| NOTE | Give students a five minute break. |
|  | C. Carbon Dioxide - Extinguishes fires mainly by smothering. 1. Dilutes the air surrounding the fire until the oxygen content is too low to support combustion. 2. Limited cooling effect. 3. Effective on class B fires where the main consideration is to keep the flammable vapors separated from oxygen in the air. 4. Can be used on class A fires in confined spaces, where the atmosphere may be diluted sufficiently to stop combustion. 5. CO2 takes time. 6. Sometimes used to protect areas containing valuable articles, because it dissipates without leaving a residue. 7. Can be used on electrical fires. 8. Used primarily on class B and C fires. Some use on class A fires. 9. Not effective on substances that contain their own oxygen (oxidizing agents). 10. Fully effective the gas must be confined. 11. High possibility of flashback. 12. Not poisonous to the human system, but can in strong concentrations cause dizziness, unconsciousness, and even death. |
| NOTE: | Ask check question. |
|  | D. Extinguishing Effects of Dry Chemicals. 1. Cooling - Very low capacity, however because the dry chemical is a lower temperature than the fire some cooling occurs. 2. Smothering - Limited affect because of the reaction of the heat and burning material with the dry chemical. 3. Shielding of Radiant Heat - Dry chemicals produce an opaque cloud in the combustion area. This cloud reduces the amount of heat that is radiated back to the heart of the fire. |
|  | E. Uses of Dry Chemical. 1. Class A, B, and C fires and combinations of these. 2. Note that ABC dry chemical may only control, not extinguish, some deep-seated class A fires. |
|  | F. Limitations on the Use of Dry Chemicals. 1. Large amounts could affect people in the vicinity. 2. Are not effective on materials that contain their own oxygen . 3. May deposit an insulating coating on electronic or telephonic equipment, affecting the operation of the equipment. 4. Not effective on combustible metals and in some cases may cause a violent reaction. 5. Where moisture is present, a dry chemical agent may carrod or stain surfaces on which it settles. 6. Very effective on gas fires; however, gas flames should not be extinguished until the supply of fuel has been shut down upstream of the fire. |
| WARNING: | Dry chemical extinguishing agents are considered nontoxic but they may have irritating effects when breathed. |
| NOTE: | Ask check question. |
|  | G. Dry Powders. 1. Dry powders are the only extinguishing agents that can control and extinguish metal fires without causing violent reactions. 2. Mainly a smothering agent, limited cooling. |
| WARNING: | Dry powders are not the same as Dry chemicals. |
|  | H. Halogenated Extinguishing Agents (HALON). 1. Made up of carbon and one or more of the halogen elements: fluorine, chlorine, bromine, and iodine. 2. Enters the area as a gas. 3. Chain breaker. 4. Stored and shipped as a liquid under pressure. When released it vaporizes to an odorless, colorless gas and is propelled to the fire by its storage pressure, 5. Does not conduct electricity. |
| WARNING: | When inhaled, Halon 1301 may cause dizziness and impaired coordination. |
| Note: | Conduct a check on learning and summarize the learning activity. Administer the oral quiz. |
| Review: | Using APC, critique the quiz. |
| Transition: | The next hour we will discuss Portable Fire Extinguishers. |
|  | ENABLING LEARNING OBJECTIVE |
| Attention: | Obtain students attention, introduce self and lesson unit as appropriate. |
| Motivator: | The alarm for fire is sounded aboard your ¦Army Watercraft, and you are the first to respond. You discover a small fire in the galley. Would you know which portable fire extinguisher to use, and the correct method to use it? |

| ACTION: | After this lesson unit the student will answer oral questions on Portable Fire Extinguishers. |
| --- | --- |
| CONDITIONS: | Given a four question oral quiz. |
| STANDARD: | To receive a go for this lesson unit, the student must answer all the questions correctly. |
| FORCE PROTECTION: | There is not safety consideration for this lesson unit. |

| 1. | Learning Step/Activity 1 |
| --- | --- |
| Transition: | We will begin this lesson unit by showing the VCT “Using Fire Extinguishers” (running time 13 minutes). |
|  | Method of instruction: C/TV Instructor to student ratio is 1:C Time of instruction: . 3 HR. Media TV. |
| NOTE: | Ask check and developmental questions throughout body. |
|  | PORTABLE FIRE EXTINGUISHERS. |
|  | A. Using Fire Extinguishers. (13 Minutes) 1. Introduce tape. 2. Show tape. 3.. Critique tape. |
| NOTE: | Refer students to FM 55-501, pg. 11-16 thru 11-28. |
|  | B. Facts About Portable Fire Extinguishers. 1. Can be carried to a fire for fast attack. 2. Limited supply of extinguishing agent, usually one minute or less. 3. Extremely important to back up the extinguisher with a hose line. |
|  | C. Classification of Fire Extinguishers. 1. Classified in two ways. a. With one or more letters, the letter indicating what class of fire the extinguisher can be used on, for example AB can be used on a class A and B fire. b. With a numeral which indicates either the relative efficiency, or its size. c. The National Fire Protection Association (NFPA) rates extinguisher efficiency with Arabic Numbers. d. The Coast Guard uses Roman numerals to indicate the sizes of portable extinguishers. |
| NOTE: | Ask check question. |
|  | D. Test and Inspection. 1. Tested at least once in every 12 months. 2. When test are completed a tag will be placed on each extinguisher, showing the date and person who completed the test. 3. Any CO2 extinguisher that has lost more than 10 percent of its CO2 weight should be recharged by the manufacturer. Also after each use. |
| NOTE: | Refer student to FM 55-501. pg. 11-27 thru 11-28. |
|  | E. Portable Pumps. 1. P - 250 pump is a self priming 250 gpm portable pump with a 2 - cylinder, 2 cycle, 25 horse power engine. Explain the difference from the PE - 250 pump, which is electric start. 2. Designed for fire fighting and dewatering. |
| NOTE: | Use of the portable pump will occur during the PE phase of this lesson. |
|  | F. Fixed Fire Stations. 1. Purpose of the fire-main system is to deliver water to the fire stations that are located throughout the ship. 2. Weekly visual inspections of fire stations should be standard procedure to make sure that all required equipment is in its proper place. 3. Fire stations are located to insure that the water streams from at least two hydrants will overlap. Numbered in numerals at least 2 inches high. |
| NOTE: | Ask check questions. |
| NOTE: | Refer student to FM 55-501, pg. 11-30 thru 11-31. |
|  | G. Fire Hoses, Nozzles, and Appliances. 1. Hoses. a. Weather decks use 2 1/2 inch hose and enclosed areas use 1 1/2 inch hose. b. Hoses are 50 feet in length. c. Connected at all times, with the nozzle always attached. d. May not be used for any purpose other than fire fighting, testing, and fire drills. e. Stowed faked and racked. 2. Nozzle. a. NAP is used. 3. Appliances. a. Spanner wrench is a special tool designed for tightening or breaking apart connections. b. Pike head fire axe is a multipurpose, portable, fire-fighting tool. c. Axes should be inspected periodically and sharpened, cleaned, or repaired as necessary. |
| Note: | Conduct a check on learning and summarize the learning activity. Administer the oral quiz. |
| Review: | Using APC, critique the quiz. |
| Transition: | The next hour we will discuss use of the OBA. |
| NOTE: | Release class for a ten minute break. |
| Note: | Conduct a check on learning and summarize the learning activity. |
| E. | ENABLING LEARNING OBJECTIVE E |
| Attention: | Obtain students attention, introduce self and lesson unit as appropriate. |
| Motivator: | There is a major fire aboard your Army Watercraft, and a fellow crew member is unconscious in the Boatswain locker. If you were requested to donn an OBA and go in to recover this individual, would you have the knowledge to do so? |

| ACTION: | After this lesson unit the student will answer oral questions about the self-contained breathing apparatus. |
| --- | --- |
| CONDITIONS: | Given a four question oral quiz and class notes. |
| STANDARD: | To receive a go for this lesson unit, the student must answer all the questions correctly. |
| FORCE PROTECTION: | There is not safety consideration for this lesson unit. |

| 1. | Learning Step/Activity 1 |
| --- | --- |
| Transition: | We will begin this lesson by discussing the breathing apparatus. |
| 5H+.05 | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: 2.9 HR. Media ______(State if there is none.)_____. |
| NOTE: | Ask check and developmental questions throughout body. |
|  | SELF-CONTAINED BREATHING APPARATUS. |
|  | A. Breathing Apparatus. 1. Device that provides the user with breathing protection. 2. Includes a facepiece, body harness, and equipment that supplies air or oxygen. 3. Are available in several types, we are concerned with the OBA (oxygen breathing apparatus), and the SCOTT AIR PACK. |
|  | B. OBA. 1. Must not be used in any atmosphere that contains, has contained, or is suspected of containing flammable or combustible liquids or gases. 2. Never wear the OBA in a cofferdam or any compartment fouled by fuel oil. |
| NOTE: | Refer student to FM 55-501, pg. 11-34 thru 11-41. |
|  | C. Self-Generating (canister type) OBA. 1. The wearer’s exhaled breath reacts with chemicals in a canister to produce oxygen. This oxygen is then breathed by the wearer. 2. Canister-type unit consists of the five following parts. a. Facepiece with inhalation tube, exhalation tube, pressure relief valve. b. Breathing bag. c. Canister holder and canister. d. Manual timer. e. Breast plate with attached body harness. 3. Stored in a suitcase-type container with room for three canisters. 4. The manual timer is set when the equipment is put into operation. It gives an audible alarm to warn the operator when the canister is nearly expended. |
| NOTE: | Ask check question. |
| NOTE: | Refer students to FM 55-501, pg. 11-36 the illustration on the bottom of the page. |
|  | D. Operating Cycle (canister-type unit). 1. Exhaled air opens exhalation valve and is pushed into the canister. 2. Chemicals in the canister absorb CO2 and produce oxygen. 3. Pressure forces 02 into the breathing bag. 4. Inhaled breath opens the inhalation valve, drawing air from the breathing bags. 5. 02 flows into the facepiece through the inhalation tube. Note there is a pressure relief valve for excess 02 in the breathing bags. |
| NOTE: | Briefly go over putting on the OBA, and have the student follow along in their FM 55-501, pg. 11-37 thru 11-39. |
| WARNING: | If the lenses fog up, any part of the unit malfunctions, or the wearer experiences any discomfort or difficulty in breathing, he must immediately retreat to safety. |
| NOTE: | Go over removing the canister in FM 55-501, pg 11-39 thru 11-40, and have the student follow along. |
| NOTE: | Inform students they will donn and wear an OBA, in the PE portion of this lesson. |
|  | E. Self-Contained Demand Type Breathing Apparatus (Scott Air Pack). 1. It is convenient, supplies cool fresh air on demand to the user, is versatile, and can be put into action quickly. 2. Similar to Scuba gear in that it supplies the user with clean uncontaminated air, while in a contaminated environment. 3. Construction. a. Face piece. b. Regulator. c. Air Cylinder. d. Back pack or sling pack. |
| NOTE: | Inform students they will don and wear a self-contained breathing apparatus on the PE portion of this lesson. |
|  | F. Lifeline Signals Between OBA Wearer and Tender. 1. Definitions. a. Wearer - Person who actually wears the OBA. b. Tender - Person who monitors the lifeline in case of any problems. c. Lifeline - Small piece of line attached to the wearer and payed out to the tender for means of communication. 2. Signals. Pulls on Line Tender to Wearer Wearer to Tender 1 Are you alright? I am alright. 2 Advance. I am going ahead. 3 Back out. Take up my slack. 4 Come out immediately. Send help. |
| Note: | Conduct a check on learning and summarize the learning activity. Administer the oral quiz. |
| Review: | Using APC, critique the quiz. |
| Transition: | The next 09 hours we will put into practical use all of the things you have covered so far in Shipboard Fire Fighting. |

SECTION IV - SUMMARY

|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: .3 HR |
| --- | --- |
| Review/ Summarize Lesson | Review the previous 9 hours of instruction with focus on chemistry of fire, fire fighting equipment and techniques, classes of fire, along with their proper extinguishing agent. Review PE 421-110. |
| Check on Learning | Determine if students have learned the material presented by -- |
|  | a. Soliciting questions and explanations. |
|  | b. Asking questions and getting answers from the students. |
|  | c. Correcting student misunderstandings. |
| Transition To Next Lesson | Now that you have completed the practical exercise and learned how to fight Shipboard Fires , lets recapitulate what we have learned. |

SECTION V - STUDENT EVALUATION

| Testing Require- ments | Describe how the student must demonstrate accomplishment of the TLO. This subject matter is examinable on examination 9G6-102-02. |
| --- | --- |
| Note: | Refer student to the Student Evaluation Plan |
| Feedback Requirement | a. Schedule and provide feedback on the evaluation and any information to help answer students’ questions about the test. |
|  | b. Provide remedial training as needed. |
| Note: | Rapid, immediate feedback is essential to effective learning. |

PRACTICAL EXERCISE SHEET NO.

|  | Shipboard Fire Fighting. |  |
| --- | --- | --- |
| Intro- duction | Gain students attention and introduce the practical exercise. |  |
| Motivator | After becoming acquainted with all the fire fighting equipment aboard an Army LCU as well as the proper fire fighting techniques, you will have a good understanding of shipboard fire fighting when you are assigned to your own vessel. |  |
| Terminal Learning Objective | Note: Inform the students of the following terminal learning objective requirements. At the completion of this lesson you [the student] will: |  |
|  | ACTION: | The student will demonstrate his/her knowledge of shipboard fire fighting, and how to put them to actual use. |
|  | CONDITIONS: | Given PE 421-110, Sec. I, Army LCU, Fire Fighting Equipment, and class notes, the student will; |
|  | STANDARD: | Complete PE 421-110, Sec I within 9. hours. |

Method of instruction: PE1

Instructor to student ratio is: 1:6

Time of instruction: 7.0 HRS

| Safety Requirements | Safety briefing will be conducted prior to each task. |
| --- | --- |
| Risk Assessment Level | Medium |
| Environmental Consider- ations | [State if there are none.] |
| Evaluation |  |
| Instructional Lead-in |  |
| Resource Require- ments | Fire fighting facilities |
| Special Instructions | This PE require a fire fighting facilities Give the students normal class breaks during this PE. |
| After-Action Review: | conduct an After -Action-Review with focus on fire fighting techniques. |
| Procedures | 1. 2. |
| Feedback Requirements |  |