RAINING SUPPORT PACKAGE (TSP)

Note: This is the TSP title page

| TSP Number | 88K13C11 |
| --- | --- |
| Task Number(s) Title(s) | Hazcom Safety Briefing No Task Number Available |
| Effective Date | 1 Oct 97 |
| Supersedes TSP(s) | 421-146-01 13 Sep 94 |
| TSP User | Skill level 20, 30, 40 MOS 88 K |
| Proponent | TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street Suite 227 Ft. Lee, VA 23801-1511 |
| Comments/ Recommen- dations | Send comments and recommendations directly to: TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street Suite 227 Ft. Lee, VA 23801-1511 |
| Foreign Disclosure Restrictions | This product has been reviewed by the product developers in coordination with the Commander, U.S. Army Combined Arms Support Command foreign disclosure authority. This product is releasable to military students from foreign countries on a case-by-case basis. |

PREFACE

Purpose	This training support package provides the instructor with a

standardized lesson plan for presenting instruction for:

TASK NUMBER:

TASK TITLE: Hazcom Safety Briefing

ACTION: After this lesson unit, the student will demonstrate knowledge of occupational safety and health administration requirements for training in the workplace.

CONDITIONS: Given advance sheet, a one hour block of instruction and a five question quiz.

STANDARD: Students must answer all five questions correctly.

# This TSP

contains

| TABLE OF CONTENTS |  |  |
| --- | --- | --- |
|  |  | Page |
| Preface |  | 2 |
| Lesson | Section I - Administrative Data | 3 |
| Plan | Section II - Introduction | 6 |
|  | Terminal Learning Objective | 6 |
|  | Section III - Presentation | 7 |
|  | A. Enabling Learning Objective 1 | 7 |
|  | B. Enabling Learning Objective 2 |  |
|  | C. (etc.) |  |
|  | Section IV - Summary | 15 |
|  | Section V - Student Evaluation | 16 |
| Appendixes | A. Viewgraph Masters |  |
|  | B. Test and Test Solutions |  |
|  | C. Practical Exercises and Solutions |  |
|  | D. Student Handouts |  |

SECTION I - ADMINISTRATIVE DATA

| All Courses Including This Lesson | COURSE NUMBER(S) 88K10 ___________________ |  | COURSE TITLE(S ## Watercraft Operator ________________________________ |  |  |
| --- | --- | --- | --- | --- | --- |
| Task(s) Taught or Supported | TASK NUMBER ## None |  | TASK TITLE |  |  |
| Reinforced Task(s) | TASK NUMBER None |  | TASK TITLE |  |  |
| Academic Hours: | The academic hours required to teach this course are as follows: |  |  |  |  |
| TOTAL HOURS | PEACETIME HOURS/METHOD .3/VCT .3/VG/ .6/C / /0000000000 1.0 Hr. |  | MOBILIZATION HOURS/METHOD / / / / /000000000000000 |  |  |
| Test Lesson [List the lesson number in which the terminal learning objective of Number this lesson are tested and the test results are reviewed with the students.] |  |  |  |  |  |
|  |  |  |  | Hours | Lesson No. |
|  |  | Testing: |  | _______________ _______________ | _______________ _______________ |
|  |  | Review of test results: |  | _______________ | _______________ |

| Prerequisite Lesson(s) | (State if there are none.) LESSON NUMBER None | LESSON TITLE |
| --- | --- | --- |
| Clearance and Access | UNCLASSIFIED (U) |  |

| References |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Number | Title | Date | Para No. | Additional Information |
|  | FM 55-501 | Marine Crewman’s Handbook |  |  |  |

| Student Study Assignments | None |
| --- | --- |
| Instructor Requirements | None [These are special qualifications or additional requirements.] |
| Additional Support Personnel Requirements | None |
| Equipment Required | VCT. (VCR & TV ) Overhead (VG) Projector |
| Materials Required | Instructor: Chalkboard & Chalk, VCT. 30 minutes (“Hazcom Training Program”, Segments 1, 2A, 2B, and 3A.), Student Handout 431-002, Advance Sheet 421-146-01 / 421-265-01 / 421-350-01 / 421-480-01. Students: Pen Pencil & Notebook. |
| Classroom, Training Area, and Range Requirements | Standard Classroom |
| Ammunition Requirements | None |
| Instructional Guidance | Note: Ensure facilities, equipment, and training materials are ready prior to class start time. Instructors will have a thorough knowledge by studying this lesson and identified reference material. |

| Proponent Lesson Plan Approvals | NAME ___________________ ___________________ | Rank _____ _____ | Position __________________ __________________ | Date _______ _______ |
| --- | --- | --- | --- | --- |

SECTION II - INTRODUCTION

|  | Method of instruction: C/ VG /VCT ## Instructor to student ratio is 1:C Time of instruction: .2 hours. |
| --- | --- |
| ### Motivator | As a watercraft operator it is important to know the health risks, safety precautions, and controlling measures of the various chemicals used on the Army’s watercraft. |
| Show VG 2-11 Terminal Learning Objective | Note: Inform the students of the following terminal learning objective requirements. At the completion of this lesson you [the student] will: To fulfill occupational safety and health administration (OSHA), requirements for training in the workplace. |

| ACTION: | After this lesson unit, the student will demonstrate knowledge of occupational safety and health administration requirements for training in the workplace. |
| --- | --- |
| CONDITIONS: | Given advance sheet, a one hour block of instruction and a five question quiz. |
| STANDARD: | Students must answer all five questions correctly, to receive a “GO” for this lesson unit |

| Safety Requirements | [State if there are none.] None |
| --- | --- |
| Risk Assessment Level | Low |
| Environmental Considerations | [State if there are none.] None |
| Evaluation | Check on learning ( 5 Questions ) |
| Instructional Lead-in |  |

SECTION III - PRESENTATION

| Note: | Inform the students of the Enabling Learning Objectives requirements. |
| --- | --- |
| A. | ENABLING LEARNING OBJECTIVE A |

| ACTION: | After this lesson unit, the student will demonstrate knowledge of occupational safety and health administration requirements for training in the workplace. |
| --- | --- |
| CONDITIONS: | Given advance sheet, a one hour block of instruction and a five question quiz. |
| STANDARD: | Students must answer all five questions correctly, to receive a “GO” for this lesson unit. |

|  | Learning Step/Activity 1 [A statement describing what is to be done.] |
| --- | --- |

|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: .9 Media VG / VCT_(State if there is none.)_____. |
| --- | --- |
|  | A. Hazard Communication Standard. 1. “The hazard communication standard was issued by the occupational safety and health administration (OSHA) in 1983 and revised in 1987. Executive order 12196 of 1980 and code of federal regulations part 29, 1960 provide the authority for implementing this standard within the federal sector. The hazard communication standard helps protect your right to work in a safe and healthful environment. It requires that you be:” a. “Informed about hazardous chemicals in your workplace.” b. “Trained to work safely with these materials.” |
|  | 2. Goals of the hazard communication standard. a. ”Reduce the incidence of injury and illness caused by hazardous chemicals in the workplace. b. “Identify and evaluate chemical hazards.” c. “Establish uniform requirements for communicating information about chemical hazards to both management and workers.” |
|  | 3. Actions required by chemical manufacturers and importers. a. Identify hazards and appropriate control measures for the chemicals they produce. b. Label all containers of hazardous chemicals, identifying the material, appropriate hazard warnings, and the name and address of the responsible party. c. Obtain an accurate and up-to-date MSDS for each hazardous chemical, and provide a copy to every employer that purchases the chemical. d. Update new information to the MSDS within three months after becoming aware of such information. |
|  | 4. Employers are required to do the following. a. Maintain and make available to workers an MSDS for all hazardous chemicals they use. b. Insure containers of hazardous chemicals are properly labeled and tagged. c. Maintain and make available to workers an up-to-date list of all hazardous chemical materials present in the workplace. d. “Inform and train workers.” e. Maintain a written local hazard communication program. |
|  | B. Physical Forms and Exposure Hazards. 1. Physical hazards - Chemicals that cause explosion, fires, violent chemical reactions, or other hazardous situations. 2. Health hazards - Chemicals that can cause illness or injury when inhaled or swallowed, or through contact with the skin or eyes. |
|  | 3. The three basic forms of all chemicals: a. Solids. b. Liquids. c. Gases. |
|  | 4. Exposure routes - There are four ways that chemical s enter your body: a. Breathing/Inhalation. b. Skin/Eye contact. c. Skin Absorption. d. Swallowing/Ingestion. |
|  | 5. Degree of hazard - “The degree of hazard associated with exposure to health hazards depends on the following. a. Toxicity of the chemicals 1 Low 2 Medium 3 High b. Exposure route. c. Dosage: 1 How much 2 How long 3 How often d. Individual differences |
|  | C. Types of Physical and Health Hazards. 1. Physical hazards include: a. Compressed gases. b. Explosives. c. Fire hazards. d. Unstable/Reactive chemicals. |
|  | 2. Fire hazards include: a. Pyrophorics - Ignite spontaneously in air below 130 degrees. b. Flammable liquids. c. Combustible liquids. d. Oxidizers. |
|  | 3. Unstable/Reactive chemicals include: a. Decomposition hazards. b. Polymerization hazards. c. Water-reactive chemicals. |
|  | 4. Health hazards include: a. Irritants. b. Corrosives. c. Cryogenics d. Specific organ or system damaging chemicals. e. Sensitizers. f. Carcinogens. g. Reproductive hazards. |
|  | 5. Reproductive hazards include: a. Mutagens b. Teratogens |
|  | D. Controlling Chemical Hazards. 1. Three basic methods of controlling chemical hazards are: a. Engineering controls. b. Personal protective equipment (PPE) c. Administrative controls. |
|  | 2. Engineering controls include: a. Substitution. b. Isolation. c. General ventilation. d. Local exhaust ventilation. |
|  | 3. Personal protective equipment include: a. Protective gloves and clothing. b. Eye and face protection. c. Air-purifying respirators. d. Air supplied respirators. |
|  | 4. Administrative controls include: a. Documentation, information and training. b. Work practices. c. Housekeeping. d. Monitoring. |
| NOTE: | Show MSDS training aid to class. |
|  | E. Material Safety Data Sheets (MSDS) 1. General information contained on a MSDS: a. Name, address, and telephone number of the party responsible for preparing or distributing the MSDS. b. Name of the chemical and hazardous chemical inventory. c. Health hazards, including signs and symptoms of exposure. d. Precautions for safe handling and use. e. Identify all hazardous ingredients in a mixture. |
|  | 2. The following sections show physical hazard information on the MSDS: a. Fire and explosion hazard data section. b. Reactivity Data section. c. Precautions for safe handling and use section. |
| VCT | Introduce and show 30 minute VCT “Hazcom training program” segments 1, 2A, 2B, and 3A. |
| Note: | Conduct a check on learning and summarize the learning activity. Students will answer the following questions pertaining to the hazcom program. |
|  | Q. What are the requirements employers must meet in reference to hazcom? A. (1) Maintain/make available all MSDS to workers. (2) Insure hazardous chemical containers are properly tagged or marked. (3) Maintain/make available to workers a list of hazardous chemicals used in the workplace (4) Inform and train workers. (5) Maintain written hazcom program. Q. What are the three basic forms of chemicals? A. Solids, liquids, gases. Q. What are the four exposure routes? A. (1) Breathing/inhalation. (2) Skin eye contact. (3) Skin absorption. (4) Swallowing/ingestion Q. What are the three basic methods of controlling chemical hazards? A. (1) Engineering controls. (2) Personal protective equipment. (3) Administrative controls. Q. What does personal protective equipment include? A. (1) Protective gloves. (2) Eye and face protection. (3) Air-purifying respirators. (4) Air supplied respirators. |
| Note: | Conduct a check on learning and summarize the learning activity. |

SECTION IV - SUMMARY

|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: .3 HR |
| --- | --- |
| Review/ Summarize Lesson | A. Review last hour of instruction. B. This block if instruction is not examinable. |
| Check on Learning | Determine if students have learned the material presented by -- AFTER-ACTION-REVIEW: Critique quiz in class in the form of an after-action-review. |
|  | a. Soliciting questions and explanations. |
|  | b. Asking questions and getting answers from the students. |
|  | c. Correcting student misunderstandings. |
| Transition To Next Lesson | This completes the hazcom briefing. Your next lesson will be..... |

SECTION V - STUDENT EVALUATION

| Testing Require- ments | Describe how the student must demonstrate accomplishment of the TLO. |
| --- | --- |
| Note: | Refer student to the Student Evaluation Plan |
| Feedback Requirement | a. Schedule and provide feedback on the evaluation and any information to help answer students’ questions about the test. |
|  | b. Provide remedial training as needed. |
| Note: | Rapid, immediate feedback is essential to effective learning. |

PRACTICAL EXERCISE SHEET NO.

| Title |  |  |
| --- | --- | --- |
| Intro- duction |  |  |
| Motivator |  |  |
| Terminal Learning Objective | Note: Inform the students of the following terminal learning objective requirements. At the completion of this lesson you [the student] will: |  |
|  | ACTION: |  |
|  | CONDITIONS: |  |
|  | STANDARD: |  |

| Safety Require- ments | [State if there are none.] |
| --- | --- |
| Risk Assessment Level |  |
| Environmental Consider- ations | [State if there are none.] |

| Evaluation |  |
| --- | --- |
| Instructional Lead-in |  |
| Resource Require- ments |  |
| Special Instructions |  |
| Procedures | 1. 2. |
| Feedback Require- ments |  |