TRAINING SUPPORT PACKAGE (TSP)

Note: This is the TSP title page

| TSP Number | 88K13B07 |
| --- | --- |
| Task Number(s) Title(s) | TEMPORARY HULL REPAIR |
| Effective Date | 1 OCTOBER 1997 |
| Supersedes TSP(s) | 421-113-03 1 OCTOBER 1988 |
| TSP User | SKILL LEVELLS 20, 30, 40 |
| Proponent | TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street, Suite 227 Fort Lee, VA 23801-1511 |
| Comments/ Recommen- dations | Send comments and recommendations directly to: TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street, Suite 227 Fort Lee, VA 23801-1511 |
| Foreign Disclosure Restrictions | This product has been reviewed by the product developers in coordination with the Commander, U.S. Army Combined Arms Support Command foreign disclosure authority. This product is releasable to military students from foreign countries on a case-by-case basis. |

PREFACE

Purpose	This training support package provides the instructor with a

standardized lesson plan for presenting instruction for:

TASK NUMBER: NONE

TASK TITLE: TEMPORARY HULL REPAIR

CONDITIONS: Giving class note, the evaluation plan and five questions quiz.

STANDARD: To receive a “GO” for this lesson Unit the student will answer five questions correctly.

This TSP

contains

| TABLE OF CONTENTS |  |  |
| --- | --- | --- |
|  |  | Page |
| Preface |  | 2 |
| Lesson | Section I - Administrative Data | 3 |
| Plan | Section II - Introduction | 6 |
|  | Terminal Learning Objective | 6 |
|  | Section III - Presentation | 8 |
|  | A. Enabling Learning Objective 1 | 11 |
|  | B. Enabling Learning Objective 2 |  |
|  | C. (etc.) |  |
|  | Section IV - Summary | 22 |
|  | Section V - Student Evaluation | 23 |
| Appendixes | A. Viewgraph Masters |  |
|  | B. Test and Test Solutions |  |
|  | C. Practical Exercises and Solutions | 30 |
|  | D. Student Handouts |  |

Temporary Hull Repair

[Effective Date]

SECTION - I. ADMINISTRATIVE DATA

| All Courses Including This Lesson | COURSE NUMBER(S) # 88K10 |  | COURSE TITLE(S) ## Water Craft Operator |  |  |
| --- | --- | --- | --- | --- | --- |
| Task(s) Taught or Supported | TASK NUMBER ## None |  | TASK TITLE |  |  |
| Reinforced Task(s) | TASK NUMBER ## None |  | TASK TITLE |  |  |
| Academic Hours: | The Academic hours required to teach this course are as follows: |  |  |  |  |
| TOTAL HOURS | PEACETIME HOURS/METHOD 3.7/C 2.0/PE1 .3/TV /0000000000 6 |  | MOBILIZATION HOURS/METHOD / / /000000000000000 |  |  |
| Test Lesson [List the lesson number in which the terminal learning objective of Number this lesson are tested and the test results are reviewed with the students.] |  |  |  |  |  |
|  |  |  |  | Hours | Lesson No. |
|  |  | Testing: |  |  |  |
|  |  | Review of test results: |  |  |  |

| Prerequisite Lesson(s) | (State if there are none.) LESSON NUMBER # None | LESSON TITLE ## None |
| --- | --- | --- |
| Clearance and Access | UNCLASSIFIED (U) N/A |  |

| References |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Number | Title | Date | Para No. | Additional Information |
|  | FM 55-501-2 | Haborcraft Crewman Handbook |  | CH.10 |  |

| Student Study Assignments | See Student Packet |
| --- | --- |
| Instructor Requirements | [These are special qualifications or additional requirements.] |
| Additional Support Personnel Requirements | None |
| Equipment Required | Damage Control Kit |
| Materials Required | Standard Classroom Materials |
| Classroom, Training Area, and Range Requirements | Classroom and Damage Control Facility |
| Ammunition Requirements | None |
| Instructional Guidance | Note: Ensure facilities, equipment, and training materials are ready prior to class start time. Instructors will have a thorough knowledge of reference. Prepare by studying this lesson and identified reference material. |

| Proponent Lesson Plan Approvals | NAME ___________________ ___________________ | Rank _____ _____ | Position __________________ __________________ | Date _______ _______ |
| --- | --- | --- | --- | --- |

SECTION II - INTRODUCTION

|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: .2 HR |
| --- | --- |
| Motivator | No matter what your job or position aboard your vessel may be, you must be able to function as a member of an emergency team member. Everyone from the skipper to the lowest deckhand, must be able to apply the knowledge you will gain from this block of instruction. During this period we will instruct you in the principals of Temporary hull repair. This is commonly called “Damage Control”. This term will be used during the class. Both terms are more or less the same thing. A Practical Exercise will be held at the conclusion of this block of instruction to reinforce what you have learned. |
| Terminal Learning Objective | Note: Inform the students of the following terminal learning objective requirements. At the completion of this lesson you [the student] will: |

| ACTION: | Temporary Hull Repair |
| --- | --- |
| CONDITIONS: | After this lesson the student will define the emergency squad signal used, the compartmentation numbering and the objectives of the DC team |
| STANDARD: | To receive a “GO” for this lesson the student must answer 4 of the 5 questions correctly within the time allowed |
| FORCE PROTECTION | This lesson do not require force protection |

| Safety Requirements | [State if there are none.]None |
| --- | --- |
| Risk Assessment Level | Low |
| Environmental Considerations | [State if there are none.]None |
| Instructional Lead-in |  |

SECTION III - PRESENTATION

|  | ENABLING LEARNING OBJECTIVE |
| --- | --- |
|  | INTRODUCTION TO DAMAGE CONTROL. |
| Motivator: | During this period we will cover the basic damage control organization aboard a typical vessel. We will cover the emergency signals commonly used to alert the crew. We will cover compartmentation, numbering system, and some of the objectives of the DC team. |

| ACTION: | After this lesson the student will define the emergency squad, signals used, the compartmentation, numbering and the objectives of the DC team. |
| --- | --- |
| CONDITIONS: | Given a 5 question quiz and without reference to class notes. |
| STANDARD: | To receive a “GO” for this lesson the student must answer 4 of the 5 questions correctly within the time allowed. |

|  | Learning Step/Activity 1 [A statement describing what is to be done.] |
| --- | --- |
|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: 3.2 HR Media ______(State if there is none.)_____. |
| NOTE: | Gain students attention, ask questions from the reading assignment or give a pop quiz if desired. |
|  | A. Organization of the Damage Control Team Aboard Ship. 1. This team will usually consist of the following personnel (this is “typical”). a. Chief Engineer. b. Engineer. c. Bos’n. d. Two or more seaman. e. Engineman (two or more). 2. This DC team should have at least 90 percent of its work accomplished before getting underway. Should an emergency occur while at sea, its too late to provide a crossout saw to the scene if you failed to procure one while in port. 3. Signals used for various emergencies. a. Fire and Emergencies - b. Abandon Ship - More than 6 short blasts and one long blast on the whistle and the same signal on the general alarm bell. c. Man Overboard - Hail and pass the word “Man Overboard” to the bridge. (Hail means to shout). Indicate which side the person fell overboard. Point to the person to help the bridge locate him. d. Dismissal - From fire and emergency stations is 3 short blasts on the whistle followed by the same signal on the alarm. 4. Drills are required to be held weekly. |
|  | B. Compartmentation Aboard Ship. |
| NOTE: | Use chalkboard to describe shipboard compartmentation. |
|  | 1. Horizontal division. a. Decks are numbered from the main deck both upward and downward. b. Below the main deck, the numbers will increase. Example 1, 2, 3, and so on. c. Above the main deck, the numbers will also increase but will have a zero preceding it. Example 01,02,03 and so on. 2. Vertical division. a. Uses bulkheads to mark vertical boundaries. b. Vertical and horizontal division make up compartments. 3. Numbering system. a. First two numbers indicate which deck. b. Second two numbers indicate the frame number where the compartment starts (usually the forwardmost frame). c. Single or double numbers indicate the compartments relation to the centerline. d. Single or double letter indicate the use of the compartment. e. Compartment use list (partial list). A - Storeroom. AA - Cargo storeroom. C - Ship control. E - Engineering space. F - Fuel (tanks) L - Living space. V - Void. W - Water (tanks) f. Compartment 01-46-0-C would be located on the 01 deck (one deck above the main deck) starting at frame 46, would be located on the centerline and it is used for ship control. This is probably the wheelhouse on an LCU. |
|  | C. Material Conditions of Readiness. 1. Three conditions of readiness. 2. Condition X-Ray. a. Black “X” on white or metallic background. Will be on the fitting to be closed or close to the fitting. b. Closed at all times unless actually in use. This condition is set at all times aboard ship. It is the lowest most relaxed setting aboard. c. Examples of this setting are the ships magazines, storerooms, and void. d. Can be modified. 3. Condition YOKE. a. Black “Y” on white or metallic background. Will be on or close to the fitting. b. Will be set at sea in peacetime. It is also set when entering or leaving port. c. During wartime it will be set while in port. This provides the second step in setting the basic battle condition. d. This condition may be modified to meet existing conditions. 4. Condition ZEBRA. a. This is a “RED Z” on a metallic or white background. b. This is the third step or battle condition. This will be set for fire, collision. General emergency and enter or leaving port in wartime. c. Do not open an X,Y, OR Z fitting without permission from the proper authority. d. This setting may also be modified. These fittings will have a circle around the X,Y or Z. This means you may open the fitting to pass and close it after you. |
|  | D. Scope of Damage Control. 1. Damage control covers both battle and non-battle damage caused by: a. Fire. b. Collision. c. Grounding. d. Explosions. e. Loading incorrectly or too heavy a load. 2. Personnel assigned to the Emergency squad must have a good knowledge of the vessel, its construction, characteristics, compartments, and stability. Since you may be needed to replace one of the members of the regular squad, you must know the above items. |
|  | E. Objectives of Damage Control. 1. Prevention - Before damage control occurs, insure that watertight and fumetight fittings are in place and working. Remove and eliminate fire hazards. The vessels watertight integrity is what keep you floating should you start to take on water. 2. Control of damage - Minimize and localize damage when it occurs by control of flooding and preserving stability and bouyage. Use firefighting procedures, patching, plugging procedures along with first aid to personnel. 3. Repair - Accomplish emergency repairs as quickly as possible. This includes emergency power, replace structure and stability. |
|  | F. Training of Crewmembers. We mentioned earlier who would normally be assigned to the emergency squad. All crewmen should be trained in Damage Control procedures. |
| Note: | Conduct a check on learning and summarize the learning activity. |
| Check-On Learning: | Administer the attached quiz and allow five minutes for completion. |
| Review: | Using the APC method, critique the quiz in form of an after action review. |
| Transition: | We will discuss the types of shoring, plugs and patches and their usage during the next hour. |

|  | ENABLING LEARNING OBJECTIVE B |
| --- | --- |
|  | PLUGS, PATCHES, AND SHORING MATERIAL. |
| Motivator: | During this hour we will discuss some of the first actions to take with respect to Damage Control. We are talking about plugs, patches and shoring material. This is usually the first action taken to control damage. This material is made up prior to its use. This is part of the 90 percent we spoke of earlier. |

| ACTION: | After this lesson unit the student will state the first actions to be taken following a damage control situation. |
| --- | --- |
| CONDITIONS: | Given a 5 question quiz and without class notes. |
| STANDARD: | To receive a “GO” for this lesson, the student must answer 4 of the 5 questions correctly within the time allowed. |

|  | Learning Step/Activity 1 |
| --- | --- |
|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: .9 HR Media ______(State if there is none.) N/A_____. |
| NOTE: | Have the damage control static display set up to demonstrate the use of plugs, patches, and shoring. If the trainer is not available use the chalkboard to demonstrate their uses. |
| NOTE: | Ask frequent questions throughout the body. |
|  | A. Plugs. 1. Almost anything available can be used to plug a hole. Your ship will have a large assortment of plugs available to the repair lockers for use when needed. |
| NOTE: | Show an assortment of plugs and how they are used on the damage control trainer. |
| NOTE: | Have students turn to page 10-2 in FM 55-502-2. |
|  | 2. This graph will show how much water will be admitted in an non-plugged hole versus a partially plugged hole. |
|  | B. Patches. |
| NOTE: | Use the D/C trainer to demonstrate the various patches. |
| NOTE: | Demonstrate this type of patch on the DC trainer. |
|  | 1. Hinged or folding plate patch. This is simply two half-circles of either wood or metal that is hinged to allow folding a length of line is attached in the center to provide a tie off point. “Gasket” material is used to seal the patch. It can be used without going on the outside the ship. 2. Plate patch. Made from a length of steel plate. This plate patch is usually spot welded to the sides of the vessel. It may be tied off with lengths of line attached to its four corners. Gasket material (such as pillows and blankets) may be used to seal the patch. 3. Flexible plate patch. Same as the above except the metal is lighter and more flexible. It is used to seal the curved sides of the vessel. Strips of metal are provided every 6 to 8 inches to provide rigidity. 4. Collision mat. Large canvas mat covered on one side with long hairlike fibers. Used line fibers are worked into one side of the canvas. This is called “Thrumming”. Another canvas layer is sewn to it so that the thrumming acts as a gasket. Four lines are used to anchor the mat when in place. This should not be used where fire may occur. It can be a fire hazard. 5. Box patch. This particular patch is used over a jagged hole that has edges protruding inward. It is a box with one side open that is fitted over the hole. Again, some type of gasket material is used to seal the leak. It may be held in place with either shoring or a system of hookbolts. 6. Hookbolts. This is essentially a long bolt that is threaded on one end and the other end is shaped into either a T,J, or L shape. These bolts are provide with washers and wingnuts to facilitate tightening with the fingers. 7. Folding T hookbolt. This is much the same except one end is free to move. It will fasten objects at angles of less than 90 degrees. |
|  | C. Shoring and Bracing Material. 1. Clear the decks of wreckage. 2. This is a process of placing supporting beams either above, below, or against a structure. It is used to brace a patch or to prevent sagging, metal fatigue or bulging. 3. Good judgment is necessary when erecting shoring - if you have doubt that a structure will hold - SHORE IT UP. 4. Some indicators are listed below to help you in the decision to shore. a. Deep bulges in metal plating. b. Bowed frames and stanchions. c. Loose or missing rivets or welds and seams leaking. d. Bulkheads “panting” (bending with the sea). 5. Shoring material. a. We use the following types of wood for shoring. [1] Douglas fir. [2] Yellow pine. [3] Spruce. [4] Hemlock. b. Douglas fir and Yellow pine are the primary choices of wood used for shoring. c. Hemlock and spruce are secondary choices for shoring use. d. Shoring is never painted, however it may be treated with a fire resistant chemical. e. Shores usually are received on board in 20 feet lengths. f. These shores are distributed about ship it in “accessible areas. If all possible they are located above the waterline. g. Maximum length of shores are 30 times the maximum width, for example a 2x4 can be only 5 feet long. |
|  | D. Types of Shoring Structures. 1. Direct compression. |
| NOTE: | Demonstrate the types of structures either on the chalkboard or utilize the DC trainer. |
|  | a. Pressure is acting on the axis. b. Will support a greater load of weight. c. This type of structure will be used to secure a patch in place or to hold a hatch shut. Strongest type because it is at a 90 degree angle. 2. Cross-axial. a. Pressure is acting parallel to the axis. b. This is not as strong as the Direct pressure of shoring. c. This type of structure should be used as much as possible. 3. Triangulation. a. Both shores are under direct compression. b. Ends are cut with a combination of degrees that equal 90. c. Install at angles of less than 90 degrees. |
|  | E. Wedges. |
| NOTE: | Show an assortment of wedges to the class. Wedges provide a means of tightening or adjusting shoring. |
|  | 1. Made from the same material as the shoring material. It is a triangular block of wood. It is cut with a course saw so that the wood will soak up water and swell. This will tighten the repair and tend to seal small holes such as plating. The width of the wedge is fitted to the width of the shore. 2. The wedge is 5 to 6 times the thickness of the butt end, for example a 2 inch butt will mean the wedge will be 12 inches long. 3. Installing wedges. a. Width of wedge is fitted to the width of the shore. b. Rough cut of wedge to rough cut of the shore. c. Use wedges in pairs. Two wedges equal a rectangle. d. Use equally weighted hammers or mauls and drive wedges in simultaneously. 4. Drill two small holes at each end of a small crack then seal with wedges. The small holes stop the crack from getting bigger. 5. If wedges need to be secured use a cleat. |
|  | F. Shole: This is a flat block of wood that is used to distribute pressure. It is generally made of soft wood and is about 1 X 8 inches long. It is normally made up on scene to fit the requirements. |
|  | G. Strongback: Quite simply this is used to distribute pressure or as an anchor over a hole. It can be made from a shore or some heavy plank.. |
| Check-On Learning: | Administer the attached quiz and allow five minutes for completion. |
| Review: | Using the APC method, critique the quiz in form of an after action review. |
| Transition: | During the next hour we will describe how to measure and cut shoring to install structure. |
| Note: | Conduct a check on learning and summarize the learning activity. |
|  | ENABLING LEARNING OBJECTIVE |
|  | MEASURING AND CUTTING SHORES. |
| Motivator: | We have covered material that will be necessary to have aboard your ship for the control of damage. Since we could not possibly prefabricate shoring for all instances where it could be used, it therefore stands that you must be able to cut shoring to the correct size. During this hour we will discuss how to accurately measure and cut shoring for repairs. |

| ACTION: | After this lesson the student will answer questions concerning measuring and cutting shoring. |
| --- | --- |
| CONDITIONS: | Given a five question quiz and without class notes. |
| STANDARD: | To receive a “GO” for this lesson the student must answer 4 of the 5 questions correctly within the time allocated. |

| 1. | Learning Step/Activity 1 [A statement describing what is to be done.] |
| --- | --- |
|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: .1 HR Media ______(State if there is none.)_____. |
| NOTE: | We will start by looking at what measurements should be set up in front of the class. |
| NOTE: | All measuring and cutting material should be set up in front of the class. |
| NOTE: | Gain students attention, ask questions during the conference period. |
|  | A. Using the Carpenter’s Square to Cut Shoring. 1. Use a tape measure or folding rule to measure the height of the compartment. This measurement will be from the center of the strongback to the deck. This is the vertical measurement. |
| NOTE: | Use the Chalkboard to explain these steps. |
|  | 2. Again using a tape or folding rule measure from the anchor point to the bulkhead and subtract from the measurement the thickness of the strongback. All measurements must be to the nearest 1/16 inch. 3. To find the length of the shore it is first necessary to lay off the two measurements described above on the carpenters square. Use a scale of one inch to one foot. 4. Lay off the vertical measurement on the “tongue” of the square and horizontal measure on the “body”. |
| NOTE: | Explain the “tongue” and “body” of the Carpenter’s square. |
|  | 5. From these two marks on the square, simply measure the diagonal line between the two for the length of the shore. |
| NOTE: | Have students turn to table found in handout 421-301. |
|  | 6. Place the carpenter’s square on the shore to be out. Align the square with the top of the shore and the vertical and horizontal measurements marked on the square. Mark the end of shore and cut. 7. After this cut, mark the center of stock. Lay the carpenter’s square on the cut side and mark a 90 degree angle from the center of stock. 8. One end of the shore is now is ready for use. 9. Measure for the length of the shore along the center of stock. Reverse the angles on the square for this end. 10. Your shore is now cut to fit a specific location. 11. Cutting is usually done with a handsaw circular saw or chisel, axes, hatchets or a chain saw. |
|  | B. Using the Shoring Batten. 1. After the preceding discussion concerning measurement and cutting of shores, I’m sure you will be pleasantly surprised *** to find that the shoring batten is much more accurate and provides very quick results. |
| NOTE: | Have shoring batten ready for demonstration. |
|  | 2. It consists of two lengths of wood joined by a strip of metal and secured with a wing nut. 3. A 90 degree angle is located on each end which is secured by a wing nut. When this nut is not secured the angle will rotate. 4. This device is NOT in the supply system. Units must fabricate the shoring batten from material available on the ship. 5. In use, the device is placed against the anchor point, unlocked and extended to the strongback. It is looked again, and handed out to the person cutting the shore. He lays the batten on the shore, subtracts a half inch for wedges , marks and cuts the lumber. 6. Each vessel should have several of these handy devices available in several locations. |
|  | C. Trimming Shores. 1. Shoring should be trimmed so that chipping and splitting do not occur. 2. The ends of shores should not have a sharp point. When subjected to heavy pressure the shore will tend to slip, curl, or work loose in moderate seas. 3. Shoring that is subjected to very heavy compression should have the ends cut at a 90 degree angle. 4. To increase holding power, sockets may be cut into other shores. |
|  | D. Types of Pipe Patching. 1. Thumb and C-clamp - Uses a block of soft wood that has been roughly shaped to fit the pipe and held in place with either a thumb or C-clamp. 2. Soft patch - Use a soft wood plug or a wedge to drive in the split, cut in off, then use gasket material to seal the area off. You may then wrap the outside with either marline, or other suitable material. 3. Jubilee patch - Uses a prefabricated cylinder or metal with flanges and holes in pipe. Wing nuts are provided to anchor the patch. Gasket material may be used to patch high pressure lines that have been damaged. A mix of liquid resin and hardening agent is smeared over fibrous glass material. |
| Note: | Conduct a check on learning and summarize the learning activity. |
| Check-On Learning: | Administer a five question quiz and allow 5 minutes to complete. |
| Review: | Using the APC method, critique the quiz in form of an after action review. |
| Transition: | During the next hour we will discuss the use of the dewatering pump aboard ship and how it is used. |

|  | ENABLING LEARNING OBJECTIVE |
| --- | --- |
|  | DEWATERING PUMPS. |
| Motivator: | During the preceding 3 hours we have followed a sequence of events from the initial call for the emergency team to various types of ways to “fix” the problem. One problem now remains for us to complete, repairs. The vessel has several ways to remove water. You should have a general idea of the systems used aboard ship for this purpose. |

| ACTION: | After this lesson unit the student will identify the systems used for dewatering a vessel. |
| --- | --- |
| CONDITIONS: | Given a 5 question quiz and without class notes. |
| STANDARD: | To receive a “GO” for this lesson the student must answer 4 of the 5 questions correctly within the time allowed. |

| 1. | Learning Step/Activity 1 [A statement describing what is to be done.] |
| --- | --- |
|  | Method of instruction: C/TV Instructor to student ratio is 1:C Time of instruction: 1.0 HR Media ______(State if there is none.)_____. |
| NOTE: | We will begin by discussing the drainage system aboard ship. |
| NOTE: | Gain students attention, ask questions throughout the lesson. |
|  | A. Drainage Systems (without pumps). 1. Plumbing and deck drains. a. Very simple drainage system which used gravity to drain some compartments. Normally the extreme forward and after compartment. b. This system is found mostly above the waterline and on the weather decks. c. A danger may exist with this type of system because it could back up and flood the compartment. 2. Main drainage system. a. Serves the machinery spaces such as the engine room. b. Larger ships will have secondary systems for auxiliary spaces. c. This system has its own eductor to pump waste over the side. d. Smaller vessels will have a primary system only. e. The overboard discharge connection is located on the port and starboard side to pump excess water overboard. f. The drains are used to let water flow from forward to the primary drainage system and then pumped over the side. |
|  | B. Types of Emergency Pumps. 1. Electric submersible pump. a. Available in two types either AC or DC. b. Used to pump uncontaminated fresh or salt water. c. It may be used to pump fuel oil or JP5 for short period in an emergency. d. Never pump low ignition fuels or hot water, this will crack the casing or cause an explosion. e. These pumps are either AC or DC, and will pump a minimum of 160 gallons per minute at a 70 foot head. 2. PE-250 pump. 1. The primary purpose of the pump is for fighting fires. Its secondary purpose is dewatering. 2. The pump is a self-priming., 250 gallon per minute, two cylinder, two cycle 25 horsepower engine. The engine is water cooled by water being circulated through the block. The water only circulates when the engine without having the intake hose submerged. The intake is a 3 inch diameter, hard rubber hose (usually black) available in 10 feet sections. 3. The discharge side is a two and one half inch outlet. This outlet will use a tri-gate to accept the use of a one and half inch fire hose. |
| NOTE: | Show film 551-0537-B and 551-0538-B “Emergency Fire Pumps Parts 1 and 2. |
| NOTE: | Show film MN-10036/Z4978 “Seven Sailors” |
| Note: | Conduct a check on learning and summarize the learning activity. |
| Check-On Learning | Administer a 5 question quiz and allow 5 minutes to complete. |
| Review: | Critique the quiz using the APC method in the form of an after action review. |
|  | The remaining time will be used to practice what you have learned. The Damage Control Trainer located behind Building 2716-K* will be used for this training. |

SECTION IV - SUMMARY

|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: .2 HR |
| --- | --- |
| Review/ Summarize Lesson | Review last 4 hours. |
| Check on Learning | Determine if students have learned the material presented by -- |
|  | a. Soliciting questions and explanations. |
|  | b. Asking questions and getting answers from the students. |
|  | c. Correcting student misunderstandings. |
| Transition To Next Lesson | [If applicable] |

SECTION V - STUDENT EVALUATION

| Testing Requirements | Describe how the student must demonstrate accomplishment of the TLO. The information contained in this lesson plan will be tested on IBM Examination 9G8-105-02. |
| --- | --- |
| Note: | Refer student to the Student Evaluation Plan |
| Feedback Requirement | a. Schedule and provide feedback on the evaluation and any information to help answer students’ questions about the test. |
|  | b. Provide remedial training as needed. |
| Note: | Rapid, immediate feedback is essential to effective learning. |

PRACTICAL EXERCISE SHEET NO.

| Title | Temporary Hull Repair |  |
| --- | --- | --- |
|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: 2.0 HRS |  |
| Introduction | During the next hour you will be assigned to a billet as you would be on board ship. You will be required to react to a Damage Control situation by plugging, patching and building structure to contain damage. You may or may not be required to dewater the compartment. |  |
| Motivator | You will be divided into the approximate number of crew board ship. One student will be in charge of the DC team. He/she will assign responsibilities to the remainder of their crew. |  |
| Terminal Learning Objective | Note: Inform the students of the following terminal learning objective requirements. At the completion of this lesson you [the student] will: |  |
|  | ACTION: | During the Practical Exercise the student will supervise emergency patching, shoring and dewatering operations under simulated shipboard conditions. |
|  | CONDITIONS: | Given a Damage Control trainer, all necessary tools, equipment, lumber, patching material, dewatering pump, and hoses. |
|  | STANDARD: | To receive a “GO” for this exercise the student must control the inflow of water with the material provided. The exercise is considered to be successful if the water does not reach the lower level of the porthole, or water is being pumped out. |

| Safety Require- ments | Read safety statement to assembled students. Statement is contained elsewhere in this lesson plan. |
| --- | --- |
| Risk Assessment Level | Medium |
| Environmental Considerations | [State if there are none.] |
| NOTE: | Depending on size of class, divide students into groups similar to the size of an Emergency Squad aboard ship. |
| Instructional Lead-in |  |
| NOTE: | The following items will be done prior to commencing the exercise: 1. The Assistant Instructor (AI) will coordinate with the Engineering Committee to arrange for pumps, lumber, tools, and equipment needed to conduct the PE. 2. Students will arrange the material so that it is accessible to the scene but away from overflow from the trainer. 3. Designate one student to act as the senior person. He will designate the team members duties. The Senior member should not be the highest ranking student. 4. Designate one person to stand by the fire hydrant for emergency shut-down if necessary. 5. Any of the following conditions shall indicate a termination of the exercise: a. Water over the lower edge of the porthole glass. b. Report of a student slipping, tripping, or falling inside the trainer with or without water present. c. Inclement weather as designated by current directives. |
| Resource Requirements | Damage control Facility |
| Special Instructions |  |
| Procedures | 1. 2. |
| Review: | Critique the PE. After use have the students restore the area around the trainer and police up the area. |
| Feedback Requirements |  |