U. S. ARMY TRANSPORTATION SCHOOL

WATERCRAFT OPERATOR

MARLINESPIKE SEAMANSHIP

POI FILE NUMBER:  421-101-04

ENABLING LEARNING OBJECTIVES	METHOD	TIME

MATERIALS AND CONSTRUCTION OF FIBER LINE	C/VCT	1.0 HOUR

SIZE AND STRENGTH OF FIBER LINE	C	1.0 HOUR

MARLINESPIKE FORMULAS	C	1.0 HOUR

STOWING LINE	C	1.0 HOUR

RISK ASSESSMENT FOR THIS LESSON UNIT:  LOW

LESSON PLAN WRITER:	FORMAT APPROVAL:

NAME:  ROGER C. ALLEN                         	NAME:

TITLE:  INSTRUCTOR/WRITER                 	TITLE:

DATE:                                                             	DATE:

TECHNICAL CONTENT CERTIFICATION

NAME:  CW3 JEFFREY HOCH

TITLE:  CHIEF, NAV/SEA COMMITTEE

DATE:

RISK ASSESSMENT APPROVAL AUTHORITY:  	Not required:

SECTION II - INTRODUCTION

Method of instruction:  C	Instructor to students ratio is:  1 : class

Time of instruction:  4.0 hours	Media:  FM 55-501, VCT, flip chart, line, 				    Outline of Presentation

__________________________________________________________________________

MOTIVATOR:  Motivate students to learn the Terminal Learning Objective

TERMINAL LEARNING OBJECTIVE	Composition, Characteristics, and uses of Fiber and 	Synthetic lines.  Computations and Formulas 	determining the Safety Factor, Safe Working Load, 	and Breaking Strength of Fiber and Synthetic lines.

Note:	Inform the students of the following terminal learning objective requirements.

At the completion of the lesson you (the student) will:

ACTION:  Demonstrate knowledge of materials, construction, strength, formulas and 				stowage of fiber line.

CONDITIONS:  Given classroom instruction, FM 55-501, OP and class notes.

STANDARDS:  The students must answer check-on-learning questions correctly.

SAFETY REQUIREMENTS:  None

RISK ASSESSMENT LEVEL:  Low

ENVIRONMENTAL CONSIDERATIONS:  None

EVALUATION:  You will be evaluated by check-on-learning questions and a hands-on test 					9G6-301-04 , and Marine Technical Examination

INSTRUCTIONAL LEAD IN:  Inform students they will begin by discussing materials 									 and construction of fiber line.

SECTION III - PRESENTATION

NOTE:	Inform the students of the enabling learning objective.

A.	Materials and construction of fiber line.

ACTION:  The student will be able to demonstrate their knowledge of line.

CONDITIONS:  Given classroom instruction, FM 55-501, OP, and class notes.

STANDARD:  To receive a “GO”, student must answer all check-on-learning 							questions.

Method of instruction C/VCT	Instructor to student ratio is:  1 : class.

Time of instruction:  1.0 hour	Media:  FM 55-501, VCT, flip chart and Line

Note:	Show VCT # 55-4737 “Seamanship Fiber Line”(run time 28 minutes).

Note:	Briefly review the film and answer students questions.

Note:	Conduct students interactions throughout the lesson.

1.	Materials.

(a)	Vegetable Fiber; are from natural plant fibers that include.

(1)	Manila.

(a)	A strong fiber made from the leaf stems of the Abaca plant.

(b)	Grows in lengths varying from 4 to 15 feet in its natural state.

(c)	Most of the vegetable fiber line in use by the Army is manila.

(2)	Sisal.

(a)	Made from the Agave plant.

(b)	Sisal fibers are 2 to 4 feet in length.

(c)	Although Sisal is not often used in the Army, it is mentioned here because 				it is a good substitute for manila.

(3)	Hemp.

(a)	Hemp is a tall plant that has useful fiber for making rope and cloth.

(b)	Hemp was used extensively before Manila was introduced.

(c)	The Hemp fibers are tarred to make them resistant to water. The result is 				called Marline.

(d)	Marline is used for lashings and whipping line larger than 3 inches in 					circumference.

(4)	Cotton;  a soft, white, downy fiber attached to the seeds of the cotton plant.

(5)	Flax;  is the same plant that produces linseed oil and is similar to wheat.

(b)	Synthetic Fibers;  are a mineral / petroleum based chemicals that include:

(1)	Nylon;  is the primary synthetic used for line in the Army.

(2)	Dacron;  is a trademark for a polyester textile fiber.

(3)	Polyethylene.

(4)	Polypropylene.

(5)	The following apply to synthetic line:

(a)	Absorbs shock.

(b)	Is waterproof.

(c)	Can stretch and resume its original length.

(d)	Resist abrasion, decay and fungus growth.

2.	Construction.

(a)	Vegetable.

(1)	The stocks, stems, and leaves of the plants are bailed and sent to the factory.

(2)	The factory cleans, combs, and re-combs the plant into fibers

(b)	Synthetic.

(1)	Produced from minerals and processed into filaments.

(2)	Filaments are made as long as the length of line.

3.	Production.

(a)	Both vegetable and synthetic line are formed in similar manners to create line.

(b)	Most line is right laid line.

(c)	Twisting and laying of line:

(1)	Fibers are twisted from left to right to form yarns.

(2)	Yarns are twisted from right to left to form strands.

(3)	Strands are twisted from left to right to form line, the finished product.

(d)	Reversing this process will form left laid line.

Note:	Conduct a check on learning and summarize the learning activity.

1.	Check-On-Learning:

Q.	What are the two types of  material used to make fiber line?

A.	Vegetable and synthetic.

Q.	What plant is manila line made from?

A.	The Abaca plant.

Q.	What are the three parts of line?

A.	Fibers, yarns, and strands.

Q.	Sisal is a good substitute for what type of line?

A.	Manila.

2.	Summerize:  This past hour we disscussed materials and construction of fiber line.

3.	Transition into the next lesson:  Next hour we will discuss size and strength of  fiber line

Note:	Inform the students of the enabling learning objective.

B.	Size and strength of fiber line.

ACTION:  The student will be able to identify size and strength of line.

CONDITIONS:  Given classroom instruction, FM 55-501, OP, and class notes.

STANDARD:  To receive a “GO”, student must answer all check-on-learning 							questions.

Method of instruction:  C	Instructor to student ratio is:  1 : class.

Time of instruction:  1.0 hour	Media:  FM 55-501, flip chart and Line

Note:	Conduct students interactions throughout the lesson.

1.	Size.

(a)	Fiber line is measured by its circumference.

(b)	Line 1¾ inches to 5 inches:

(1)	Is referred to as line.

(2)	Is the most common line used for rigging and mooring.

(3)	Is used for towing small vessels.

(c)	Line larger than 5 inches:

(1)	Is referred to as hawser.

(2)	Used for mooring and towing of larger vessels.

(d)	Line smaller than 1¾ inches:

(1)	Is referred to as small stuff.

(2)	Is used for lashing larger lines together.

(3)	Is used to hoist signal flags and for other general purpose uses.

(e)	Determine the size of small stuff.

(1)	Unlay a strand from the line.

(2)	Count the number of yarns within the strand.

(3)	Multiply that number by 3.

(4)	The result will be the thread size for the line.

Note:	Develop on chalk board the following example.

Example: 				You count		    6     yarns in 1 strand of line.

You multiply by	    3    .

The product is	  18    .

You now know you are working with a size 18 thread count line.

2.	Strength.

(a)	Manila.

(1)	Is the standard against which all other types of line are measured.

(2)	It is implied that the other lines we are comparing have the same 						circumference.

(3)	Manila line has a strength of 100%.

(b)	The strength of the other line are as follows:

TYPE OF LINE	STRENGTH

SISAL	80%

MANILA	100%

3 STRAND NYLON	250%

2-IN-1 BRAIDED NYLON	300%

(c)	Nylon;  is rapidly replacing other types of line due to strength and durability.

(d)	Three Strand Nylon.

(1)	Stretches 30 to 35 percent under an average load that does not exceed its 				Safety Factor for that size line.

(2)	It can stretch up to 40 percent and still return to its original length.

(3)	The line will part at 50 percent.

(e)	Two-N-One Braided Nylon line.

(1)	Stretches 28 percent under an average load.

(2)	It also will return to its original length without damage.

(3)	The line will part at 35 percent.

(f)	Marlinespike Seamanship Computations; the following have no relationship to 				formulas used to make marlinespike seamanship computations:

(1)	Percentage of strength of a line.

(2)	Length of a given line.

Note:	Conduct a check on learning and summarize the learning activity.

1.	Check-On-Learning

Q.	What Is the term used to describe lines over 5 inches in circumference?

A.	Hawser.

Q.	What is the term used to describe a line with a circumference of one inch?

A.	Small stuff.

Q.	What type of line is used as the standard for measuring the strength of line?

A.	Manila.

Q.	What is the strength rating of sisal line?

A.	Eighty percent.

2.	Summerize:  This past hour we disscussed size and strength of fiber line.

3.	Transition into the next lesson: Next hour we will discuss marlinespike formulas.

NOTE:	Inform the students of the enabling learning objective.

C.	Marlinespike formulas.

ACTION:  The student will demonstrate knowledge of marlinespike formulas.

CONDITIONS:  Given classroom instruction, FM 55-501, OP, and class notes.

STANDARD:  To receive a “GO”, student must answer all check-on-learning 							questions.

Method of instruction:  C	Instructor to student ratio is:  1 : class.

Time of instruction:  1.0 hour	Media:  FM 55-501, flip chart and Line

Note:	Conduct students interactions throughout the lesson.

1.	Determine strength of a line or load it can withstand.

(a)	Refer to manufactures specifications first; or

(b)	Refer to table or chart found in most marine handbooks, ie: FM 55-501, pg 12-3; or

(c)	Determine Safe Working Load and Breaking Strength by the “Rules of Thumb”.

(1)	These rules give results that only approximate manufacture’s tables.

(2)	Completely accurate results are questionable, but do err on the side of safety.

(3)	This is due to the constants or figures used in the formulas.

2.	Formulas and Definitions

(a)	Line Strength; is a percentage more or less than Manila which has a strength of 				100%.  We would say that three strand nylon is 300% stronger than manila.

(b)	Constant; is determined by doubling line strength percentage.

Note:	Review chart of constants from FM 55-501,  Pg. 12-3; develop on chalk board.

Sisal	160

Manila	200

Three-strand nylon	500

2-in-1 braided nylon	600

(c)	Safe Working Load (SWL),

(1)	Is the maximum weight or load a line will support safely.

(2)	SWL is computed in pounds.

(3)	Formula is circumference squared times constant equals Safe Working Load.

C²  cnst? = lb. SWL

Note:	Figure SWL for 3 inch fiber line.  Develop examples on chalk board as discussed.

Example:	3 inch sisal	3  3  160 = 9  160 = 1,440 lb. SWL

3 inch manila	3  3  200 = 9  200 = 1,800 lb. SWL

3 inch three strand nylon	3  3  500 = 9  500 = 4,500 lb.. SWL

3 inch two-in-one braided nylon	3  3  600= 9  600 = 5,400 lb. SWL

(d)	Breaking Strength(BS)

(1)	Is a statement of strength in pounds at which a line resist parting.

(2)	Breaking Strength is important to know, it tells us when damage to line,					equipment or personnel may occur.

(3)	Formula is Safety Factor times Safe Working Load equals Breaking Strength

SF  SWL = BS lb.

(4)	Conversely, if you received a shipment of line that only shows the 							manufactures breaking strength, you may determine the SWL by;

Formula is Breaking Strength devided by safety factor equals SWL.

BS  SF? = SWL lb.

(e)	Safety Factor(s); are the relationship or ratio between the BS and the SWL.

(1)	Expressed as numbers or problem elements as determined by test facilities

Note:	Develop on chalk board a list Safety Factors and when they are applied.

factor	USAGE

5	Used for working with new line.

3	Used for working with old line.

8	Used when working with life.

Note:	Refer students to FM 55-501, Pg. 12-3 (Line Strength Table) with SF of 5.

Note:	For the following examples use 3 inch, 2-in-1 braided nylon.  BS = 27,000 lbs.

Note:	Develop examples on chalk board as discussed.

(2)	Safety Factor  of (5)

(a)	Is applied in most marine operations.

(b)	Only applies to new line.

Example:	27,000  5 = 5,400 lb.. SWL

(3)	Safety Factor of (3)

(a)	Only applies to old line.  If your not sure of lines status, assume it is old.

(b)	The SF of (3) is figured in addition to SF of (5).

(c)	Safety factor (s) are not combined.

CORRECT	WRONG

Example:	27,000  5 = 5,400 lb.. and	27,000  8 = 3,375

5,400  3 = 1,800 lb.. SWL

(4)	Safety Factor of (8)

(a)	Applies where life is involved, such as lifeboat falls or bosun’ chair.

(b)	The SF of (8) is figured in addition to SF of (5).

(c)	Safety factor (s) are not combined.

CORRECT	WRONG

Example:	27,000  5 = 5,400 lb.. and	27,000  13 = 2,077

5,400  8 = 675 lb.. SWL

Note:	Develop on the chalk board, solve each problem  with class participation.

Example:		Determine SWL for 4 inch manila. C²  cnst? = lb.. SWL

4  4  200 = 3,200 lb. SWL

Example:		Determine BS of 2 inch three strand nylon. SF  SWL = BS

5  2,000 = 10,000 lb. BS

Example:		Determine SWL of old 5 inch two-in-one nylon. BS  SF? = SWL

75,000  5 = 15,000 lb., & 15,000  3 = 5,000 lb. SWL

Example:		Determine SWL for 1.5 inch sisal. C²  cnst? = lb.. SWL

1.5  1.5  160 = 360 lb. SWL

Example:		Determine BS of 8 inch three strand nylon. SF  SWL = BS

5  32,000 = 160,000 lb. BS

Example:		Determine SWL of life bearing 1.5 inch 2-in-1 braided nylon.

BS  SF? = SWL

6,750  5 = 1,350 lb. & 1,350  8 = 168.75 lb. SWL Note:

Note:	Conduct a check on learning and summarize the learning activity.

1.	Check-On-Learning:

Q.	What is the standard safety factor used for new line?

A.	Five.

Q.	What must you do to the circumference of the line when computing SWL?

A.	Square it.

Q.	What is the formula for determining BS?

A.	SF  SWL = BS

Q.	What Safety Factor is applied when determining SWL where life is involved?

A.	Eight.

2.	Summerize:  This past hour we disscussed marlinespike formulas.

3.	Transition into the next lesson:  Next hour we will discuss stowage of line.

NOTE:	Inform the students of the enabling learning objective.

D.	Stowing Line.

ACTION:  The student will be able to demonstrate knowledge of stowing line.

CONDITIONS:  Given classroom instruction, FM 55-501, OP, and class notes.

STANDARD:  To receive a “GO”, student must answer all check-on-learning 							questions.

Method of instruction:  C	Instructor to student ratio is:  1 : class.

Time of instruction:  1.0 hour	Media:  FM 55-501, flip chart and Line

Note:	Conduct students interactions throughout the lesson.

1.	Stowing Line.

(a)	Never stow line wet.

(b)	Stow in a well ventilated locker.

(c)	Never stow in contact with metal surfaces.

(d)	Stow either coiled, flemished or faked.

2.	Handling Procedures.

(a)	New Coil.

(1)	Remove binding and cover just prior to use.  Cover is usually of burlap.

(2)	Ensure the end of the line is in the center bottom of the coil.

(b)	Coiling / Uncoiling.

(1)	Coiling means to lay line down in circles roughly one on top of the other.

(2)	Always coil down right laid line in a clockwise manner.

(3)	Line will uncoil in a counter-clockwise direction.

(4)	If the bottom end must be run out first, then the coil must be turned over.

(c)	Flemishing.

(1)	Means to lay line in successively larger circles.

(2)	The bitter end is placed in the center of the circle to begin.

(3)	Always lay right laid line clockwise and left laid line counter-clockwise.

(4)	Flemishing is most practical with shorter lengths of line or

exposed ends of line being used around the deck.  It is aesthetically pleasing.

(d)	Faking.

(1)	Similar to coiling, except the line is laid out in long flat bights.

(2)	Faking a line allows the line to run out easily.

(e)	Inspection.

(1)	Load or strain causes friction and heat in a line.

(2)	Exposure to moisture and elements causes mildew and dry rot, especially in 				vegetable line.

(3)	Untwist and open a strand to inspect inside the line.

(4)	You should destroy line that is unserviceable by cutting it into short pieces.

(5)	Inspect line before you use it and on a regular basis if not being used.

(f)	Cleaning.  Clean with fresh water and ensure it is dried thoroughly.

(g)	Do’s & Don’ts.

(1)	Do stow line in dry unheated locker.

(2)	Do coil, fake or flemish line.

(3)	Do stow line, especially hawser on wood grate or wood pegs.

(4)	Don’t stow line in contact with metal surfaces.

Note:	Conduct a check on learning and summarize the learning activity.

1.	Check-On-Learning:

Q.	When should you inspect line?

A.	Before use.

Q.	How do you should open a new coil, from the inside or the outside?

A.	Inside.

Q.	What is the most common defect found in manila?

A.	Rot.

Q.	How should line be stowed, on draped over lifelines dry or wood hangers?

A.	On wood hangers, pegs or grates.

2.	Summerize:  This past hour we disscussed Stowage of line.

SECTION IV - SUMMARY

1.	Review/Summarize:  During the last four hours we have disscussed:

Materials and Construction of Fiber Line

(a)	Materials

(1)	Vegetable

(a)	Manila

(b)	Sisal

(c)	Hemp

(d)	Cotton

(e)	Flax

(2)	Synthetic

(a)	Nylon

(b)	Dacron

(c)	Polyethylene

(d)	Polypropylene

(b)	Construction

(1)	Vegetable

(2)	Synthetic

(c)	Production

Size and Strength of Fiber Line

(a)	Size

(1)	Measure by circumference.

(2)	Line; 1¾ inches to 5 inches.

(3)	Hawser; 5 inches and larger.

(4)	Small stuff; 1¾ inches and smaller.

(5)	Determining size of small stuff.

(b)	Strength

(1)	Manila, the standard - 100%.

(2)	Other line;	Sisal		80%

3 strand		250%

2 n 1 braid	300%

(3)	Nylon strength and durability.

(4)	3 strand specifics.

(5)	2-n-1 braid specifics.

(6)	Computations, relationship of strength to formulas.

Marlinespike Formulas

(a)	Determining line strength

(1)	Manufacture

(2)	Tables & Charts

(3)	Determining Safe Working Load by rules of thumb.

(a)	Results are approximate.

(b)	Results err on side of safety.

(c)	Results are because of constants.

(b)	Formulas & Definitions.

(1)	 Line strength review.

(2)	Determining constants, double or multiply line strength by two.

(3)	Determine Safe Working Load (SWL);  C²  cnst? = lb. SWL

(4)	Determine Breaking Strength (BS);  SF  SWL = BS

(5)	Determine SWL by BS; BS  SF? = SWL

(6)	Safety Factor(s) are (5 for New line), (3 for Old line) and (8 where life is involved).

Marlinespike Formulas

(a)	Stowing Line

(1)	Not wet

(2)	Ventilated

(3)	Not on Metal

(4)	Coiled, faked or flemished

(b)	Handling

(1)	New coil

(2)	Coiling & Uncoiling

(3)	Flemishing

(4)	Faking

(5)	Inspection

(6)	Cleaning

(7)	Do’s & Don’ts

2.	Check on learning,

a.	Solicit student questions and explanations.

b.	Questions and answers.

c.	Correct student misunderstandings.

3.	Transition into the next lesson.