TRAINING SUPPORT PACKAGE (TSP)

Note: This is the TSP title page

| TSP Number | 88K13A04 |
| --- | --- |
| Task Number(s) Title(s) | 551-735-1507 LAUNCH LIFE RAFT |
| Effective Date | 1 OT 97 |
| Supersedes TSP(s) | PFN 421-138-10 STP- 55-88K 1-4-SM-TG DEC 91 |
| TSP User | 88K WATERCRAFT OPERATOR |
| Proponent | DEPARTMENT OF THE ARMY TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street, Suite 227 Fort Lee, VA 23801-1511 |
| Comments/ Recommen- dations | Send comments and recommendations directly to: DEPARTMENT OF THE ARMY TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street, Suite 227 Fort Lee, VA 23801-1511 |
| Foreign Disclosure Restrictions | This product has been reviewed by the product developers in coordination with the Commander, U.S. Army Combined Arms Support Command foreign disclosure authority. This product is releasable to military students from foreign countries on a case-by-case basis |

PREFACE

Purpose	This training support package provides the instructor with a

standardized lesson plan for presenting instruction for:

TASK NUMBER: 551-735-1507

TASK TITLE: LUNCH A LIFE RAFT

CONDITIONS: Given classroom instruction, FM 55-501 and use of class notes.

STANDARD: The student must answer all check on learning questions correctly.

This TSP

contains

| TABLE OF CONTENTS |  |  |
| --- | --- | --- |
|  |  | Page |
| Preface |  | 2 |
| Lesson | Section I - Administrative Data | 3 |
| Plan | Section II - Introduction | 6 |
|  | Terminal Learning Objective | 6 |
|  | Section III - Presentation | 7 |
|  | A. Enabling Learning Objective 1 |  |
|  | B. Enabling Learning Objective 2 | 11 |
|  | C. (etc.) | 20 |
|  | Section IV - Summary | 21 |
|  | Section V - Student Evaluation |  |
| Appendixes | A. Viewgraph Masters |  |
|  | B. Test and Test Solutions |  |
|  | C. Practical Exercises and Solutions | 22 |
|  | D. Student Handouts |  |

Launch a Life Raft

[Effective Date]

SECTION - I. ADMINISTRATIVE DATA

| All Courses Including This Lesson | COURSE NUMBER(S) |  |  | COURSE TITLE(S) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Task(s) Taught or Supported | TASK NUMBER |  |  | TASK TITLE |  |  |
| Reinforced Task(s) | TASK NUMBER |  |  | TASK TITLE |  |  |
| Academic Hours: | The Academic hours required to teach this course are as follows: |  |  |  |  |  |
| * TOTAL HOURS | PEACETIME HOURS/METHOD 4/C 1/D 4/PE1 1/TV /0000000000 10.0HRS |  |  | MOBILIZATION HOURS/METHOD / / / / /000000000000000 |  |  |
| Test Lesson [List the lesson number in which the terminal learning objective of Number this lesson are tested and the test results are reviewed with the students.] |  |  |  |  |  |  |
|  |  |  |  |  | Hours | Lesson No. |
|  |  |  | Testing: |  |  |  |
|  |  |  | Review of test results: |  | _______________ | _______________ |
| Prerequisite Lesson(s) |  | (State if there are none.) LESSON NUMBER |  |  | LESSON TITLE |  |
| Clearance and Access |  | UNCLASSIFIED (U) |  |  |  |  |

| References |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Number | Title | Date | Para No. | Additional Information |
|  | FM 55-501-2 | Marine Crewman’s Handbook |  | Chap. 10 |  |

| Student Study Assignments | See instructor and student packet chapter 10 |  |  |  |
| --- | --- | --- | --- | --- |
| Instructor Requirements | [These are special qualifications or additional requirements.] |  |  |  |
| Additional Support Personnel Requirements | Life Guard |  |  |  |
| Equipment Required | For the total lesson the following equipment are required: emersion suit, life raft TV, VCR, |  |  |  |
| Materials Required | Pen, pencil and paper |  |  |  |
| Classroom, Training Area, and Range Requirements | Classroom Swimming Pool |  |  |  |
| Ammunition Requirements | None |  |  |  |
| Instructional Guidance | Note: Ensure facilities, equipment, and training materials are ready prior to class start time. Instructors will have a thorough knowledge of reference Prepare by studying this lesson and identified reference material. |  |  |  |
| Proponent Lesson Plan Approvals | NAME ___________________ ___________________ | Rank _____ _____ | Position __________________ __________________ | Date _______ _______ |

SECTION II - INTRODUCTION

|  | Method of instruction: C Instructor to student ratio is: 1:C Time of instruction: .2hours. |
| --- | --- |
| Motivator | As a crewmember aboard Army Watercraft, your very survival at sea may depend on your knowledge of how to launch your vessel’s life raft and use the equipment provided in the raft. |
| Show VG 1-21 Terminal Learning Objective | Note: Inform the students of the following terminal learning objective requirements. At the completion of this lesson you [the student] will: Procedures for launching, recovering, boarding and righting an overturned life raft; and wearing the cold water immersion suit. |

| ACTION: | Discuss the life raft and all survival equipment. |
| --- | --- |
| CONDITIONS: | Given classroom instruction, FM 55-501 and use of class notes. |
| STANDARD: | The student must answer check on learning questions correctly and pass the practical exercise, |

| Safety Requirements | There are not safety considerations for this lesson unit. |
| --- | --- |
| Risk Assessment Level | Medium |
| Environmental Considerations | None. |
| Evaluation | Check on Learning and Practical Exercise. |

SECTION III - PRESENTATION

| ACTION: | Overview of life rafts. |
| --- | --- |
| CONDITIONS: | Given classroom instruction, use of class notes and FM 55-501. |
| STANDARD: | The student must answer all check on learning questions correctly. |

| 1. | Learning Step/Activity 1 [A statement describing what is to be done.] |
| --- | --- |
|  | Method of instruction: C Instructor to student ratio is 1:C. Time of instruction: .8 hr Media ______(State if there is none.)_____. |
|  | LAUNCH A LIFE RAFT. |
|  | A. Life Rafts. 1. Identify life rafts. |
| NOTE: | Show VCT “The Sea Can Kill”, 27 minutes). |
|  | a. Inflation. [1] Carbon dioxide is usually used to inflate the raft. The CO2 cylinder is on the bottom of the raft. The 100 feet operating cord activates it. A raft will inflate in 30 seconds. [2] Pressure relief valves are installed in most rafts. This allows excess gas to escape right after the inflation of the raft. The rise in temperature may cause the gas to expand and activate the valves. At night, when the temperature drops, you may have to pump up the tubes because the air in the tubes might contract. [3] The floor of the raft is also inflatable. In cold climates the floor should be inflated to keep you warmer and in hot climates the floor should be deflated to keep you cooler. [4] Buoyancy tubes are located on the outer edge of the raft. They are divided into at least two compartments so that if one tube is punctured, the other will remain partially inflated. b. Sizes and types. [1] Life rafts can hold from 4 to 25 people. The amount of space inside the raft is 4 square feet per person. The capacity is marked on the container and the raft itself. An inflatable raft complete with case and equipment weighs no more than 400 pounds. [2] There are two types of rafts. Oceangoing also known as unlimited and Inland also known as limited. The oceangoing has extra equipment for longer periods of survival at sea. The inland has less equipment for shorter periods of survival at sea. c. Shapes. Life rafts come in various shapes. There are 5 total. Square, round, octagonal, hexagonal, and oval. No shape has been proven better than the other. d. Stowage. [1] Life rafts are kept in a cradle on an open deck. This is done so they can float free if the vessel sinks before you can launch it manually. [2] The life raft container is weathertight and tamperproof. It has small holes on the bottom for condensation drainage and air circulation. [3] The container must be stowed with the word “THIS SIDE UP” on top to be sure the holes are on the bottom. Most containers are made of fiberglass. [4] The raft container is usually held together with packing bands, which break when the raft inflates. A watertight gasket seals the two halves of the container together. e. Features. [1] The canopy of life rafts are international orange in color and will stand out on whitecapped seas. They are also equipped with retro-reflective tape and some even have radar reflectors. The canopy has two layers to insulate the inside from extreme temperatures. It pops up automatically as the arch tubes inflate. The canopy also has tubes to collect rainwater for drinking. [2] A boarding ladder is fitted at one end of the raft. Lifelines are provided inside and outside the raft for survivors to steady themselves. [3] Two lights are installed on the canopy. These lights are automatically activated when the raft inflates. They are powered by either dry cell or water activated batteries. The lights can operate at least 12 hours. The external light can be seen from 2 miles away. The other light is inside the canopy. [4] Water pockets are located under the floor of the raft. They have holes in them to seawater to fill them up when the raft has been launched. Water pockets have two purposes: to slow in drifting and keep it from capsizing. |
| Note: | Conduct a check on learning and summarize the learning activity. |

| NOTE: | Ask students check on learning questions. Q. How long does it take to fully inflate a life raft? A. 30 seconds. Q. Where are life rafts stowed? A. In a cradle on an open deck. Q. How long will the battery for the lights last? A. At least 12 hours. Q. What two places would you find the capacity of a life raft? A. On the outside of the container and on the raft itself. Q. What are two types of life rafts? A. The limited (inland) and the unlimited (ocean going). |
| --- | --- |

|  | ENABLING LEARNING OBJECTIVE B |
| --- | --- |

| ACTION: | Launch a life raft and properly wear an immersion suit. |
| --- | --- |
| CONDITIONS: | Given classroom instruction, use of class notes and FM 55-501. |
| STANDARD: | The student must answer all check on learning questions correctly. |

| 1. | Learning Step/Activity 1 |
| --- | --- |
|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: .9 hours. Media ______(State if there is none.)_____. |
|  | B. LAUNCHING PROCEDURES AND IMMERSION SUIT. |
|  | 1. Identify to Launch a Life Raft. |
| NOTE: | Have students turn to page 10-17 in FM 55-501. |
|  | a. Automatic operation. [1] When the vessel sinks to a depth of 10 to 15 feet, the hydrostatic release will automatically let go and free the lift raft container. The container will rise to the surface. [2] The pull of the sinking vessel will cause the container bands to part and trigger the inflation of the raft. The raft will be completely inflated and ready for boarding within 30 seconds. The buoyancy of the raft will cause the operating cord to part. b. Manual operation. [1] If the raft is to be moved to another location, remove the weak link ensuring the operation cord does not pay out until the proper time and location. Pull open the hook at the hydrostatic release to undo the tiedown straps. [2] With two or more crewmembers lift the life raft out of the cradle and move it to another launch location if necessary. [3] Secure the operating cord to the vessel using a cleat, making sure it’s not going to tangle while paying out. [4] With the life raft and container in the water, yank on the operating core. The bands on the container will break and the life raft will automatically inflate. [5] Leave the operating cord attached to the cleat. Board the raft as soon as possible. Remove the knife from the pocket on the canopy and cut the operating cord to free the raft from the sinking vessel. [6] If moving the raft is not necessary, follow all steps and secure the operating cord to the cleat provided next to the life raft cradle. c. Righting an overturned life raft. [1] Swim to the side marked “RIGHT HERE”. If no mark is present, go to the side where the CO2 cylinder is. Reach up and grab the righting strap. Start pulling yourself up on to the raft. [2] Stand on the very edge where the CO2 cylinder is located. Lean back with all your weight while pulling on the righting strap, located on the center underside of the raft, with your hands. The raft will follow you. [3] If the raft is large, it will land on your head unless you bend your knees and spring backwards just as the raft begins to free fall. This should allow your head to clear the raft. [4] If the raft does land on top of you do not panic. The bottom of the raft is flexible. There will be an air pocket caused by your head pushing against the floor. Use your arms while face up, pull yourself out from underneath the raft. [5] If one person cannot right the raft try two person pulling on the righting strap while others push up on the opposite end of the raft. d. Capsizing. [1] With a sea anchor trailing and the water pockets filled with water, the life raft is very difficult to overturn. In case this happens try to save as much equipment as possible. [2] Clear the raft of all occupants and right the raft. e. Boarding a life raft. [1] climbing down a ladder and stepping directly into the raft. This method is the safest and driest therefore the best. [2] Jumping directly from the vessel into the canopy opening making sure not to jump directly on the canopy. You may injure someone already on board the raft. [3] Entering from the water. |
| NOTE: | Show VCT “All Hands Abandon Ship”, (22 minutes). |
| NOTE: | Conduct check on learning questions and summarize the learning activity. Ask students check on learning question Q. What is the hydrostatic release for A. At a depth of 10 to 15 feet it will automatically release and free the life raft container. Q. How much time does it take a life raft to fully inflate? A. Within 30 seconds. Q What is the first step in manually launching a life raft? A Pull open the hook at the hydrostatic release to undo the tiedown straps . |

|  | Learning Step/Activity 2 [A statement describing what is to be done.] |
| --- | --- |
|  | Method of instruction: C Instructor to student ratio is: 1:C Time of instruction: .9 hours. Media FM 55-501 Immersion suit |
|  | 2. Identify the Immersion Suit. a. The make-up of the suit. [1] The immersion suit is made up of a closed cell foam. This gives it inherent buoyancy. It’s designed as a one piece coverall design, fitted with integral gloves, boots, and hood. The suit is international orange with retro-reflective tape. [2] The material can withstand 300 pounds crosswise pull for 30 minutes. It will not tear or come apart at the seams when jumping from a height of 15 feet. Also, oil and sea water will not harm the suit. It can withstand direct exposure to fire for a least 2 seconds. [3] Some added features can be a helicopter lifting strap, several types of lights, whistles, mini EPIRB and buddy and leg take-up straps. b. Mobility of the suit. [1] Although it looks bulky, the suit must allow the wearer to right themselves within 5 seconds in the water. It must not hinder the wearer more than 1.25 times normal walking or climbing ability. [2] The suit must be donned in 1 minute. c. Capabilities of the suit. [1] The minimum U.S. Coast Guard and IMO (International Maritime Organization) specifications required that a temperature drop of no more than 2C be experienced over a six hour period in 35F water. [2] The suit, when worn over normal clothing, easily surpassed their specifications. In some instances, extrapolation of temperature loss indicates the suit could provide survival for an indefinite period of time. [3] An individual can survive for 18 hours in the North Sea. |
| Note: | Conduct a check on learning and summarize the learning activity. |

|  | ENABLING LEARNING OBJECTIVE C |
| --- | --- |

| ACTION: | Survive aboard a life raft. |
| --- | --- |
| CONDITIONS: | Given classroom instruction, use of class notes and FM 55-501. |
| STANDARD: | The student must answer all check on learning questions correctly. |

| 1. | Learning Step/Activity 1 |
| --- | --- |
|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: .9 hour. Media ______(State if there is none.)_____. |
|  | C. SURVIVAL EQUIPMENT AND SURVIVAL ABOARD LIFE RAFTS. |
|  | 1. Identify Survival Equipment. |
|  | a. Heaving line. [1] A buoyant line, 100 feet long, that has a small buoyant ring at one end is thrown to people outside the raft to pull them in. [2] Also used to pull two rafts together. b. Instruction/survival manual. [1] The manual describes how to use the raft’s equipment. It also contains internationally recognized distress signals and survival information. [2] It also provides the occupants with some reading material and the possibility of foreign language training. c. Jackknife. [1] One knife is provided on rafts holding up to 12 persons. Two are required on larger rafts. [2] The knife has a can opener on it. Depending on the size of the raft, one of the knives will be located in a pocket near the forward entrance. d. Inflation/dewatering pump. [1] On the water the temperature will cool down at nigh. This will cause the air inside the buoyancy tube to condense, causing the tube to appear partly deflated. The inflation pump is used to top off the tubes, as well as inflate the floor. [2] In warmer weather the gas inside the tubes will expand and you will have to release the excess pressure. e. Sea anchors. [1] Two sea anchors are provided. One sea anchor attaches to the outside of the raft and streams automatically when the raft is inflated. The other one acts as a spare. [2] It trails behind the raft and keeps the raft into or out of the waves. f. Signaling equipment. [1] Signaling mirrors are the best form of signaling during the day. They should be used at all times during the day, even when clouds are present or when you cannot see or hear rescue craft. [2] The flash from a signal mirror can be seen as far as 12 miles away. Different types of flares may also be provided. g. EPIRB (Emergency Position Indicating Radio Beacon). [1] Your vessel may have at least one EPIRB. They are usually stowed on the outside of the vessel, so it will float free if the vessel sinks. [2] Most EPIRBs work on one-way automatic operation only and cannot be used for two-way communication. They transmit a continuous two-tone (hi/low) signal. [3] If your EPIRB is floated, tie it to the survival craft so it won’t drift away. h. Seasickness pills and urination. [1] If you have seasickness pills aboard ship, take one before you abandon ship if you can. If you can’t, take one provided in the supply kit on the raft as soon as everyone has boarded. The pills will keep you from vomiting. Vomiting empties your stomach of valuable fluids. You must preserve those body fluids. If you lose them, they will be difficult or even impossible to replace as long as you’re in the raft. [2] Urinate soon after boarding. If you do not urinate within a few hours before boarding the survival craft, you should do so within 2 hours. You could damage your bladder if you do not pass urine. i. Drinking water. [1] The normal, healthy body can stay alive for over 40 days with as little as 11 ounces of fresh water each day and no food. Without fresh water a person often becomes delirious in about 4 days and might die in 8 to 12 days. [2] Do not issue water for the first 24 hours unless you have an unlimited supply. [3] Some parts of the life raft are designed to catch rainwater. Rainwater catchment tubes will take the water into storage bags on the inside of the raft. Salt spray may dry on the canopy. The salt might be washed in with the first few ounces of rainwater. [4] Water might condense on the inside of the canopy. Use one of the cellulose sponges to soak up the water. Squeeze the water out and drink or store it. j. Obtaining food. [1] Do not eat for the first 24 hours, you may eat 4 ounces each day. If you have no water, do not eat food. [2] A fishing kit is provided to catch fish. Most fish that are found in the open sea can be eaten. If they are caught closer to shore they may be poisonous k. Repair kit. [1] Used to repair the buoyancy tubes is provided. The kit contains a roughing tool, five rubber patches and cement. [2] Do not completely inflate the raft until the patch has had 24 hours to dry. |
| NOTE: | Conduct check on learning questions and summarize the learning activity. |
| NOTE: | Ask students check on learning questions. Q. Should a person eat without having water? A. No. Q. When should you take a anti-seasickness pill? A. As soon as possible or right after you board the life raft. Q. How long must you wait before you are allowed to drink water? A. 24 hours. Q. When are signaling mirrors used? A. At all times during the day. Q. What does EPIRB stand for? A. Emergency Position Indicating Radio Beacon. |
| Note: | Conduct a check on learning and summarize the learning activity. |
| 2 | Learning Step/Activity 2 |
|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: .9 hour Media ______(State if there is none.)_____. |
|  | 2. Identify the Seven Steps to Survival. |
|  | a. Recognition - Admitting your life is in danger. Realize that the next few minutes will be the most important minutes of your life. b. Inventory - For every new situation an inventory must be done. Separate the equipment that will not be used, and if water will not harm it, trail it behind the raft on line. This will allow you more room inside. c. Shelter - Use everything around you that will protect your body against harmful elements. Clothes will keep you warm. Your PFD will keep you afloat. An Immersion Suit will keep you afloat and warm. Blankets are good for warmth as well as protection against the sun. d. Signals - Anything that will make you bigger, brighter, or different. An EPIRB or Marine Radio will make you bigger by the distance the signal will travel. Radar reflectors, flares and smoke will make your raft brighter. Beaching on some unknown island with the top side of the raft showing and getting three or more fires burning will make you look different. e. Water - Drink at least 1 pint per day. Drink 6 pints a day if you can. Most rafts have built in rain catchers and must be washed off with fresh water before they are used. f. Food - Food is not necessary for survival, however it’s great for morale. If you do not have water to drink do not eat. g. Play - Stories, jokes and mental games are preferred due to the reduced calorie and water intake. Keep a positive attitude. The Coast Guard estimates the average length of stay aboard a life raft is 72 hours. Remember some of the worst enemies on the water are boredom, fear and the unknown length of stay. |
| Note: | Conduct a check on learning and summarize the learning activity. |
|  | Q. What is the first step in survival aboard a life raft? A. Recognition. Q. When do you conduct an inventory? A. After every situation. Q. What are signals used for? A. They make you bigger, brighter and different. Q. What is the first step in catch rain water? A. The rain catcher must be cleaned. |

SECTION IV - SUMMARY

|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: .2 hour |
| --- | --- |
| Review/ Summarize Lesson |  |
| Check on Learning | Determine if students have learned the material presented by -- |
|  | a. Soliciting questions and explanations. |
|  | b. Asking questions and getting answers from the students. |
|  | c. Correcting student misunderstandings. |
| Transition To Next Lesson | [If applicable] |

SECTION V - STUDENT EVALUATION

| Testing Require- ments | Describe how the student must demonstrate accomplishment of the TLO. This POI file number is examinable in IBM 9G6-103-02 and IBM Examination 9G6-601-16. |
| --- | --- |
| Note: | Refer student to the Student Evaluation Plan |
| Feedback Requirement | a. Schedule and provide feedback on the evaluation and any information to help answer students’ questions about the test. |
|  | b. Provide remedial training as needed. |
| Note: | Rapid, immediate feedback is essential to effective learning. |

PRACTICAL EXERCISE SHEET NO.

| Title | Right a Life Raft/Board and Don an Immersion Suit. |  |
| --- | --- | --- |
| Intro- duction |  |  |
| Motivator | Using personnel experience or a related story, give a brief explanation why this lesson unit is important to the student. |  |
| Terminal Learning Objective | Note: Inform the students of the following terminal learning objective requirements. At the completion of this lesson you [the student] will: |  |
|  | ACTION: | Demonstrate proficiency at performing the procedures and techniques on how to right a life raft and don an immersion suit. |
|  | CONDITIONS: | Given a demonstration, an indoor pool, life raft, and an immersion suit. |
|  | STANDARD: | The student must right a life raft correctly and properly don an immersion suit. |

Method of instruction: C

Instructor to student ratio is 1:C

Time of instruction: 5.0 hours

| Safety Require- ments | There are special safety requirements for this lesson unit. See the “SAFETY ANNEX” enclosed at the back of this lesson unit. |
| --- | --- |
| Risk Assessment Level | Medium. |
| Environmental Considerations | None. |
| Evaluation | This lesson unit examinable on IBM Examination and IBM Examination (MTE). |
| Instructional Lead-in |  |
| Resource Require- ments | This practical exercise requires an indoor pool, life raft, immersion suits (6 per class of 24) and a life guard other than the instructors presenting the class. Students must have swim suits and towel. |
| Special Instructions | Particular attention must be paid to SAFETY considerations and the “Rules of the Pool” for the location in use. |
| Procedures | Complete the Practical Exercise. |
| Feedback Require- ments | Schedule and provide feedback on the practical exercise and any information to help answer student questions about the test. Provide remedial training as needed. |

U.S. ARMY TRANSPORTATION SCHOOL

WATERCRAFT OPERATOR COURSE

PRACTICAL EXERCISE:  Section I

SAFETY

The purpose of this Practical Exercise is to practice the RIGHTING A LIFE RAFT/DONNING AN IMMERSION SUIT skills learned in the classroom. Instructors must exercise extreme caution at all times while dealing with students in the water. Some of the students may be non-swimmers or weak swimmers. Regardless of the perceived ability of the class, the safety procedures stated in this exercise will be strictly adhered to at all times.

GENERAL CLASS GUIDELINES.

1.  All training sessions in the pool will be conducted by a minimum of two instructors.  Instructors will not act as lifeguards.  The Anderson Field House Indoor Pool must furnish the required number of lifeguards as prescribed in their SOP.  At least one of the instructors will be certified by the RED CROSS as a lifeguard.

2.  Your most important responsibility is to the safety of the students.  Your second responsibility is the rescue and emergency care of an accident victim.  No other duty or responsibility will be allowed to interfere with these two functions.

3.  Prior to any instruction in the pool, the instructor in charge will brief all assistant instructors on their responsibilities and the provisions of this safety annex.  A complete understanding of all procedures, safety guidelines and emergency procedures will be had by all before the class commences.  An emergency plan will be developed and fully understood by all instructors which will cover the procedures to follow in the event of an accident.  The following points will be considered as a minimum;

a.  The emergency signal.

b.  Rescue procedures and the emergency care of any victims.

c.  Procedures to follow to notify the authorities for assistance.  The telephone numbers for dispatch of an ambulance will be provided to each instructor.

d.  The location of all emergency equipment will be discussed.  The use of safety and rescue equipment will be discussed as the primary means for aiding a swimmer in distress.

e.  Supervision and control procedures to be followed during class.

f.  The importance of maintaining vigilance at all times.  This means not only watching for trouble, but looking for signs of extreme anxiousness nor any sign that may indicate possible trouble for a student.  Any person who is judged to be demonstrating abnormal behavior will be identified to the instructor in charge immediately.

4.  A safety briefing will be conducted by the instructor in charge for every class BEFORE EVERY POOL SESSION.  As a minimum, the following provisions will be covered;

a.  The “BUDDY” system will be employed during all pool sessions.  Each person in the class will be paired off with a “buddy” who is responsible for remaining with his or her “buddy” at all times.  Instructors will emphasize the importance of remaining with the “buddy” at all times in or out of the water.

b.  No one will be allowed in the pool unsupervised.

c.  The pool layout will be briefed to include the location of the nearest telephone and the depths of water around the pool.

d.  Non-swimmers will be identified by wearing a T-shirt in the water and around the pool at all times.

e.  Discuss running, pushing and bolsterous play which is not allowed under any circumstances.

f.  Brief any special rule established by the pool facility whether they are posted or not.  Included in these are;

[1] Running in the pool area.

[2] Horseplay.

[3] No diving from shallow end.

[4] No swimming during break.

[5] No entering the pool unless directed.

[6] Use of the ladder when entering/exiting the pool.

[7] Spitting, nose blowing, urinating in the pool.

[8] No chewing gum in the pool.

[9] One person on the diving board at a time.

[10] No rescue by students.

[11] Notify instructor of any problems.

[12] Shower before entering pool.

[13] Students will not wear;

[a] Cut off shorts.

[b] Shoes.

[c] Goggles.

[d] Ear plugs.

[e] Nose plugs.

[f] Watches, rings, dog tags, or jewelry of any kind.

[g] Partial or complete dentures

[h] Eye glasses or contact lenses.

g.  Brief the intent of the class.  This may well be the most important point you will cover.  The class must understand the importance of the techniques that will be learned without being pressured into any act that they do not want to do.  There is to be NO harassment, pressure or intimidation to perform any skill by any individual.  If an individual expresses a desire NOT to perform a task, he or she will be counseled on the importance of the task relative to class standing.  After counseling, if the student maintains the same position no further actions will be taken in an effort to “make” them participate.  NO ONE WILL BE FORCED TO DO ANYTHING.

h.  The grading system to be employed will be fully briefed so that all students understand the consequences of failure to perform a task and that it is possible for certain tasks to be omitted without failing this practical exercise or the Performance Test 9G6-103-02.

DEMONSTRATION

NOTE:  The following tasks will be demonstrated/performed by instructor.

1.  Donning of the immersion suit.

2.  Enter the water wearing an immersion suit.

3.  Take off an immersion suit while in the water and lay on top.

4.  Put immersion suit back on while in the water.

5.  Righting an overturned life raft.

6.  Boarding a life raft.

END OF DEMONSTRATION

HANDS-ON PRACTICAL EXERCISE

NOTE:  Divide the class into groups of six.  Only six students in the pool at a time.

NOTE:  Give the class breaks at points YOU feel prudent.

NOTE:  Begin the Practical Exercise.

NOTE:  Use one group at a time for each event.

1.  Don an Immersion Suit.

2.  Enter the water while wearing the immersion suit.

3.  Take off immersion suit while in the water and lay on top.

4.  Put immersion suit back on while in the water.

5.  Right on overturned life raft.

6.  Board a life raft.

7.  Assist an injured or unconscious victim aboard the life raft.

NOTE:  Continue exercise until all students have practiced all tasks.

NOTE:  Secure from Practical Exercise.

NOTE:  Critique the Practical Exercise.

NOTE:  As this is a subjective Practical Exercise, there is no written Solution Sheet for Section I.