TRAINING SUPPORT PACKAGE (TSP)

Note: This is the TSP title page

| TSP Number | 88K12B07 |
| --- | --- |
| Task Number(s) Title(s) | Introduction To Navigation (Piloting) |
| Effective Date | 1 October 1997 |
| Supersedes TSP(s) | PFN 421-155-06 22 FEBUARY 1995 |
| TSP User | MOS 88K WATERCRAFT OPERATOR |
| Proponent | DEPARTMENT OF THE ARMY TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street Suite 227 Ft. Lee, VA 23801-1511 |
| Comments/ Recommen- dations | Send comments and recommendations directly to: DEPARTMENT OF THE ARMY TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street Suite 227 Ft. Lee, VA 23801-1511 |
| Foreign Disclosure Restrictions | This product has been reviewed by the product developers in coordination with the Commander, U.S. Army Combined Arms Support Command foreign disclosure authority. This product is releasable to military students from foreign countries on a case-by-case basis. |

PREFACE

Purpose	This training support package provides the instructor with a

standardized lesson plan for presenting instruction for:

TASK NUMBER: Not found in STP 55-88K 14 –SM-TG

TASK TITLE: Not found in STP 55-88K 14 –SM -TG

CONDITIONS: Given class notes, the evaluation plan and a five question quiz.

STANDARD: To receive a “GO” for this Lesson Unit the student will answer five questions correctly.

This TSP

contains

| TABLE OF CONTENTS |  |  |
| --- | --- | --- |
|  |  | Page |
| Preface |  | 2 |
| Lesson | Section I - Administrative Data | 3 |
| Plan | Section II - Introduction | 6 |
|  | Terminal Learning Objective | 6 |
|  | Section III - Presentation |  |
|  | A. Enabling Learning Objective 1 | 7 |
|  | B. Enabling Learning Objective 2 |  |
|  | C. (etc.) |  |
|  | Section IV - Summary | 12 |
|  | Section V - Student Evaluation | 13 |
| Appendixes | A. Viewgraph Masters |  |
|  | B. Test and Test Solutions |  |
|  | C. Practical Exercises and Solutions | 14 |
|  | D. Student Handouts |  |

Watercraft Operator

[Effective Date]

SECTION I - ADMINISTRATIVE DATA

| All Courses Including This Lesson | COURSE NUMBER(S) 88K10 |  |  |  | COURSE TITLE(S Water Craft Operator |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TASK NUMBER 88K11C18 |  |  |  | TASK TITLE Introduction To Navigation (Piloting) |  |  |  |  |  |
| Reinforced Task(s) | TASK NUMBER |  |  |  | TASK TITLE |  |  |  |  |  |
| Academic Hours: | The Academic hours required to teach this course are as follows: |  |  |  |  |  |  |  |  |  |
| *TEST *TEST REVIEW TOTAL HOURS | PEACETIME HOURS/METHOD 3.0/C 3.0/PE3 / /0000000000 6.0 |  |  |  | MOBILIZATION HOURS/METHOD / / / /000000000000000 |  |  |  |  |  |
| Test Lesson [List the lesson number in which the terminal learning objective of Number this lesson are tested and the test results are reviewed with the students.] |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Hours |  |  | Lesson No. |  |
|  |  |  | Testing: |  |  | _______________ _______________ |  |  | _______________ _______________ |  |
|  |  |  | Review of test results: |  |  | _______________ |  |  | _______________ |  |
| Prerequisite Lesson(s) |  | (State if there are none.) LESSON NUMBER None |  |  |  | LESSON TITLE |  |  |  |  |
| Clearance and Access |  | UNCLASSIFIED (U) |  |  |  |  |  |  |  |  |
| References |  |  |  |  |  |  |  |  |  |  |
|  |  | Number FM 55-501 |  | Title Marine Crewman’s Handbook |  |  | Date 15 Mar 1983 | Para No. |  | Additional Information |

| Student Study Assignments | None |  |  |  |
| --- | --- | --- | --- | --- |
| Instructor Requirements | [These are special qualifications or additional requirements.] |  |  |  |
| Additional Support Personnel Requirements | None |  |  |  |
| Equipment Required | Classroom Overhead Projector |  |  |  |
| Materials Required | Pencil, paper, chalk and eraser |  |  |  |
| Classroom, Training Area, and Range Requirements | with chalkboard |  |  |  |
| Ammunition Requirements | None |  |  |  |
| Instructional Guidance | Ensure facilities, equipment, and training materials are ready prior to class start time. Instructors will have a thorough knowledge by studying this lesson and identified reference material. |  |  |  |
| Proponent Lesson Plan Approvals | NAME | Rank | Position | Date |

SECTION II - INTRODUCTION

|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: 0.1 hr |
| --- | --- |
| Motivator | Motivate students to learn the Terminal Learning Objective |
| Show VG 2-9 Terminal Learning Objective | Identification of the earth’s system of coordinates, determining distance, position using Longitude and Latitude, and Time, Speed and Distance. |

| ACTION: | After this lesson unit the student will demonstrate his/her knowledge of Navigation (Piloting) |
| --- | --- |
| CONDITIONS: | Given FM 55-501, five question check-on-learning quiz. |
| STANDARD: | To receive a "GO" for this lesson unit the student must answer all check-on-learning questions correctly. |

| Safety Requirements | [State if there are none.] None |
| --- | --- |
| Risk Assessment Level | Low |
| Environmental Considerations | [State if there are none.] None |
| Evaluation | None |
| Instructional Lead-in | Let's begin by discussing Navigation (Piloting) |

SECTION III - PRESENTATION

| Note: | Inform the students of the Enabling Learning Objectives requirements. |
| --- | --- |
| A. | ENABLING LEARNING OBJECTIVE A |

| ACTION: | After this Lesson Unit, the students will answer five questions regarding the policies and procedures governing the conduct of this course/track of instruction. |
| --- | --- |
| CONDITIONS: | Given class notes, the evaluation plan and a five question quiz. |
| STANDARD: | To receive a “GO” for this Lesson Unit the student will answer five questions correctly. |

| 1. | Learning Step/Activity 1 [A statement describing what is to be done.] |
| --- | --- |
|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: 0.1hr Media VGT. |
| NOTE: | Ask check and developmental questions throughout Refer students to FM 55-501 |
| NOTE NOTE NOTE: NOTE: | A. MATERIALS EQUIPMENT Have students read definitions aloud and discuss - EQUIPMENT: Dividers, Pencil, Paper, Overhead Projector. - MATERIALS: Chart TR 1210 BODY #### Ask check and development questions throughout Develop the following on the chalkboard. ##### A. CHARACTERISTICS OF A CHART - Lines (1) Lines that run horizontal on a chart are known as parallels of latitude, measured from the equator North 0 to 90 and South 0 to 90. (2) Lines that run vertical on a chart known as meridians, from Greenwich prime meridian 0 to 180 East and West. - Compass Rose is a circle graduated in degrees, clockwise 000 at the reference station to 360. Compass Rose are placed at locations on charts to ease measurement of direction. - Legend (1) Legend will show the title (place) of the chart which describes the waters covered. (2) Type of projection used. (3) Scale of the chart. (4)Method of measuring soundings (depth of water) and other Useful information. d. Boxes and Notes: Used for information such as warnings and caution areas and general information to assist the mariner. Boxes #### and notes are printed in the margins or on the face of the chart at locations where it will not block out other navigational information e. Color of the charts: (1)Yellowish or Buff: Land areas. (2) Blue: Shallow or shoal waters. (3) White: Deep water (4) Green: Submerged at some tidal stage, such as sandbars, mud Flats, marshes. (5) Nautical Purple (Magenta): Used for written information listed on the chart; because it is easier to read under red lights that are used during hours of darkness on the bridge. f. Aids To Navigation (ATONS): Aids to navigation are put at various points along the coasts and navigable waterways as markers and guides to help mariners determine their position. They also serve to warn of hidden dangers and assist in making Landfall when approaching from high seas. #### Use training chart TR1210, show the students different types of aids To navigation. Conduct check on learning and summarize the learning activity. |

| Show VG | ###### B. DETERMINE DISTANCE and FINDING ONES OWN POSITI0N ACTION: After this lesson unit you (the student) will demonstrate His/her ability to determine distance and find your position on a chart. CONDITIONS: Given a check on learning with student notes. STANDARSDS: To receive a “GO” the student must answer all Questions correctly. Ask check questions throughout the lesson. Determine Distance and find ones own position. Type of instruction: C Instructor to student ratio is: 1:Class Time of instruction: 0.5 hours. Media: View graph, chart Have student follow along using TR1210. Show slide one - There are two primary scales used for determining distance on a chart. - Nautical miles scale. (2) Latitude scale is located on the left and right side of the chart as the chart is read. One minute latitude is equal to one one (1) nautical mile. |
| --- | --- |
| NOTE: NOTE: NOTE: NOTE: NOTE: | Demonstrate the use of each scale using the view graph and a pair of dividers. Show slide two. b. To determine your location you must use the Latitude and Longitude scale. Demonstrate to the students how to determine your position using the view graph and a pair of dividers. Conduct a check on learning and summarize the learning activity. ## C. COMPUTE TIME, SPEED and DISTANCE ACTION: After this lesson unit you (the student) will compute time, Speed and distance. CONDITIONS: Given problems involving time, speed and distance Without student notes. STANDARDS: To receive a “GO” in this lesson unit the student must Compute the distance within +/- 0.5 nautical miles; time Within +/- 1.0 minute and speed within +/- 0.5 knots. Ask check questions throughout the lesson. Compute Time, Speed and Distance Type of instruction: C Instructor to student ratio is 1:Class Time of instruction: 1.0 hours. Media: Chalkboard. a. The 60 minute rule. (1) Sixty (60) is used as a constant of time (2) This method for computing time, speed or distance requires that you now Two factors in order to determine the third. (3) The formula are as follow. a. T=D x 60 S b. S=D x 60 T c. D=T x S 60 |
| NOTE: NOTE: NOTE: | Demonstrate step by step, the following problems on the chalk board. (4)Time = 6 x 60 = 360 = 45 minutes - 8 (5) Speed =7 x 60 = 420 = 14 knots 30 30 - Distance = 20 x 15 = 300 = 5 miles - 60 Continue to develop more problems as needed to train the student. Conduct a check on learning and summarize the learning activity. |

SECTION IV - SUMMARY

|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: 0.2hr |
| --- | --- |
| Review/ Summarize Lesson | SUMMARY 1. Have students remove everything off their desks except pencil and paper. a. Soliciting questions and explanations. b. Asking questions and getting answers from the students. b. Correcting student misunderstandings 2. Review last hour of instruction in the form of an after-action review with focus on all learning steps in this lesson. IV, ADMINISTRATIVE INFORMATION: B. Inclement weather plan: N/A A. Major support failure plan: N/A |
| Check on Learning | Students will answer the following questions without the use of books or notes. SUMMARY #### CHECK-ON-LEARNING Q. What items are generally covered in a chart’s legend? A. Chart title, type projection, scale used, unit of measurement for water depths, and datum plane for the soundings. Q. What colors are depicted on a chart and what do they represent? A. Buff or yellowish and gray- Land areas, blue- Shallow waters, white- deep water areas, green – Areas that may be submerged during tidal changes, and magneta- general information. Q. What are the lines which run horizontally and vertically called? A. Horizontal- Paralells of latitude, Vertically- Meridians of Longitude? Q. When computing time, speed or distance, how many factors must be given? A. Two Q. How is distance measured on a chart? A. Latitude scale and Nautical mile scale. |
| Transition To Next Lesson | This concludes the instruction on Introduction to Navigation (Piloting). Your next lesson will be on. |

SECTION V - STUDENT EVALUATION

| Testing Requirements | This block of instruction it testable on IBM exam 9G-103-02 and the MTE. Exam 9G-601-16. |
| --- | --- |
| Note: | Refer student to the Student Evaluation Plan |
| Feedback Requirement | a. Schedule and provide feedback on the evaluation and any information to help answer students’ questions about the test. |
|  | b. Provide remedial training as needed. |
| Note: | Rapid, immediate feedback is essential to effective learning. |

PRACTICAL EXERCISE SHEET NO.

| Title | Determining Position using Latitude, Longitude, Determining Distance and Chart Characteristics. |  |
| --- | --- | --- |
| Introduction | This exercise is designed to enhance your ability to determine positions using latitude and longitude. To determine distance using the nautical mile scale, and identify the different types of aids to navigation on a chart. |  |
| Motivator | Motivate students to learn the Terminal Learning Objectives. |  |
| Terminal Learning Objective | Identification of the earth’s system of coordinates, determining distance, position using Longitude and Latitude, and Time, Speed and Distance. At the completion of this lesson you (the student) will: |  |
|  | ACTION: | Determine latitude and longitude, distances, and characteristics of a chart. |
|  | CONDITIONS: | Given practical exercise 421-155-06, set of dividers, parallel Ruler and training chart 1210. |
|  | STANDARD: | To receive a go in this practical exercise you must compute all answers accurately to within +/- 0.2 nautical miles and =/-4 minutes. |

| Safety Requirements | [State if there are none.] None |
| --- | --- |
| Risk Assessment Level | Low |
| Environmental Considerations | [State if there are none.] None |
| Evaluation |  |
| Instructional Lead-in | Let’s begin by doing the first requirement together. |
| Resource Requirements | None |
| Special Instructions | None |
| Procedures | 1. 2. |
| Feedback Requirements | None |

PRACTICAL EXERCISE I*

TITLE: Determining Position using Latitude, Longitude, Determining Distance and Chart teristics.

INTRODUCTION: This exercise is designed to enhance your ability to determine positions using latitude and longitude. To determine distance using the nautical mile scale, and identify the

Different types of aids to navigation on a chart.

MOTIVATOR: Motivate students to learn the Terminal Learning Objective.

TERMINAL LEARNING OBJECTIVE: Identification of the earth’s system of coordinates,

Determining, distance, position using Longitude and Latitude, and Time, Speed and Distance.

At the completion of this lesson you (the student) will:

ACTION: Determine latitude and longitude, distances, and characteristics  of a chart.

CONDITIONS: Given practical exercise 421-155- 06, set of dividers, parallel ruler and    Training chart TR 1210.

STANDARD: To receive a go in this practical exercise you must compute all answers

Accurately to within +/- 0.2 nautical miles and +/- 4 minutes of latitude and longitude.

SAFETY REQUIREMENTS; None

RISK ASSESSMENT LEVEL: Low

ENVIROMENTAL CONDSIDERATIONS: None

EVALUATION:

INSTRUCTIONAL LEAD IN: Let’s begin by doing the first requirements together.

RESOURCE REQUIRES: None

SPECIAL INSTRUCTIONS: None

FIRST REQUIREMENTS

What is the aid to navigation at latitude 4115’07”N and longitude 7046’03”W ?

______________________________________

SECOND REQUIREMENTS

What is the aid to navigation at latitude 4117’N and longitude 7104’W ?

______________________________________

THIRD REQUIREMENTS

What is the latitude and longitude of Buzzards Fl 2.5 Sec. 101 Ft. 22M Horn?

LATITUDE_________________

LONTIUDE_________________

FOURTH REQUIREMENTS

What aid to navigation is located 4125’06”N and longitude 7123’03 W?

___________________________________

FIFTH REQUIREMENTS

What type of bottom is located at latitude 4111’25’’N and 7057’04’’ W ?

___________________________________

SIXTH REQUIREMENTS

What is the latitude and longitude of point Judith light ?

LATITUDE_____________________

LONGITUDE____________________

SEVENTH REGUIREMNT

What are the characteristics of Point Judith light?

EIGHTH REQUIREMENTS

What is the distance between the Fl   G 4 Sec Gong (off of Gay Head) to the RW “NA”

Mo (A) Whistle (north of Gay Head) ?

NINTH REQUIREMENT

What is the distance between Buzzards Fl 2.5 Sec 101 Ft. 22M Horn to Gay Head

Alt Gp Fl (3) W& (1) R 40 Sec 170 Ft 17 M Light

TENTH REQUREMENTS

From the ninth requirement, what is the direction from Buzzard light to the Gay Head light.

________________________________________________

FEEDBACK REQUIREMENT:

- Provide feedback on the practical exercise and any misunderstandings the student may have.

b.    Provide remedial training as needed.

SOLUTION SHEET II

TITLE: Determining Position using Latitude, Longitude, Determining Distance and

Chart Characteristics.

INTRODUCTION:  This exercise is designed to enhance your ability to determine position using

Latitude and longitude. To determine distance using the nautical mile scale, and

Identify the different types of aids to navigation on a chart.

MOTIVATOR:        Motivate students to learn the Terminal Learning Objective.

TERMINAL LEARNING OBJECTIVE: Identification of the earth’s system of coordinates,

Determining distance, position using Longitude and Latitude, and Time, Speed and Distance.

At the completion of this lesson you (the student) will:

ACTION:  Determine latitude and longitude, distances, and characteristics of a chart.

CONDITIONS:  Given practical exercise 421-155- 06, set of dividers, parallel ruler and

Training chart TR 1210.

STANDARD:   To receive a go in this practical exercise you must compute all answers

Accurately to within +/- 0.2 nautical miles and +/- 4 minutes of latitude

And longitude.

SAFETY REQUIREMENTS: None

RISK ASSESSMENT LEVEL: Low

ENVIRONMENTAL CONSIDERATIONS: None

EVALUTION:

INSTRUCTIONAL LEAD IN: Let’s begin by going over the first requirement.

RESOURCE REQUIREMENTS: None

SPECIAL INSTRUCTIONS: None

FIRST REQUIREMENT

What is the aid to navigation at latitude 4115’07” N and longitude 7046’03” W ?

ANSWER: Fi G 4Sec Bell.

SECOND REQUIREMENT

What is the aid to navigation at latitude 4117’ N and longitude 7104” W ?

ANSWER: Unexploded depth charge.

THIRD REQUIREMENT

What is the latitude and longitude of Buzzards Fl 2.5 Sec 101 Ft. 22M Horn?

LATITUDE: 4124’08” N

LONGITUDE: 7103’ W

FOURTH REQUIREMENT

What aid to navigation is located at latitude 4125’06’’ N and longitude 7123’03” W ?

ANSWER: Brenton Reef Gp Fl (2) 10 Sec 87 Ft 25  M Horn.

FIFTH REQUIREMENT

What type of bottom is located at latitude 4111’25” N and 7057’04 W ?

ANSWER: Hard bottom

SIXHT REQUIREMENT

What is the latitude and longitude of Point Judith light ?

ANSWER: Latitude: 4121’07” N

Longitude: 7128’09” W

SEVENTH REQUIREMENT

What are the characteristics of Point Judith light ?

ANSWER: Gp Occ (1+2) 15 Sec 65 Ft 16 M Horn

EIGHTH REQUIREMENT

What is the distance between the Fl  G 4 Sec Gong (off of Gay Head) to RW “NA”

Mo (A) Whistle (north of Gay Head) ?

ANSWER: 2.5 NM.

NINTH REQUIREMENT

What is the distance between Buzzards Fl 2.5 Sec 101 Ft  22M Horn to Gay Head

Alt Gp Fl (3) W& (1) R 40 Sec 170 Ft 17 M Light.

ANSWER: 9.4 NM

TENTH REQUIREMENT

From the ninth requirement what is the direction from Buzzards light to the Gay Head light ?

ANSWER: 108

FEEDBACK REQUIREMENT:

- Provide feedback on the practical exercise and any misunderstandings the student may have.

- Provide remedial training as needed.

PRACTICAL EXERCISE III

TITLE: Time, Speed and Distance

INTRODUCTION: This exercise is designed to enhance your ability to determine the appropriate

Formula and use to solve time, speed and distance problems.

MOTIVATOR:       Motivate students to learn the Terminal Learning Objective.

TERMINAL LEARNING OBJECTIVE: Identification of the earth’s system of coordinates,

Determining distance, position using Longitude and Latitude, and Time, Speed and Distance.

At the completion of this lesson you (the student) will:

ACTION: Compute time, speed and distance.

CONDITIONS: Given practical exercise 421-155-06 and formula form  for time, speed and

Distance.

STANDARD:    To receive a go in this practical exercise you must compute all answers

Accurately to within +/- 0.2 of nautical miles, time within +/- 1.0 minutes

And speed within +/- 0.5 knots.

SAFETY REQUIREMENTS: None

RISK ASSESSMENT LEVEL: Low

ENVIRONMENTAL CONSIDERATIONS: None

EVALUATION:

INSTRUCTIONAL LEAD IN: Let’s begin by doing the first requirements together.

RESOURCE REQUIREMENTS: None

SPECIAL INSTRUCTIONS: None

FIRST REQUIREMENTS

- Utilizing the appropriate formula, solve for time, speed and distance. Compute to the nearest minute in time, nearest tenth in speed and nearest tenth in distance.

TIME (in hr minutes)                                 SPEED                                       DISTANCE

1.  00hr 40 min                                           ______                                                  10

2.  00hr 45 min                                           ______                                                  12

3.  00hr 11 min                                           ______                                                  3.7

4.  00hr 12 min                                           ______                                                  2.6

5.  00hr 21 min                                               14                                                      _______

6.  00hr 22 min                                               12                                                      _______

7.  00hr 32 min                                               11                                                      ________

8.  00hr 45 min                                               16                                                       ________

9.  __________                                               18                                                         12

10.__________                                               10                                                         6.5

11.__________                                                7                                                          4.3

12.__________                                                9                                                          3.6

13.__________                                               8.5                                                        12

14.  00hr 20 min                                              _______                                              4.9

15.  00hr 50 min                                              9                                                          ________

INTRODUCTION TO NAVIGATION

SOLUTIO0N SHEET

FIRST RQUIREMENT

- Answer sheet for PE time, speed and distance.

TIME (in hr and minutes)                                         SPEED                                         DISTANCE

1. 00hr 40 min.                                                          15 kts.                                          10 NM

2. 00hr 45 min.                                                          16 kts.                                          12 NM

3. 00hr 11 min.                                                          20.18 or 20.2 kts                          3.7 NM

4. 00hr 12 min.                                                          13 kts.                                           2.6 NM

5. 00hr 21 min.                                                          14 kts                                             4.9 NM

6. 00hr 22 min.                                                          12 kts                                             4.4 NM

7. 00hr 32 min                                                           11 kts                                      5.86 or 5.9 NM

8. 00hr 45 min.                                                          16 kts                                             12 NM

9. 40 min.                                                                  18 kts                                             12 NM

10. 39 min.                                                                10 kts.                                            6.5 NM

11.37 min                                                                  7 kts.                                              4.3 NM

12. 24 min.                                                                9 kts.                                              3.6 NM

13. 1 hr. 25 min.                                                       8.5                                                  12 NM

14. 00hr 20 min.                                                        14.7 kts.                                         4.9 NM

15. 00hr 50 min.                                                       9 kts.                                                7.5 NM

SECOND REQUIREMENT

How log would it take you to travel 30 miles at a speed of 11 knots ?

THIRD REQUIREMENT

Your vessel traveled 35 miles in 3 hours. What  was your speed ?

FOURTH REQUIREMENT

You have been underway for 4 hours and 30 minutes your was 20 knots. What is the distance you

traveled?

SOLUTION SHEET

SECOND REQUIREMENT

How long would it take you to travel 30 miles at a speed of 11 knots?

T= D x 60

S

30 x 60 = 1800 = 164 min or 2 hors 44 minutes

- 11

THIRD REQUIREMENT

Your vessel traveled 35 miles in 3 hours. What was your speed?

S = D x 60

T

35 x 60 = 2100 = 11.66kts. or 11.7 kts.

- 180

FOURTH REQUIREMENT

You have been underway for 4 hours and 30 minutes your speed was 20 knots. What is the distance you

Traveled?

D = S x T

60

D = 20 x 270 = 5400 = 90 NM

60            60