TRAINING SUPPORT PACKAGE (TSP)

Note: This is the TSP title page

| TSP Number | 88K13C10 |
| --- | --- |
| Task Number(s) Title(s) | Marine Radio Telephone Procedure and PMCS for Marine Radio |
| Effective Date | 1 October 1997 |
| Supersedes TSP(s) | PFN 421-157-04 22 FEBUARY 1995 |
| TSP User | MOS 88K WATERCRAFT OPERATOR |
| Proponent | DEPARTMENT OF THE ARMY TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street Suite 227 Ft. Lee, VA 23801-1511 |
| Comments/ Recommen- dations | Send comments and recommendations directly to: DEPARTMENT OF THE ARMY TRAINING DIRECTORATE ATTN: ATCL-AT 401 First Street Suite 227 Ft. Lee, VA 23801-1511 |
| Foreign Disclosure Restrictions | This product has been reviewed by the product developers in coordination with the Commander, U.S. Army Combined Arms Support Command foreign disclosure authority. This product is releasable to military students from foreign countries on a case-by-case basis. |

PREFACE

Purpose	This training support package provides the instructor with a

standardized lesson plan for presenting instruction for:

TASK NUMBER:

TASK TITLE: Marine Radio Telephone Procedure and PMCS for Marine Radio

CONDITIONS: Given class notes, the evaluation plan and a five questions quiz.

STANDARD: To receive a “GO” for this Lesson Unit the student will answer five questions correctly.

This TSP

contains

| TABLE OF CONTENTS |  |  |
| --- | --- | --- |
|  |  | Page |
| Preface |  | 2 |
| Lesson | Section I - Administrative Data | 3 |
| Plan | Section II - Introduction | 6 |
|  | Terminal Learning Objective | 6 |
|  | Section III - Presentation |  |
|  | A. Enabling Learning Objective 1 | 7 |
|  | B. Enabling Learning Objective 2 |  |
|  | C. (etc.) |  |
|  | Section IV - Summary | 12 |
|  | Section V - Student Evaluation | 13 |
| Appendixes | A. Viewgraph Masters |  |
|  | B. Test and Test Solutions |  |
|  | C. Practical Exercises and Solutions | 14 |
|  | D. Student Handouts |  |

Watercraft Operator

[Effective Date]

SECTION I - ADMINISTRATIVE DATA

| All Courses Including This Lesson | COURSE NUMBER(S) 88K10 |  | COURSE TITLE(S #### Water Craft Operator |  |  |
| --- | --- | --- | --- | --- | --- |
|  | TASK NUMBER #### 88K12E10 |  | TASK TITLE #### Marine Radio Telephone Procedure and PMCS for Marine Radio |  |  |
| Reinforced Task(s) | TASK NUMBER 551-733-2612 |  | TASK TITLE Maintain Radio Communication Equipment |  |  |
| Academic Hours: | The Academic hours required to teach this course are as follows: |  |  |  |  |
| TOTAL HOURS | PEACETIME HOURS/METHOD 2.0/C 2.0/PE / /0000000000 4.0 |  | MOBILIZATION HOURS/METHOD / / / /000000000000000 |  |  |
| Test Lesson [List the lesson number in which the terminal learning objective of Number this lesson are tested and the test results are reviewed with the students.] |  |  |  |  |  |
|  |  |  |  | Hours | Lesson No. |
|  |  | Testing: |  | _______________ _______________ | _______________ _______________ |
|  |  | Review of test results: |  | _______________ | _______________ |

| Prerequisite Lesson(s) | (State if there are none.) None | LESSON TITLE |
| --- | --- | --- |
| Clearance and Access | UNCLASSIFIED (U) |  |

| References |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Number FM 55-501 | Title Marine Crewman’s Handbook | Date 15 Mar 1983 | Para No. | Additional Information |

| Student Study Assignments | None |
| --- | --- |
| Instructor Requirements | [These are special qualifications or additional requirements.] |
| Additional Support Personnel Requirements | None |
| Equipment Required | VRC Radio’s Overhead Projector May use a vessel with the proper radio equipment for the PE. |
| Materials Required | Pencil, paper, chalk and eraser |

| Classroom, Training Area, and Range Requirements | Classroom with chalkboard |
| --- | --- |
| Ammunition Requirements | None |
| Instructional Guidance | Note: Ensure facilities, equipment, and training materials are ready prior to class start time. Instructors will have a thorough knowledge by studying this lesson and identified reference material. |

| Proponent Lesson Plan Approvals | NAME ___________________ ___________________ | Rank _____ _____ | Position __________________ __________________ | Date _______ _______ |
| --- | --- | --- | --- | --- |

SECTION II - INTRODUCTION

|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: .1HR |
| --- | --- |
| Motivator | As a seaman onboard a U.S. Army watercraft you will be required to perform PMCS on different types of equipment. The importance of PMCS can’t be stressed enough. If the equipment is not maintained properly it will not function when you need it. This could prolong your mission and could cause it to be a failure. |
| Show VG 1-13 Terminal Learning Objective | Performing preventive checks and services on marine radios. |

| ACTION: | After this lesson unit, the student will have a basic understanding of PMCS, the use of the DA Form 2404 and how to perform PMCS on marine radios. |
| --- | --- |
| CONDITIONS: | Given a six question quiz, DA Form 2404, technical manual, and class notes. |
| STANDARD: | To receive a "GO" for this lesson unit, the student must answer all check-on-learning questions correctly and perform a correct PMCS on the marine radios. |

| Safety Requirements | See the safety annex |
| --- | --- |
| Risk Assessment Level | Low |
| Environmental Considerations | [State if there are none.] None |
| Evaluation | None |
| Instructional Lead-in | Let’s begin by discussing PMCS. |

SECTION III - PRESENTATION

| Note: | Inform the students of the Enabling Learning Objectives requirements. |
| --- | --- |
|  | ENABLING LEARNING OBJECTIVE |

| ACTION: | After this Lesson Unit, the students will answer six questions regarding the policies and procedures governing the conduct of this course/track of instruction. |
| --- | --- |
| CONDITIONS: | Given class notes, the evaluation plan and a six questions quiz. |
| STANDARD: | To receive a “GO” for this Lesson Unit the student will answer six questions correctly. |

| 1. | Learning Step/Activity 1 [A statement describing what is to be done.] |
| --- | --- |
|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: 1.0HR Media VGT |
| NOTE: | Issue each student a TM 11-582-890-10-1 and/or 11-5820-401-10-1and a copy of a DA form 2404. |
| # NOTE # NOTE # NOTE # NOTE # NOTE | A.. PREVENTIVE MAINTENANCE CHECKS AND SERVICES - PMCS. performed daily, is the only way for operators to ensures that equipment is mission ready. Each piece of Army equipment comes with technical manual (TM). The TM is the operator’s reference in the daily operation and maintenance of essued equipment. The TM contains a section which gives, step by step, instruction on how to perform the PMCS for the equipment cover in the manual. - PMCS is performed on each piece of equipment before use. It is the supervisor’s job to ensure that the assigned operator performs the PMCS and that the equipment is mission ready. 3. The operator is responsible for notifying immediately the supervisors, or whoever is designated, if the equipment is not operating correctly 4. Operating a defective radio can cause injury to personnel and equipment, as well as jeopardize the success of the mission. 5 . To perform the PMCS, the soldier must use the applicable TM for the equipment and record all findings on DA Form 2404. B. DA FORM 2404 - The DA form 2404, equipment Inspection Maintenance worksheet is a widely used form. It is use to record all deficiencies of equipment. For this lesson unit the procedures on how to complete the 2404 will be based on chapter 5 of TM 11-5820-10-1 and/ or chapter 3 of TM 11-582-401-10-1 that you have in front of you. Have students follow with copy of DA form 2404. 2. Entries: a. Heading: (1) Organizational: Your unit name. (2) Nomenclature and model: Name and model of the equipment. (3) Registration /serial /NSN: Serial number or USA registration number. (4) Blocks: (a) Miles (only on vehicles with odometers). (b) Hours (only on equipmentwith odometers). (c) Rounds fired (only on weapons). (d) Hot starts (used with diesel and turbine engines) . (5) DATE: Calendar date. (6) Type Inspection: Technical Inspection (TI) PMCS, Turn In. (7) Reference: TM used to include number and date. (if two TMe are used list both). - Signature: Note the statement above the signature block. Refer students to DA form 2404 and explain status symbols, have student also explain in their own words what symbols mean. B. Status symbols: (1) “x” - Indicates equipment is in a operable status. (2) Circled “x” - indicates deficiency however the equipment may be operated under limitations directed by the commander or a higher authority. (3) “ - “ Indicates that required maintenance is due but has not been accomplished. (4) “ / “ Indicates a deficiency that must be correctd to increase the efficiency. C. Colum instruction: (1) Colum a - Enter TM item number located in PMCS section of the TM. Advise students that the item number will be consistent with the TM item number. (Example: if there are no deficiencies for them #1, #2 and item #4 has deficiencies then the the 2404 will start with item number4.) (2) Column b-Enter the applicable condition status symbol. (See status symbols.) - Colum c-Enter deficiencies and shortcoming. (4) Colum d - Show corrective actions. (5) Colum e - Initials of person who actually performed the corrective action. D. Disposition of DA form 2404: (1) Turn in copy to supervisor. (2) Keep a copy for your records. Prior to break, ask if there are any questions. Have students turn to table of contents on TM 11-5820-401-1 and discuss each chapter. C. RADIO TECHNICAL MANUAL 1. The technical manual provides the operator with information on the equipment, operational features, operational instrutions, and maintenance instructions. a. Chapter 1. Introduction b. Chapter 2. Operation instructions. c. Chapter 3. Maintenance Instructions. 2. Intervals: When items should be checked. B = Before operation. D = During operation. A = After operation. W = Weekly. M = Monthly. Ensure that the students are familiar with the meaning of “A” (annual), “BA” (Biannual). Have students turn to the PMCS section in the manual and explain the following. 3. Item number - The number of the PMCS sequence of items to be checked starting with number I through the last number int he checklist. 4. Items to be inspected - Components of the equipment you will inspect. 5. Procedures - Indicates what you are checking or looking for on the equipment. (a) chips (i) worn (q) missing parts (b) dents (j) cuts (r) dirty (c) loose fittings (K) stuck (s) secured (d) broken (l) cracks (t) other (e) frail (m) complete (f) splits (n) corrosion (g) rust (o) operational (h) rips (p) salt deposits 6. Readiness Reporting: This column states the condition the item- will be in for the equipment to be considered.not mission capable, commonly referred to as deadlined. If the equipment is in the condition stated in this column, or worse, the equipment should not be operated. Operation of the equipment, may cause further damage to the equipment or injury to the operator. TYPE OF PMCS’s - In addition to the normal intervalfor PMCS, you may have to perform a PMCS for the following reasons. 1. Receipt of a new radio. 2. Turn-in of the radio for storage. 3. Turn-in of the radio for repair. 4. Turn-in of the radio for temporary use by another vessel or party. 5. Turn-in of the radio due to receipt of a new model or new type of radio. Have students refer to the DA form 2404 for quiz. |

SECTION IV - SUMMARY

|  | Method of instruction: C Instructor to student ratio is 1:C Time of instruction: .2 HR |
| --- | --- |
| NOTE Review/ Summarize Lesson | Review the last four hours block of instructions. |
| Check on Learning | Q. What does a circled x indicate on a DA form 2404? A. Q. When should PMCS be performed on equipment? A. Q When should Column e of the DA Form 2404 be initialed? A. Q The TM provides the user with three important factors, what are they? A. Q What are the type of PMCS in addition to the normal inerval PMCS requirments? A . Q List eight (8) items of the possible twenty (20) that the operator should look for when performing PMCS? A |
| Transition To Next Lesson | This concludes the class on preventive maintenance checks and services on marine radios. |

SECTION V - STUDENT EVALUATION

| Testing Require- ments | B. This block of instruction it testable on IBM exam number 9G6-101-02- and the MTE. |
| --- | --- |
| Note: | Refer student to the Student Evaluation Plan |
| Feedback Requirement | a. Schedule and provide feedback on the evaluation and any information to help answer students’ questions about the test. |
|  | b. Provide remedial training as needed. |
| Note: | Rapid, immediate feedback is essential to effective learning. |

PRACTICAL EXERCISE SHEET NO.

| Title | Marine Radio Telephone Procedure and PMCS for Marine Radio. |  |
| --- | --- | --- |
| Introduction |  |  |
| Note |  |  |
| Motivator |  |  |
| Terminal Learning Objective | Now that you have mastered the paper involved with performing PMCS, you will now perform PMCS on a marine radio. |  |
|  | ACTION: | After this lesson unit, the student will have a basic understanding of how to perform PMCS on marine radios. |
|  | CONDITIONS: | Given a DA form 2404, technical manual and class notes. |
|  | STANDARD: | To receive a GO for this lesson unit, the student must correctly perform a correct PMCS on the marine radios. |

| Safety Requirements | See the safety annex |
| --- | --- |
| Risk Assessment Level | Low |
| Environmental Considerations | [State if there are none.] None |

| Evaluation |  |
| --- | --- |
| Instructional Lead-in | None |
| # NOTE | Issue students TM’s and DA form 2404. Divide students into groups small enough to perform PMCS effectively. |
| Resource Require- ments | None |
| Special Instructions | None |
| Procedures | The students will conduct PMCS TAW TM’s provided and DA form 2404. |
| Feedback Require- ments | None |
| # SUMMARY | A. Review last four hours of instruction. - This block of instruction is not examinable. - Summarize the practical exercise, answer all students questions. |
| # AFTER ACTION REVEW | Critique practical exercise. |

U.S. ARMY TRANSPORTATION SCHOOL

WATERCRAFT OPERATION COURSE

PREVENTIVE MAINTENANCE CHECKS AND SERVICES FOR MARINE RADIOS

SAFETY ANNEX

I.	INTRODUCTION:

This annex is directed toward resident and non-resident students involved in radios or electrical equipment repairs.  It is developed to aid the instructor in the reduction of elimination of shock, damage to equipment and injury to students during training.

A. In order to reduce electrcal shock or injury,  the instructor will provide, equipment necessary for safety and thoroughly brief students prior to each pralctical exercise.

I .  Remove watches and rings when working on equipment.

2.  Observe all warning hazards thechnical manuals.

3.  Use the buddy system.

4.  Know where the safety board is located.

5.  Turn of or unplug equipment during repairing.

6.  Insure is off and no one will key radio while inspecting the antenna.

7   Use safety ground strap.

8.  Use rubber mat.

9.  Ground all equipment.

10. Do not put liquid on top of equipment

11  Do not operate equipment with wet hands.

12. Do not unplug equipment by pulling on the cord.

13. Replace fuses with the same size and type.

14. Do not transmit two radios side by side.

15  Do not key the radio more than five minutes at a time.

16. Insure that the antenna matches the radio.

B.      The instructor must inform the students of all hazards of working with electrical equipment.

1.  Turn off power source during electrical shock.

2.  Reac to a shock victim.

3.  Be familiar with the purpose of each item on the safety board.

4.  “ THINK “  and apply safety in all areas of training.