# Section B

# Training

# Overview

This page intentionally left blank

Training and Use of the AN/PSS-12 Handheld Mine Detector

### How the Detector Works

The detector emits a magnetic field symmetrically above and below the detector head. When this field encounters metal objects, it induces currents in these objects. The larger the metal object, the stronger the induced currents, and thus the greater distance at which the receiver component of the detector can pick up the induced currents. Thus, the closer you can position the detector head to metal in a mine, the greater chance you have of detecting the mine.

### Physical Setup

Extend telescopic pole to comfortable length for sweeping.

Attach cable from search head to the electronic unit.

Attach cable from earpiece to electronic unit.

Install fresh batteries in the battery compartment of the electronic unit.

Clamp cable from detector head only onto lower (plastic) portion of telescopic pole. Do not attach cable to upper aluminum shaft! Doing so may cause variation in detector performance.

Hang earpiece around neck or wear over ear, set volume to low level, turn power on, and reset volume to comfortable listening level.

### Initial Sensitivity Setting

Select a sensitivity target. The preferred target is the most difficult to detect type of mine (made safe without changing the metal content) that is expected to be encountered. In the absence of an actual mine, the 5cm test piece that comes with the AN/PSS-12 packed components may be used.

Bury and mark the sensitivity target. Bury the sensitivity target in soil similar to type expected in minefield and where there is no metal clutter. Depth of burial should be at the deepest depth that you expect to encounter a threat mine. If using the test piece, bury it vertically with the metal at the deepest end. In the absence of more specific information, bury the test piece at a depth of 2 inches (5cm). This places the top of the test piece flush with the surface of the ground. Mark the precise location of the buried target by placing a non-metallic marker such as a plastic poker chip directly over the target’s center.

Place the search head lightly on the ground directly above the sensitivity target by visually locating the marker in the center (bulls eye) of the search head. Adjust the SENSITIVITY knob until you hear a continuous audible signal. The system provides this signal when the search head is over or very near a mine or other metallic object. At the same time, adjust the LOUDNESS control to a comfortable listening level.

Carefully adjust the SENSITIVITY knob to a position between the points where the signal just begins to be heard and where you get a strong constant signal.

Lightly float the search head on the ground at one foot-per-second movement back and forth over the sensitivity target. With the search head in motion, check to see if the signal can again be easily heard. If you cannot hear the signal during movement at one foot-per-second, increase the sensitivity until you can hear the signal.

Maintaining sensitivity setting (A limitation of the PSS-12 is that sensitivity fluctuates during normal use. If not checked frequently and corrected, this fluctuation could lead to missing a mine.)

Place the search head again lightly on the ground directly above the sensitivity target and listen carefully to the audible signal. You will need to accurately remember how this signal sounds for maintaining this sensitivity setting. Move the search head away from the sensitivity target until the signal ceases to be heard. Slide a metal object down the telescopic pole of the detector until you hear an audible signal that sounds the same as when you placed the search head on the ground directly above the sensitivity target. A common mason's trowel is a convenient metal object to use for this. If one is not available a large knife or many other metal objects could be used. (What you are doing here is feeling for the edge of the electrical field that is induced by the trowel.)

Place the top edge of one of the cable clamps from the kit onto the telescopic pole at the tip of the trowel or knife. Note the angle the telescopic pole is relative to the ground and your body.

Maintain the detector sensitivity by rechecking and readjusting sensitivity if necessary about every two meters of advance in a mine lane. Make the check by placing the point of the trowel at the top of the cable clamp just like you did when initially positioning the clamp and listening for a like sound from the detector. Be sure the trowel or other metal object you may be using, telescopic pole angle relative to head, and electronic unit are in the same position as when you initially positioned the cable clamp. If sensitivity adjustment is frequently required, checks may need to be made more often.

NOTE! The process above describes the minimal sensitivity setting required. Anything less may result in missing a mine. If the soil allows, without producing a warning tone from the detector, the sensitivity may be set at higher levels. This theoretically yields a greater probability of detection at the cost of a higher false-alarm rate. The mission dictates the optimal setting of the sensitivity between just enough to detect the hardest to detect mine at the deepest depth to the highest setting the soil type will allow without generating a warning tone that could hide the tone indicating a mine. Due to time constraints, a tactical mission such as a combat breach may require a riskier, lower sensitivity setting. A higher sensitivity setting would be used during a non-time-constrained demining mission or clearing of a lodgment area or route.

### Sweep techniques

Sweep detector head at no more than one foot per second.

Float detector head lightly over surface of ground. Loosening the nut where the head pivots on the telescopic pole sometimes makes it easier to float the head over uneven ground.

If vegetation prevents being able to float detector head over ground, then lightly pat detector head on the surface.

Advance each sweep no more than 1/2 the width of the detector head.

Advance each pat no more than 1/2 the width of the detector head.

At edge of lanes, be sure that 1/2 of detector head overlaps edge of lane.

## Investigation/Mine Identification

At the first indication of metal in the ground, place a marker on the ground (at the center of the search head). Also, with a trowel or other tool, lightly scratch a line in the soil across the lane along the center of the last clear sweep. Use the marker to indicate where the investigation procedure begins and the line to indicate the area that has already been cleared. Sweep again across the marker to determine if the signal is repeatable and that the marker is accurately placed. If so, you should assume that the signal is a mine and continue the investigation with the purpose of determining the specific location and type of mine (high-metal or low-metal).

FOOTPRINT DEVELOPMENT. Identify the precise location of the mine by developing a set of points on the ground that outline the mine’s footprint. Develop this footprint by first moving the search head away from the signal source until no signal is heard and then sweeping the search head toward the signal from several clock positions. Note and/or mark the position of the center of the search head at each location where the auditory signal begins. Never sweep past the point where the auditory signal begins. This keeps the search head from getting any closer than necessary to a potential mine. Continue sweeping away from the signal source until no signal is heard and then sweeping toward the signal from several clock positions until the size and shape of the footprint of the signal can be understood. Typically, the pattern will resemble a semi-circle with the six-o’clock position nearest the operator and the center of the mine in the geometric center of the footprint.

Identify the type of mine by examining the size and shape of the mine’s footprint. Small footprints, often as small as four to six inches in diameter indicate low metal mines. High metal mines may have footprints two to four feet in diameter. The footprint is defined as the entire area on the ground where the mine or metallic source causes the detector to generate an auditory signal.

AIRBORNE TECHNIQUE. It is sometimes important to identify the center of very large footprints. A means to do this that may be quicker then the footprint development method is called the airborne technique. This technique involves adjusting the search head so it can be held above the ground but with the head parallel to the surface. Then the detector can be manipulated above the source until the signal can be heard at only a single point. Near the surface, the signal can be heard over a broad lateral area, but as the search head is raised higher off the ground, this area becomes progressively smaller. As the search head is raised higher off the ground, the area where the signal can still be heard is reduced to a point. The center of the mine or metal source should be directly below this point. Do not ignore the potential for low metal mines to be hidden in this large footprint on the ground.