### Section C

#### Training

And

Use

of the

AN/PSS-12

Handheld Mine

Detector

### This page intentionally left blank

### Training and Use of the AN/PSS-12 Handheld Mine Detector

### Philosophy of Detection

Like many other soldier tasks, successful mine detection is one of precision. Skills must be developed through the practice of proper techniques. These techniques must be practiced to the level that operators feel they are over trained. Even then, practice should continue to build upon the fundamental skills for soldiers to appropriately adapt them for a given tactical situation. A minefield is an impassionate and indiscriminant adversary. At the end of the day, the minefield itself will swiftly and savagely identify the under-prepared and under-skilled.

Mine detection is extremely tedious, stressful and tiring. Senior leaders must limit the time a soldier is actually working in the minefield. To determine a reasonable time, the leader must consider all the factors present which may reduce a soldier’s proficiency. These factors include but are not limited to METT-TC factors, weather conditions, the threat level, the unit’s stress level, and the soldier’s fatigue level and state of mind. As a rule, 20 to 30 minutes is the maximum amount of time that an individual can operate the detector effectively.

## How the Detector Works

The detector emits a magnetic field symmetrically above and below the search head. When this field encounters metal objects, it induces currents in these objects. The larger the metal object, the stronger the induced currents and thus the greater distance at which the receiver component of the detector can pick up the induced currents. Thus, the closer you can position the search head to the metal in a mine, the greater chance you have of detecting the mine.

## Unpacking the AN/PSS-12 Mine Detector For Operation

1. Open pressure relief valve.

2. Release latches and open metal transport case. Remove carry bag from metal transport case.

3. Unzip carry bag. Check for completeness by comparing contents with list of contents card. Remove electronics unit, headphone, telescopic pole with search head, four batteries, and two cable clamps from carrying bag.

NOTE: After removing Electronic Unit, ensure switch is set to off.

4. Check search head for cracks or damage.

5. Check cable connectors for damaged or bent pins.

6. Ensure that cables are not cut, broken or frayed.

7. Check hook and pile material on headphones for serviceability.

8. Check electronics unit for cracks, damage and completeness. Ensure that switch and knobs are present and functional.

9. Ensure that telescopic pole is not bent, dented, or damaged and can be extended and retracted.

10. Check battery compartments for two sets of batteries. Each set should contain four D-cell batteries in good condition.

11. Open accessory bag and ensure it contains the following accessories:

a. 5cm test piece for setting sensitivity

b. Condensed operating instructions

c. List of contents

d.·Spare plastic shear bolt for telescopic pole

e. Spare cable clamps

## Assembly

1. Ensure that power switch is in OFF position.

2. Release latches and remove battery cover.

3. Insert batteries according to the markings.

NOTE: Under severe cold weather conditions battery life may be extended by carrying the electronics unit under outer garments in order to keep the batteries warm.

4. Install and latch battery cover.

5. Press in the catch to unlock the telescopic pole from the transport position. This catch is located just below the arm support. Turn the outer tube until the catch snaps into the guide groove. Then pull the telescopic pole out to a suitable length. Ensure the telescopic pole is locked in one of the three fixed positions.

6. Connect cables to the electronics unit. Fit the protective caps of the cable connectors to the corresponding rubber caps on the electronics unit.

7. Fasten the cable to the telescopic pole with the cable clamps

8. Adjust the position of the handle by loosening the knurl nut.

9. Adjust position search head so that search head will be parallel to the ground.

10. Attach lower cable clamp 2 in. (5cm) from the wing nut.

NOTE: Cable clamps may weaken from extended use. Use only cable clamps that hold tightly onto the smaller plastic portion of the telescopic pole.

NOTE: Clamp cable from search head only onto lower (plastic) portion of telescopic pole. Do not attach cable to upper aluminum shaft! Doing so may cause variation in detector performance.

11. Attach the second cable clamp two inches from the top of the plastic pole.

12. Attach the third cable clamp in the middle, between the other cable clamps. Do not attach the cable to this middle clamp. This clamp will become the sensitivity marker clamp.

13. Put on the electronics unit by attaching the unit to the belt. Right-handed soldiers should put on the electronic unit on the left hip and left-handed soldiers on the right hip. Put the safety strap over the shoulder.

14. Put on the headphone. Place the straps of the headphone, over the forehead and top of the head. Hook and pile material goes behind the head at the nape of the neck.

NOTE: During training have the soldier hang the speaker around the neck so both the soldier and the instructor can hear the signal. This is done for instructional purposes only.

15. Put on the electronics unit by attaching the unit to the belt. Right-handed soldiers should put on the electronic unit on the left hip and left-handed soldiers on the right hip. Put the safety strap over the shoulder.

NOTE: The AN-PSS/12 detector performance is reduced when the cable between the electronics unit and the top cable clamp is permitted to hang unrestrained during sweeps. The soldier holding the cable in his free hand can control this problem.

## Preparation

1. Remove rings, watches, jewelry, before adjusting or using the mine detector.

2. Select a sensitivity target. The preferred target is the most difficult to detect type of mine (made safe without changing the metal content) that is expected to be encountered. In the absence of an actual mine, the 5cm test piece that comes with the AN/PSS-12 packed components may be used.

3. Bury and mark the sensitivity target. Bury the sensitivity target in soil similar to type expected in minefield and where there is no metal clutter. Depth of burial should be at the deepest depth that you expect to encounter a threat mine. If using the test piece, bury it vertically with the metal at the deepest end. In the absence of more specific information, bury the test piece at a depth of 2 inches (5cm). This places the top of the test piece flush with the surface of the ground. Mark the precise location of the buried target by placing a non-metallic marker such as a plastic poker chip directly over the target’s center.

NOTE: It is important to bury the sensitivity target in soil similar to where you will be actually using the detector. Otherwise, sensitivity setting and adjusting is simply a guess because different soil types and moisture content influence the sensitivity of the detector.

NOTE: Much of the ground, particularly on military installations and battlefield areas, contains large amounts of metallic clutter that will confuse the operator and interfere with setting up and adjusting the mine detector sensitivity. As a result, you may have to clear a small area before you can set up, adjust and use a mine detector. Clearing a small area may be accomplished as follows:

a.	Select a 1-meter by 0.5-meter area that you think is likely clear of clutter.

b.	Turn the sensitivity knob counterclockwise as far as it will go.

c.	Place the search head on top of the ground in the center of this 1-meter by 0.5-meter area and slowly turn the sensitivity knob clockwise until you begin to hear an auditory signal.

d.	While listening to the constant auditory signal, begin sliding (sweeping) the search head to the left and right of the location you had placed the head.

(1)	If the sound goes away, you likely had the search head initially positioned on top of clutter. If that is the case, move a short distance and try again to find a place clear of clutter.

(2)	If the sound continues, then the signal is almost certainly from the soil and not clutter. Move the head back to the initial location.

e.	Turn the sensitivity control back down to the point where the auditory signal just goes away. The sensitivity is now set as high as it can be set in that soil without providing feedback from the soil.

f.	Sweep the search head across the area of this 1-meter by 0.5-meter area. The sweep should be with the entire bottom of the search head in light contact with the ground, moved at no faster than 1 foot per second and with each advancing sweep overlapping the previous sweep by 1/2 the width of the search head.

g.	Be prepared to dig up metallic clutter in any area on the ground where the detector provides an auditory signal.

NOTE: When clearing clutter from an area it is useful to have a piece of canvas about 1 meter by 0.5 meters to shovel suspected cluttered soil onto. Then incrementally divide the soil to isolate the metallic clutter. The detector, a strong magnet and good vision are all assets in locating the clutter. Of course, the ground underneath the canvas needs to be free of clutter else a confusing signal maybe provided. Do not place the soil back in the hole until you are sure all the metallic components have been removed from it. Collect and control all pieces of clutter.

h.	Continue searching for and clearing the clutter until you can sweep the entire 1-meter by 0.5-meter area.

## Power Up/Sensitivity Adjustments

1. Turn SENSITIVITY and LOUDNESS knobs completely counterclockwise.

2. Observe the indicator lamp and flip the switch to the ON position. The lamp should give a short flash. If it does not, check if batteries are inserted correctly or insert new batteries. If the lamp flashes continuously, the cause may be low battery voltage.

WARNING: If the indicator lamp still does not give a short flash or flashes continuously after readjusting/replacing the batteries, discontinue using this detector until the cause of the malfunction is determined.

NOTE: Collect all damaged or low batteries and transport them to Unit HAZWAS Accumulation Point.

3. Place the search head lightly on the ground directly above the sensitivity target by visually locating the marker in the center (bulls eye) of the search head.

4. Turn the SENSITIVITY knob clockwise until you hear a continuous audible signal. The system provides this signal when the search head is over or very near a mine or other metallic object. At the same time, adjust the LOUDNESS control to a comfortable listening level.

5. Turn the SENSITIVITY knob slightly counterclockwise until the signal ceases. The check tone (a clicking sound) should be heard every 1 to 2 seconds. The system provides the check tone to continuously inform the operator that the system is functioning satisfactorily. If the check tone disappears or its frequency decreases, discontinue using this detector until the cause of the malfunction is determined.

NOTE: A flashing indicator and a continuous signal on the headphone indicates low battery voltage. The detector’s performance will be degraded when battery voltage is low so change the batteries and readjust the unit before continuing operating the detector.

6. Carefully adjust the SENSITIVITY knob to a position between the points where the signal just begins to be heard again and where you get a strong constant signal. When the soil allows, without producing a response from the detector, the sensitivity may be set at higher levels. This theoretically yields a greater probability of detection at the cost of a higher false alarm rate.

7. Move the search head away from the sensitivity target until you can no longer hear the signal.

8. Lightly float the search head on the ground at one foot-per-second movement back across the sensitivity target. With the search head in motion, check to see if the signal can again be easily heard. If you cannot hear the signal during movement at one foot-per-second, increase the sensitivity until you can hear the signal.

NOTE: At this time you may want to see what happens when you sweep too fast or advance too far in consecutive sweeps. First, sweep directly over the sensitivity target at a speed faster than one foot per second. If you are searching for a low metal target, you will note that the target signal will be lost at speeds just over one foot per second. Next, while sweeping at one foot per second, repeat sweeps while slightly advancing farther from the target at each sweep. Likewise, with a low metal target you will note that the signal will be lost at offsets not much greater than one-half the width of the search head. This exercise clearly shows how easily you can miss mines if you do not maintain precise sweep and advance. You should repeat this exercise frequently to re-emphasize the importance of precision and to combat complacency.

9. Place the search head again, lightly on the ground directly above the sensitivity target and listen carefully to the audible signal. You will need to accurately remember how this signal sounds for maintaining this sensitivity setting. Move the search head away from the sensitivity target until the signal ceases to be heard. Slide a metal object down the shaft of the detector until you hear an audible signal that sounds the same as when you placed the search head on the ground directly above the sensitivity target. Mark this position on the shaft by placing the third cable clamp (cable not connected) at this point. Repeat this process until you have precisely positioned the cable clamp at the point where the audible signal is reliably repeated.

NOTE: A common mason's trowel is an effective metal object to use for this. If one is not available, a bayonet, or many other metal objects could be used.

WARNING: In some conditions, you may not be able to detect the sensitivity target at the deepest depth that you expect to encounter a threat mine. In those conditions, do not use the PSS-12 to detect mines.

Maintaining Sensitivity Setting

WARNING: A limitation of the PSS-12 is that sensitivity fluctuates during normal use. If not checked frequently and corrected, this fluctuation could lead to missing a mine.

1. Maintain the detector sensitivity by rechecking and readjusting sensitivity if necessary about every one-two meters of advance in a mine lane. Make the check by placing the search head on the ground over an area that has been cleared. Then move the point of the trowel to the top of the sensitivity marker clamp just like you did when initially positioning the clamp. Listen for the same sound from the detector that you heard when you initially set the sensitivity over the sensitivity target.

2. If you hear the same sound from the detector, the sensitivity setting needs no adjustment. If not, slightly adjust the sensitivity knob. Repeat the check with the trowel until the like sound is heard. Make adjustments with the sensitivity knob only. Do not move the cable clamp.

NOTE: If you need to make frequent adjustments, checks may need to be made more often.

NOTE: Be sure the trowel or other metal object you may be using, shaft angle relative to head, and electronic unit are in the same position as when you initially positioned the cable clamp.

WARNING: Low batteries may reduce detector performance well before the indicator light comes on. If you need to make more frequent or larger sensitivity adjustments to maintain a constant sensitivity setting, discard all batteries and replace them with new ones.

### Sweep techniques

WARNING: You should never sweep where you cannot clearly see that the search head path is clear. If trip wires are a threat, you should use other techniques to detect trip wires before sweeping the detector. Always be aware of the potential for booby traps.

NOTE: Keep mine detectors at least two meters apart during setting, adjustment and operation to prevent interference.

NOTE: The search head should be lightly floated on the ground surface. Actual contact with the ground appears to improve the electrical coupling thereby strengthening the electrical field.

1. Adjust the handle to a comfortable position by loosening the knurl nut.

2. Adjust the position of the search head so that it can be lightly floated on the ground.

3. Sweep across the lane, keeping the entire search head in light contact with the ground. The search head path should be straight or slightly arched. Sweeping speed should be approximately 1-ft (0.3 m) per second. Each sweep must be at least 1.5 meters and must overlap the edge of the lane by at least one-half the width of the search head. Each sweep should advance forward at no more than one-half the width of the search head per sweep.

WARNING: The search head path must be straight or slightly arched. Operators who sweep in a circular motion tend to miss areas.

WARNING: Sweep and advance must be maintained as precisely as possible. Otherwise, you may leave gaps between sweep paths or at the edge of the lane and may miss a low metal mine.

4. At approximately every one to two meters of forward advance in the mine lane (or more often if desired), perform a sensitivity check and make adjustments as necessary. It is important that you maintain the same body posture and the angle between the telescopic pole and the search head each time you make a sensitivity check.

NOTE: In smooth surface conditions, loosening the wing nut on the plastic bolt that attaches the search head to the shaft can ease maintaining constant contact with the ground. This allows the search head to pivot and slide lightly across the surface.

NOTE: The sweep technique may be modified in low vegetation, rocky or uneven ground where smooth sweeping cannot be performed. With the wing nut tight so the position between the head and telescopic pole is fixed, lightly pat the search head on the ground, each pat advancing across the lane no more than 1/2 the width of the search head and advancing forward no more than 1/2 the width of the search head.

NOTE: Conditions of heavy vegetation may not allow getting the search head on the ground. Bury the sensitivity target in similar conditions to see if detection can be made. Do not push through vegetation to get the search head on the ground. If you cannot detect the sensitivity target in these conditions, do not use the AN/PSS-12 to detect mines.

NOTE: The AN/PSS-12 detector performance is reduced by moisture on the search head. This commonly occurs while sweeping in contact with wet vegetation and results in multiple and often unrepeatable auditory signals. This problem can be mitigated in training by rubbing a light coat of grease such as petroleum jelly on the head. Covering the head with a plastic bag also appears to reduce the problem. However, while this technique can be useful to enable training in a wet environment, it is not recommend for use where there is a real mine threat.

## Investigation/Mine Identification

1. At the first indication of metal in the ground, place a marker on the ground (at the center of the search head). Also, with a trowel or other tool, lightly scratch a line in the soil across the lane along the center of the last clear sweep. Use the marker to indicate where the investigation procedure begins and the line to indicate the area that has already been cleared. Sweep again across the marker to determine if the signal is repeatable and that the marker is accurately placed. If so, you should assume that the signal is a mine and continue the investigation with the purpose of determining the specific location and type of mine (high-metal or low-metal).

2. Identify the precise location of the mine by developing a set of points on the ground that outline the mine’s footprint. Develop this footprint by first moving the search head away from the signal source until no signal is heard and then sweeping the search head toward the signal from several clock positions. Note the position of the center of the search head at each location where the auditory signal begins. Never sweep past the point where the auditory signal begins. This keeps the search head from getting any closer than necessary to a potential mine. Continue sweeping away from the signal source until no signal is heard and then sweeping toward the signal from several clock positions until the size and shape of the footprint of the signal can be understood. Typically, the pattern will resemble a semi-circle with the six-o’clock position nearest the operator.

3. Identify the type of mine by examining the size and shape of the mine’s footprint. Small footprints, often as small as four to six inches in diameter indicate low metal mines. High metal mines may have footprints two to four feet in diameter. The footprint is defined as the entire area on the ground where the mine or metallic source causes the detector to generate an auditory signal.

WARNING: There is a strong tendency for soldiers to loose concentration after marking the location of a mine. This loss of concentration has been observed to frequently contribute to potentially disastrous events.

- Stepping on the mine just marked

- Becoming less vigilant and missing other mines in the nearby area

- Loss of sweep precision

- Loosing track of where they were

NOTE: In some cases, the detector may provide a weak signal that cannot be repeated during the subsequent investigation. In this case, continue to assume that a mine is present and conduct a sensitivity check on the detector to insure it has not drifted to the low side. If the signal still cannot be repeated, continue to assume a mine is present until you are convinced that the detector is functioning properly and safe sweeping is possible.

NOTE: In the presence of clutter or closely placed mines, the semi-circle can become distorted. The operator must be very careful but may be able to slightly vary orientation of semi-circle development to avoid the influence of clutter. Foot position must also be monitored very closely to avoid stepping into unsafe areas.

WARNING: It is common practice for low metal mines to be placed within the footprint area of high-metal mines. The large footprints of high metal mines may mask signals from low metal mines within the footprint. Always assume that there are low metal mines within this footprint area.

3. It is sometimes important to identify the center of very large footprints. A means to do this that may be quicker then the method previously defined to identify the perimeter is called the airborne technique. This technique involves adjusting the search head so it can be held above the ground but with the head parallel to the surface. Then the detector can be manipulated above the source until the signal can be heard at only a single point. Near the surface, the signal can be heard over a broad lateral area, but as the search head is raised higher off the ground, this area becomes progressively smaller. As the search head is raised higher off the ground, the area where the signal can still be heard is reduced to a point. The center of the mine or metal source should be directly below this point. Do not ignore the potential for low metal mines to be hidden in this large footprint on the ground.

WARNING: Be sure to have a firm body position before attempting the airborne technique. Raising the search head could easily cause you to loose balance and fall onto the mine.

WARNING: The search head should never be swept where its path cannot be visually cleared first. If trip wires are a threat, other techniques to detect trip wires should be used before sweeping the detector. Always be aware of the potential for booby traps and UXO.

Packing and Storage:

1. Clean the equipment with a moist cloth and let it dry. After the mine detector has been used in salt water it should be cleaned with fresh water.

2. Check the equipment for damage.

3. Report any faults and deficiencies that have not been remedied to your supervisor.

4. Ensure switch is in off position.

5. Detach the connections from the search head and headphone and put the protective caps on the plug sockets.

6. Release latches and remove battery cover. Remove the batteries and check to ensure battery acid has not leaked out. If so, notify your supervisor. Reinstall and latch battery cover.

NOTE: Collect all damaged or low batteries and transport them to Unit HAZWAS Accumulation Point.

7. Remove the cable clamps from the telescopic pole.

8. Collapse the telescopic pole and turn its outer tube until it is locked by the catch. Fold in the search head.

9. Pack main components in the carry bag as shown. For long term storage do not store batteries in carry bag. Close and zip carry bag.

10. Place carrying bag in metal transport case and latch case. Close pressure relief valve.