# Section D

# Site

# and

# Grading

# Layout

This page intentionally left blank

Layout and Information for Construction and Use of

Model Handheld Mine Detection Training Site

Model Site Layout:

1.	This model training site was developed as a guide for designing, constructing and using mine detection training areas. It consists of several types of training areas for soldiers to learn and practice basic and more advanced skills in detecting mines with the AN/PSS-12 or other handheld mine detectors. These training areas consist of:

a.	Training Matrix

b.	Clutter Matrix

c.	Calibration Lanes

d.	Performance Lanes

e.	Calibration Points

f.	Tactical Lanes

2.	The general layout for the model training site is shown in Figure 1.

## Figure 1: Training Site Layout

Training Matrix: (15meter by 15 meter grid)

1.	The training matrix is a square area, 15 meters in length and width. Within the matrix are ten columns and ten rows. The columns are labeled from one to ten and the rows are labeled from A to J. The boundaries of the columns and rows are 1.5 meters apart and are marked with non-metallic cord. These non-metallic cords are secured to a chain that runs least one-half meter outside the matrix perimeter. The chain must be located at least one-half meter outside the matrix to prevent the chain’s metal from interfering with the metal detector. The non-metallic cords can be removed as required to more directly convert the matrix into lanes. All accuracy of grading however, is lost if these cords are not returned to their original location of attachment to the perimeter chains. The specific chain links which correspond with the 1.5 meter lane widths should be identified with paint or other durable marking. Before removing cords connected to the chain, inspect the chain to ensure link is properly marked. The layout of the training matrix is shown in Figure 2.

## Figure 2: Training Matrix Layout

2.	Each column and each row will have six mines. Six of the ten cells will contain one mine and four cells will be empty. No cells will contain more than one mine. The mines will be distributed among the cells as follows:

2 - M14 low metal anti-personnel simulants (buried at 2 inches to the top)

1 - PMA3 with Safety low metal anti-personnel simulant (buried at 2 inches to the top)

1 - VS 2.2 low metal anti-tank simulant (buried at 4 inches to the top)

1 - M15 high metal anti-tank simulant (buried at 2 inches to the top)

1 - M16 high metal anti-personnel mine simulant (buried at 2 inches to the top).

3.	The precise position for each mine is identified in the Mine Location Map in Figure 3.

### Figure 3: Mine Locations Within Training Matrix

4.	Finding the precise positions for burying and scoring the mines is accomplished by using a location grid. The location grid is a square, constructed of plastic pipe or other appropriate material, 1½ meter in length and width, divided into ten columns and rows. The columns and rows are labeled the same way as the training matrix is labeled with the columns labeled from one to ten and the rows labeled from A to J. To locate the position of a mine, position the location grid so that the top of the frame lines up with the non-metallic cord at the top of the cell and the left side of the frame lines up with the non-metallic cord on the left side of the cell. Properly positioning the matrix in relation to the upper and left side non-metallic cords is critical to accurate grading. Slightly raising and then releasing (popping) the cord ensures it is not caught on anything that might prevent it from running straight through the matrix. Bury and locate the precise position of the center of the mines directly under the intersections of columns and rows.

5.	The overall grade sheet for the training matrix is laid out like the matrix. Within cells of the matrix that have mine simulants, location of the center of the mine is indicated by the intersection of a column and a row. This column and row location on the ground is found by properly positioning the location grid in the cell of interest.

Performance Lanes:

1.	There are ten performance lanes in the training site, 1½ meters wide by 15 meters in length. There is approximately a 1-meter standoff distance at each end of the lanes used to keep the detectors out of the metal signature of the metal stakes at the ends of the lane. Plastic pipes extend out of the ground on the east side of each lane. These pipes are positioned 120 inches apart and serve as aligning rods for the grading grid.

2.	Each lane contains a total of 14 semi-randomly positioned mines. These mines consist of

5 - M14 low metal anti-personnel simulants (buried at 2 inches to the top)

4 - PMA3 with Safety low metal anti-personnel simulant (buried at 2 inches to the top)

3 - M19 simulant low metal anti-tank simulant (buried at 4 inches to the top)

1 - M15simulant high metal anti-tank simulant (buried at 2 inches to the top)

1 - M16 high metal anti-personnel mine simulant (buried at 2 inches to the top).

3.	Position of the mines for grading is determined by accurate positioning of the one inch plastic pipe grid. This grid is approximately 1½ meters by 3 meters. The columns and rows are labeled from one to twenty and from A to J, respectively. All mines are located underneath the intersection of specified rows and columns. An illustration of the grading grid is at Figure 4.

### Figure 4: Illustration of the Grading Grid

4.	The grade sheets for the performance lanes are used in a similar way as the smaller grid is used in the training matrix. However, instead of using the non-metallic cord as for orientation, the grading grid is oriented by grading stakes which are positioned at approximately three meter intervals along the left side of the lanes. The grading grid is illustrated in the picture below.

5.	Position the “1-J” corner of the grading matrix just over the first grading stake (¾ inch plastic pipe) about 1 meter from the beginning of each lane. While keeping the “1-J” corner of the grading matrix in contact with grading stake one, place the pipe extending from the “20-J” corner is place in contact with the second grading stake. This position of the grid corresponds to "Set 1" on the grade sheet. To score "Set 2," place the “1-J” corner of the grading matrix in contact with grading stake two and the pipe extending from the “20-J” corner is place in contact with the grading stake three. Continue to score “Sets 3-5” in the same manner. Properly positioning the matrix in relation to the grading stakes is critical to accurate grading. Additionally, it is recommended the grid be positioned to the next Set by one person lifting from each end. Other techniques may cause the grid to become out of square and thus not properly align with the mines.

Calibration Lanes:

1.	There are two calibration lanes in the training site. They are mirror images of each other. All mines buried in the calibration lanes are marked with orange plastic tape extending up from each side of the mines.

2.	Starting at the end near the wood stand, mine simulants will be found in the following order:

- The first mine is approximately 1 meter from the end (moving away from the stand) and in the center of the lane is an M14 mine simulant buried at a 2 inches depth.

- The second mine is one meter further down the lane and is a PMA3 with safety simulant buried at a 2 inches depth.

- The third mine is 1 meter further down the lane and is a VS2.2 simulant buried at a 4 inches depth.

- The fourth mine is 1½ meter further down the lane and is an M16 simulant and buried at a 2 inch depth.

- The fifth mine is 1½ meter further down the lane and is an M15 simulant and buried at a 2 inch depth.

- The sixth and last mine is 1 meter further down the lane and is an M14 simulant and buried at a 2 inch depth.

Tactical Lanes

1.	Eight tactical lanes in the training site, 15 meters long with approximately 1 meter standoff distance at each end. This standoff distance is required to keep the single metal stake in the ground at each end of the lanes from interfering with the metal detector. Blue stakes indicate the start of the lanes and red stakes indicate the ends of the lanes.

2.	Each tactical lane contains a total of 10 mines. These mines consist of:

4 - M14 low metal anti-personnel simulants (buried at 2 inches to the top)

2 - PMA3 with Safety low metal anti-personnel simulant (buried at 2 inches to the top)

3 - M19 simulant low metal anti-tank simulant (buried at 4 inches to the top)

1 - M16 high metal anti-personnel mine simulant (buried at 2 inches to the top).

3.	Tactical lanes are graded by running a grading chain from the start of the lane (blue post) to the end of the lane (red post). The chain has a 1½ inch section of plastic pipe attached to specific links of the chain at 1 meter increments. The plastic pipe serves as a Set Marker.” The Set Marker represents "Set 1,” (and is labeled "1") the second "Set 2" and so on through "Set 15.” Mines are then located at specific locations up the chain from the "Set Marker" and then to the left or right of the chain. A “Scoring Square” is used to locate the mines in relation to the chain between sets. The “Scoring Square” is constructed with a 1 meter (100cm) section of plastic pipe connected to a 90 degree elbow and than to a ¾ meter (75 cm) plastic pipe. The 1 meter plastic pipe is labeled in increments of 10 cm starting with 0 and ending with 100 while the ¾ plastic pipe is in increments of 10 cm starting with 5 and ending with 75. The “Scoring Square is placed along side of the chain to measure the up distance between “Set Markers” and the distance to the mine, right or left of the chain. For example, a mine might have on the grade sheet a designated location of "Set 5,” 70 - R - 15. This would mean that lying the square along the right side of the chain, you would go to the forward most edge of "Set 5 Section Marker" (end of # 5 Set Marker closest to the # 6 Set Marker) and place the 70 cm increment at this location. All increments are labeled and have a hole drilled through the Scoring Square at the precise location. Then go up the pipe to the elbow and out to the right along the ¾ meter plastic pipe to the 15 cm hole. This is then the location of the center of the mine designated by "Set 5,” 70 - R - 15.

4.	The chain snaps into the start post (blue) and attaches to the end post (red) with an elastic strap. The elastic strap helps keep the chain pulled tight and thus straight between posts. It is imperative that the scoring chain runs the proper direction. The lanes can be run either direction but the chain can only be positioned in one direction for proper scoring.

Calibration Points

1.	Three calibration points are located in the area of the tactical lanes:

- Near the start point of Tactical Lane 1

- Near the start point of Tactical Lane 4

- Between the start points of Tactical Lanes 7 and 8.

2.	A stake painted blue and red identifies these points. A marked M14 mine simulant, for calibration, is buried nearby. The mine is marked with the orange plastic tape running out of the ground on each side of it.

Grading Sheets

1.	Grading sheets are available for the following:

a.	Training Matrix

(1)	By column

(2)	By row

b.	Performance lanes

c.	Tactical lanes