# Section E

# Mine

# Simulant

# Information

This page intentionally left blank

Sources and Specifications for Mine Simulants

1.	Hockey Pucks:

Pittsburgh House of Hockey

Ph 412-784-0919

POC: Ron

2.	Metal Inserts:

KNS Engineering Inc. Chicago Ill.

Ph. 773-586-8503

Tubes Required:

"I" Insert 	0.187 in outer diameter	0.015 in wall thickness	0.5 in long

"K" Insert	0.25 in outer diameter	0.015 in wall thickness	0.5 in long

"M" Insert	0.25 in outer diameter	0.015 in wall thickness	1.5 in long

14 gauge black utility wire	0.25 in long

This metal is purchased from KNS in several foot lengths. As a result, they must be cut to the proper and precise lengths. Precision is critical to maintaining high fidelity simulants.

3.	Approximate mine replications:

"I" Insert = M14, PMA3, & VS-1.6

"K" Insert = Type 72A, PMA3 with Safety spring (PMA3 w/s), TMA-4, & M19

"M" Insert = TS-50, VS-2.2, & PT Mi-Ba-III

4.	Construct mine simulants by drilling hole in bottom center of puck slightly deeper then length of insert. Hole diameter should allow compressed fit of insert. To replicate PMA3-with safety spring, drill hole in bottom center of puck for "K" insert then drill second hole in puck 0.25 inches to side of where "K" insert is positioned, and press the 14 gage wire into the hole. The "M" insert is 1.5 inches long so requires that two pucks be first glued together. All weather carpenter's tube glue works well. After the pucks are glued together, drill the hole from the bottom center of one of the pucks. Press the "M" insert into the hole. The inserts should fit very tight in the holes. In all cases the insert is positioned from the bottom of the puck. Recommend marking the puck as to the type simulant by carving a letter into it or color-coding with paint.

5.	The fidelity of high metal mine simulants is less critical than for low metal mine simulants. An adequate simulant of a high metal anti-tank mine such as an M-15 is a 1 gallon tin can (sometimes called a "number 10" can) available from mess halls. An adequate simulant of a high metal anti-personnel mine such as an M-16 is any common 14 to 18 ounce can.

6.	POC is:

Alan D. Davison, Ph.D.

Chief, Army Research Laboratory

Human Research and Engineering Directorate

U.S. Army Maneuver Support Center Field Element

Ft. Leonard Wood, Missouri

Ph DSN 676-4008 or Comm. 573-563-4008

email: davisona@wood.army.mil

Mine Simulants Required for Standard Training Site

1.	Training Matrix

20 Ea M14 ("I" Inserts)	20 pucks

10 Ea PMA3w/Safety ("K" Inserts)	10 pucks

10 Ea VS2.2 ("M" Inserts)	20 pucks (each simulant requires two pucks)

10 Ea M16	14-16 Oz Vegetable Cans

10 Ea M15	1 Gallon (#10 Can)

2.	Calibration lanes (There are two calibration lanes)

4 Ea M14 ("I" Inserts)	4 pucks

2 Ea PMA3w/Safety ("K" Inserts)	2 pucks

2 Ea VS2.2 ("M" Inserts)	4 pucks (each simulant requires two pucks)

2 Ea M16	14-16 Oz Vegetable Cans

2 Ea M15	1 Gallon (#10 Can)

3.	Performance lanes (There are 10 performance lanes)

50 Ea M14 ("I" Inserts)	50 pucks

40 Ea PMA3w/Safety ("K" Inserts)	40 pucks

30 Ea VS2.2 ("M" Inserts)	60 pucks (each simulant requires two pucks)

10 Ea M16	14-16 Oz Vegetable Cans

10 Ea M15	1 Gallon (#10 Can)

4.	Tactical lanes (There are 8 tactical lanes)

32 Ea M14 ("I" Inserts)	32 pucks

16 Ea PMA3w/Safety ("K" Inserts)	16 pucks

24 Ea VS2.2 ("M" Inserts)	48 pucks (each simulant requires two pucks)

8 Ea M16	14-16 Oz Vegetable Cans

5.	Total Training Site Requirements:

106 Ea M14 ("I" Inserts)	106 pucks

68 Ea PMA3w/Safety ("K" Inserts)	68 pucks

66 Ea VS2.2 ("M" Inserts)	132 pucks (each simulant requires two pucks)

30 Ea M16	14-16 Oz Vegetable Cans

22 Ea M15	1 Gallon (#10 Can)

Total Pucks Required = 306