Section F

Site

Bill

of

Materials

This page intentionally left. blank

Bill of Materials -- Site Development

In control area:

10 lane x 10 lane training matrix

10 test lanes (15 meters in length)

4 x 4 clutter matrix

2 calibration (cal) lanes (7.5 meters in length)

Bill of Material:

4 ea, 2 ft. steel pickets	to establish corners or training matrix

4 ea, 2 ft. steel pickets 	to establish corners of clutter matrix

48 ea, 2 ft. steel pickets	to establish corners of each test lane (10) and cal lanes (2)

3000 ft. nylon chord	to mark edges of test, calibration lanes, and matrixes

56 ea, bungee chords	to maintain tight lane edge markers for lanes and matrices

10 ea, bags of quickcrete	to fix PVC pipe at grading guide positions along left. side of performance lanes

330 ft., light chain (2/0)	to run perimeters of matrices and enable snap in of cross members

8 ea, I-bolts	to enable connection of chain to pickets at corners of matrices

8 ea, chain link connectors	to enable connection of chain to corners of matrices

8 ea, 6" turn buckles	to keep chain on perimeter of matrices tight

In natural area:

2 test lanes (15 meters in length) prairie grass (will replicate road side slope to extent practical)

3 test lanes (15 meters in length) bush vegetation

3 test lanes (15 meters in length) forested

Bill of Material:

18 ea, 2 ft. steel pickets	to establish ends of test lanes

170 ft., light chain	to build three scoring chains

18 ea, I-bolts	to enable connection of chain to pickets at each end of lanes

2 ea, snaps	to connect to ends of chains to snap into I-bolts

2 ea, chain link connectors	to enable connection of chain to corners of matrices

2 ea, bungee chords	to maintain tight lane grading chains

50 ea, 1" sections of 3/4" pipe	to mark each 1 ft. of chain for grading 	purposes (must drill holes in each to secure to chain)

50 ea, plastic quick fasteners 	to connect pipe sections to chain links

Combined BOM for all of Site:

4 ea, 2 ft. steel pickets	to establish corners or training matrix

4 ea, 2 ft. steel pickets 	to establish corners of clutter matrix

18 ea, 2 ft. steel pickets	to establish ends of test lanes

48 ea, 2 ft. steel pickets	to establish corners of each test lane (10) and cal lanes (2)

3000 ft., nylon chord	to mark edges of test, calibration lanes and matrixes

56 ea, bungee chords	to maintain tight lane edge markers for lanes and matrices

3 ea, bungee chords	to maintain tight lane grading chains

10 ea, bags of quickcrete	to fix wood tent pegs at corners of lanes and pickets at corners of matrices (1/2 bag per post)

330 ft., light chain	to run perimeters of matrices and enable snap in of cross members

170 ft., light chain	to build three scoring chains

8 ea, I-bolts	to enable connection of chain to pickets at corners of matrices

8 ea, chain link connectors	to enable connection of chain to corners of matrices

18 ea, I-bolts	to enable connection of chain to pickets at each end of lanes

2 ea, snaps	to connect to ends of chains to snap into I-bolts

2 ea, chain link connectors	to enable connection of chain to corners of matrices

30 ea, 1" long sections of 3/4" PVC 	to mark each 1 ft. of chain for grading purposes pipe

50 ea, plastic quick fasteners 	to connect PVC pipe to chain links

8 ea, 6" turn buckles	to keep chain on perimeter of matrices tight

Estimated Cost for Combined BOM for all of Site:

4 ea, 2 ft. steel pickets	GFE

4 ea, 2 ft. steel pickets 	GFE

18 ea, 2 ft. steel pickets	GFE

48 ea, 2 ft. steel pickets	GFE

3000 ft., nylon (parachute) chord	$120	about $0.04 per ft.

60 ea, bungee chords or tie down straps	$60	about $1.00 ea

10 ea, bags of quickcrete	$30

600 ft., light chain	$330	2/0 double loop about $0.45 to 0.50 per ft.

30 ea, I-bolts 3/16 X 3 1/4"	$12

30 ea, chain link connectors 3/16"	$4

4 ea, snaps 3/4 " rope snaps	$6

75 ea, plastic quick fasteners 1/4" x 6"	$12

16 ea, 1/4 X 7 1/2" turn buckles	$20

120 ft., 1" PVC Pipe	$60	to build grading grids for performance lanes and matrix and to provide grading stakes in performance lanes

10 ft., 3/4" PVC Pipe	$3	to mark each meter of grading chain to be used in tactical lanes

8 ea, 1" 90 degree elbows 	$2	4 for lane grading grid, 4 for matrix grading grid

2 ea, 1" T's 	$1	for use in lane grading grid

500 ft., stretch cord	$50	for use in grading grids

Total Estimated Cost = 	$1000

Tools and equipment required to set up sites:

Assuming 5 soldiers are available.

2 wheel barrows	1 to mix mortar, 1 to assist in clearing lanes of clutter

4 shovels	for digging around posts for concrete pour and clearing clutter

4 stone rakes	for clearing clutter in sand box

1 or 2 magnets on a stick	for clearing clutter

6 large paper feed bags	for placing soil while searching for clutter

2 trowels	for searching soil for clutter

2 mauls or sledge hammers	for driving posts and pickets (requires eye protection)

Small set of tools	for connecting I-bolts to pickets links to chain

Drill and bits	to drill wholes in pickets and 3/4" plastic pipe sections (better to do at a metal shop)

Water supply	for drinking and concrete mixing (water buffalo if possible)

1 hoe	for concrete mixing

1 long tape measure	for making measurements

1 compass	for determining direction

1 hacksaw	for cutting plastic pipe